# SAN JOSE STATE UNIVERSITY

# Charles W. Davidson College of Engineering

DEPARTMENT OF ELECTRICAL ENGINEERING

EE 271 – Advanced Digital System Design and Synthesis

Fall 2014 Final Project Report

A Simple 8-bit Scalar Processor

| Name | MOHITKUMAR PATEL | Email | mohitkumar.patel@sjsu.edu |
| --- | --- | --- | --- |
| SJSUID | 009288552 | Phone | +1 213-300-7130 |

Executive Summary

I have designed a Simple 8-bit Scalar Processor which supports subset of MIPS instructions. This Scalar Processor is designed with a Controller, an 8-bit ALU and 256 byte Memory. The machine codes of 10 different instructions are stored in the Instruction Memory ranging from 0 to 127 bytes and Data is stored in Data Memory ranging from 128 to 256 bytes. Controller is the heart of this system which controls all other modules and performs instruction fetch, decode, execution, and write back operations. Moreover, there are four different Registers named General Register (GR), Program Counter Register (PR), Address Register (AR) and Data Register (DR) which are used to transfer the data to or from the Memory. ALU is designed to perform two operations i.e. Additions and Subtractions. The Controller sets all internal Register to zero whenever RESET is asserted. The Controller Fetches the Instruction from the Instruction Memory, Decodes the Instructions and defines Instruction Machine code, Updates the Program Counter Register (PR) for the next instruction to be fetched, and Executes the instruction and stores the data to Data Memory if instructed.

## I.	General Project Information

## Table I.1: List of EDA Tools Used

| Name | Company | Used for | Free? (Y/N) | Software Documents |
| --- | --- | --- | --- | --- |
| VCS | Synopsys | Simulation & test | No |  |
| ModelSim PE Student Edition | Mentor Graphics | Simulation & test | Yes |  |
| Design Vision | Synopsys | Synthesis | No |  |

## Table I.2: List of Libraries Used

| Library file name | Company | Used with (EDA tool) | The libraries are at (directories on eecad systems) |
| --- | --- | --- | --- |
| Tc240c.db_WCCO M25 | Toshiba | Synopsys | /apps/toshiba/sjsu/synopsys/tc240c/tc240c.db_ WCCOM25 |
| Tc240c.db_NOMI N25 | Toshiba | Synopsys | /apps/toshiba/sjsu/synopsys/tc240c/tc240c.db_ NOMIN25 |
| Tc240c.db_BCCO M25 | Toshiba | Synopsys | /apps/toshiba/sjsu/synopsys/tc240c/tc240c.db_ BCCOM25 |

## Table I.3: List of Verilog Modules (both design and test modules)

| Module Name | Input Ports | Output Ports | Inout Ports | Short Description |
| --- | --- | --- | --- | --- |
| SCALAR_PROCESSOR | clk, rst | [7:0]addr, rd, wrt | [7:0]dat | This module is basically the controller unit. Whenever rst signal is asserted, controller sets all internal register to zero and when the clk signal is preset and reset is not asserted, controller provides addr signal to point the instruction address of Instruction Memory which needs to be fetched. Then it reads the Instruction or Data through dat port executes it and writes the result back to Memory according to instruction operations through dat port. |
| SCALAR_TEST | [7:0]addr, rd, wrt | clk,rst | Dat | This is the test bench for a Scalar Processor which contains 256 byte Memory. Different Memory instructions and Data are stored in Memory for operation. This test bench module is to verify the functions implemented in the controller. |
| ALU | [7:0]a, cin, [7:0]b | [7:0]sum,cout,ov |  | This module is used to perform Addition and Subtraction. Input to this module is provided bu controller unit. Whenever cin is low, ALU performs addition and when cin signal is high ALU performs subtraction between port ‘a’ and port ‘b’ data. The output signal ov is used to define the overflow detection of ALU result |
| RCA_8 | [7:0]a, cin, [7:0]b | [7:0]sum, cout |  | This is 8 bit ripple carry adder used to perform 8 bit number addition between ‘a’ port data and ‘b’ port data and output is achieved at sum port. If the result is above 8 bit then cout bit will go high which indicates carry out. This is the sub module of ALU module. |
| add_full | a,b,cin | sum,cout |  | This is one bit full adder used to implement 8 bit ripple carry adder. This is the sub module of RCA_8 module. |
| add_half | a, b | sum, cout |  | This is one bit half adder used to implement full adder. This is the sub module of add_full module. |

## II.	The Design Overview

An 8-bit Scalar Processor block diagram is shown in figure II.1. The block diagram contains a 4x8 register array which defines AR, GR, DR and PR register for this project. This array is inside the Control unit and operated by controller only. Plus, an 8-bit ALU is connected to Controller unit to perform arithmetic operations such as addition and subtraction. The combination of both Control and ALU unit defines the 8-bit Scalar Processor. There is a Memory of 256 bytes connected to Control Unit to provide instruction machine codes and data. This Memory is basically inside the testbench of Scalar Processor. The Memory has 0 to 127 bytes for machine code and 128 to 256 bytes for data. Control Unit accesses this Memory by asserting rd, wrt and addr signals.

4 x 8 ARRAY

CONTROL UNIT

ALU

PR

DR

GR

AR

[3:0]FR

D

SCALAR  PROCESSOR

256 BYTE MEMORY

TEST BENCH

Figure II.1: Overall Processor Block Diagram

Figure II.2: Overall State Transition Diagram(s)

## Table II.1: Number of Clock Cycles Required for Each Instruction

| Instruction | # of Clock Cycles | Instruction | # of Clock Cycles | Instruction | # of Clock Cycles |
| --- | --- | --- | --- | --- | --- |
| NOP | 1 | LMS | 1 | ADD | 2 |
| JMP | 1 | CFR | 1 | SUB | 2 |
| CPR | 1 | RDM | 2 |  |  |
| LLS | 1 | WRM | 2 |  |  |

During simulation, I analyzed that for ADD and SUB instructions, it takes one clock cycle to give the input to ALU by Control Unit. Since ALU is a combinational block, it gives the output in the same clock cycle. However, storing the ALU output to destination register takes one more clock cycle for both ALU and SUB instructions. Moreover, for RDM and WRM instructions ,the processor takes one clock cycle to set the memory address to read or write and one clock cycle to read from or write to Memory Unit. Therefore, the optimum number of clock cycles for ADD, SUB, RDM and WRM instruction are two that I was able to achieve.

III.	RTL-Level Simulations/Tests for Each Instruction

Figure III.1: Test Result for JMP Instruction

Simulation Result for JMP:

1992 ns FETCH STATE: Instruction is Fetched

2000 ns DECODE STATE: Instruction is Decoded

2008 ns Instruction : JMP

UPDATE STATE:PR Register is not Incremented to Fetch the next Instruction.

2016 ns EXECUTE_1 STATE: Executed JMP and Jumped to Instruction stored at 00

Memory Location

Figure III.2: Test Result for CPR Instruction

Simulation Result for CPR:

96 ns  FETCH STATE: Instruction is Fetched

104 ns DECODE STATE: Instruction is Decoded

112 ns Instruction : CPR, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

120 ns EXECUTE_1 STATE: Executed CPR and Copied the Contents of 10 to 01 reg.

AR[00]= 00, DR[01]= 45, GR[10]= 45, PR[11]= 03

Figure III.3: Test Result for LLS Instruction

Simulation Result for LLS:

16 ns  FETCH STATE: Instruction is Fetched

24 ns  DECODE STATE: Instruction is Decoded

32 ns  Instruction : LLS, 5

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

40 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 5

AR[00]= 00, DR[01]= 00, GR[10]= 05, PR[11]= 01

Figure III.4: Test Result for LMS Instruction

Simulation Result for LMS:

56 ns  FETCH STATE: Instruction is Fetched

64 ns  DECODE STATE: Instruction is Decoded

72 ns  Instruction : LMS, 4

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

80 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 4

AR[00]= 00, DR[01]= 00, GR[10]= 45, PR[11]= 02

Figure III.5: Test Result for CFR Instruction

Simulation Result for CFR:

1584 ns FETCH STATE: Instruction is Fetched

1592 ns DECODE STATE: Instruction is Decoded

1600 ns Instruction : CFR;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1608 ns EXECUTE_1 STATE: Executed CFR and Stored data  at LSBs of GR = 1

AR[00]= 82, DR[01]= 66, GR[10]= 81, PR[11]= 26

Figure III.6: Test Result for RDM Instruction

Simulation Result for RDM:

256 ns  FETCH STATE: Instruction is Fetched

264 ns  DECODE STATE: Instruction is Decoded

272 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

280 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 80 to Memory Address...

288 ns  EXECUTE_2 STATE: Read the Data 44 from Memory Location 80 and Stored

into 10 Register

AR[00]= 80, DR[01]= 45, GR[10]= 44, PR[11]= 07

Figure III.7: Test Result for WRM Instruction

Simulation Result for WRM:

352 ns  FETCH STATE: Instruction is Fetched

360 ns  DECODE STATE: Instruction is Decoded

368 ns  Instruction : WRM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

376 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = 80  to Memory Address...

384 ns  EXECUTE_2 STATE: 10 Register Data 89 is Written at Memory Address 80

WRITEEN DATA : RAM[80h]= 89, RAM[81h]= 55, RAM[82h]= 66,  RAM[FO]= xx

Figure III.8: Test Result for ADD Instruction

Simulation Result for ADD:

304 ns  FETCH STATE: Instruction is Fetched

312 ns  DECODE STATE: Instruction is Decoded

320 ns  Instruction : ADD, 10, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

328 ns  EXECUTE_1 STATE: Executing ADD...Providing ALU Opcodes 44 and 45 and

Adding them...

336 ns  EXECUTE_2 STATE: Added the 10 Register Data 44 with 01 Register Data

45 and the Result is Stored into 10 Register

AR[00]= 80, DR[01]= 45, GR[10]= 89, PR[11]= 08

Figure III.9: Test Result for SUB Instruction

Simulation Result for SUB:

1368 ns FETCH STATE: Instruction is Fetched

1376 ns DECODE STATE: Instruction is Decoded

1384 ns Instruction : SUB, 10, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1392 ns EXECUTE_1 STATE: Executing SUB...Providing ALU Opcodes 88 and 66 and

Adding them...

1400 ns EXECUTE_2 STATE: Subtracted the 01 Register Data 66 from 10 Register

Data 88 and the Result is Stored into 10 Register

AR[00]= 92, DR[01]= 66, GR[10]= 22, PR[11]= 21

IV.	RTL-Level Simulations/Tests of Whole Design (processor)

IV.1	Test Program, Machine Codes, and Test Data

## Table IV.1: Memory Code Address (in Hex), Test Instruction, and Machine Code (in Hex)

| Memory Address | Instruction | Machine Code |
| --- | --- | --- |
| 0x00 | LLS | 0x75 |
| 0x01 | LMS | 0x84 |
| 0x02 | CPR DR, GR | 0x46 |
| 0x03 | LLS | 0x70 |
| 0x04 | LMS | 0x88 |
| 0x05 | CPR AR, GR | 0x42 |
| 0x06 | RDM GR | 0x28 |
| 0x07 | ADD GR, DR | 0x59 |
| 0x08 | WRM GR | 0x38 |
| 0x09 | LLS | 0x71 |
| 0x0a | LMS | 0x88 |
| 0x0b | CPR AR, GR | 0x42 |
| 0x0c | RDM GR | 0x28 |
| 0x0d | CPR DR, GR | 0x46 |
| 0x0e | LLS | 0x71 |
| 0x0f | LMS | 0x89 |
| 0x10 | CPR AR, GR | 0x42 |
| 0x11 | RDM GR | 0x28 |
| 0x12 | ADD DR, GR | 0x56 |
| 0x13 | LLS | 0x71 |
| 0x14 | LMS | 0x88 |
| 0x15 | CPR AR, GR | 0x42 |
| 0x16 | WRM DR | 0x34 |
| 0x17 | LLS | 0x72 |
| 0x18 | LMS | 0x88 |
| 0x19 | CPR AR, GR | 0x42 |
| 0x1a | RDM GR | 0x28 |
| 0x1b | CPR DR,GR | 0x46 |
| 0x1c | LLS | 0x72 |
| 0x1d | LMS | 0x89 |
| 0x1e | CPR AR, GR | 0x42 |
| 0x1f | RDM GR | 0x28 |
| 0x20 | SUB GR, DR | 0x66 |
| 0x21 | LLS | 0x72 |
| 0x22 | LMS | 0x88 |
| 0x23 | CPR AR, GR | 0x42 |
| 0x24 | WRM GR | 0x34 |
| 0x25 | CFR | 0x90 |
| 0x26 | CPR DR, GR | 0x46 |
| 0x27 | LLS | 0x70 |
| 0x28 | LMS | 0x8f |
| 0x29 | CPR AR, GR | 0x42 |
| 0x2a | WRM DR | 0x34 |
| 0x2b | NOP | 0x00 |
| 0x2c | LLS | 0x70 |
| 0x2d | LMS | 0x80 |
| 0x2e | CPR AR, GR | 0x42 |
| 0x2f | JMP | 0x10 |

## Table IV.2: Initial Test Data Stored at Memory Addresses (in Hex)

| Memory Address | Data | Memory Address | Data |
| --- | --- | --- | --- |
| 0x80 | 0x44 | 0x91 | 0x77 |
| 0x81 | 0x55 | 0x92 | 0x88 |
| 0x82 | 0x66 |  |  |

IV.2	Test Results

- Case 1:

WRITEEN DATA : RAM[80h]= 44, RAM[81h]= 55,  RAM[82h]= 66,  RAM[FO]= xx

16 ns  FETCH STATE: Instruction is Fetched

24 ns  DECODE STATE: Instruction is Decoded

32 ns  Instruction : LLS, 5

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

40 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 5

AR[00]= 00, DR[01]= 00, GR[10]= 05, PR[11]= 01

56 ns  FETCH STATE: Instruction is Fetched

64 ns  DECODE STATE: Instruction is Decoded

72 ns  Instruction : LMS, 4

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

80 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 4

AR[00]= 00, DR[01]= 00, GR[10]= 45, PR[11]= 02

96 ns  FETCH STATE: Instruction is Fetched

104 ns DECODE STATE: Instruction is Decoded

112 ns Instruction : CPR, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

120 ns EXECUTE_1 STATE: Executed CPR and Copied the Contents of 10 to 01 reg.

AR[00]= 00, DR[01]= 45, GR[10]= 45, PR[11]= 03

136 ns  FETCH STATE: Instruction is Fetched

144 ns  DECODE STATE: Instruction is Decoded

152 ns  Instruction : LLS, 0

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

160 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 0

AR[00]= 00, DR[01]= 45, GR[10]= 40, PR[11]= 04

176 ns  FETCH STATE: Instruction is Fetched

184 ns  DECODE STATE: Instruction is Decoded

192 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

200 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 00, DR[01]= 45, GR[10]= 80, PR[11]= 05

216 ns  FETCH STATE: Instruction is Fetched

224 ns  DECODE STATE: Instruction is Decoded

232 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

240 ns  EXECUTE_1 STATE: Executed CPR and Copied the Contents of 10 to 00 reg.

AR[00]= 80, DR[01]= 45, GR[10]= 80, PR[11]= 06

256 ns  FETCH STATE: Instruction is Fetched

264 ns  DECODE STATE: Instruction is Decoded

272 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

280 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 80 to Memory Address...

288 ns  EXECUTE_2 STATE: Read the Data 44 from Memory Location 80 and Stored

into 10 Register

AR[00]= 80, DR[01]= 45, GR[10]= 44, PR[11]= 07

304 ns  FETCH STATE: Instruction is Fetched

312 ns  DECODE STATE: Instruction is Decoded

320 ns  Instruction : ADD, 10, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

328 ns  EXECUTE_1 STATE: Executing ADD...Providing ALU Opcodes 44 and 45 and

Adding them...

336 ns  EXECUTE_2 STATE: Added the 10 Register Data 44 with 01 Register Data

45 and the Result is Stored into 10 Register

AR[00]= 80, DR[01]= 45, GR[10]= 89, PR[11]= 08

352 ns  FETCH STATE: Instruction is Fetched

360 ns  DECODE STATE: Instruction is Decoded

368 ns  Instruction : WRM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

376 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = 80  to Memory Address...

384 ns  EXECUTE_2 STATE: 10 Register Data 89 is Written at Memory Address 80

WRITEEN DATA : RAM[80h]= 89, RAM[81h]= 55, RAM[82h]= 66,  RAM[FO]= xx

- Case 2:

400 ns  FETCH STATE: Instruction is Fetched

408 ns  DECODE STATE: Instruction is Decoded

416 ns  Instruction : LLS, 1

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

424 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 1

AR[00]= 80, DR[01]= 45, GR[10]= 81, PR[11]= 0a

440 ns  FETCH STATE: Instruction is Fetched

448 ns  DECODE STATE: Instruction is Decoded

456 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

464 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 80, DR[01]= 45, GR[10]= 81, PR[11]= 0b

480 ns  FETCH STATE: Instruction is Fetched

488 ns  DECODE STATE: Instruction is Decoded

496 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

504 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 81, DR[01]= 45, GR[10]= 81, PR[11]= 0c

520 ns  FETCH STATE: Instruction is Fetched

528 ns  DECODE STATE: Instruction is Decoded

536 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

544 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 81 to Memory Address...

552 ns  EXECUTE_2 STATE: Read the Data 55 from Memory Location 81 and Stored

into 10 Register

AR[00]= 81, DR[01]= 45, GR[10]= 55, PR[11]= 0d

568 ns  FETCH STATE: Instruction is Fetched

576 ns  DECODE STATE: Instruction is Decoded

584 ns  Instruction : CPR, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

592 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 01

register

AR[00]= 81, DR[01]= 55, GR[10]= 55, PR[11]= 0e

608 ns  FETCH STATE: Instruction is Fetched

616 ns  DECODE STATE: Instruction is Decoded

624 ns  Instruction : LLS, 1

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

632 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 1

AR[00]= 81, DR[01]= 55, GR[10]= 51, PR[11]= 0f

648 ns  FETCH STATE: Instruction is Fetched

656 ns  DECODE STATE: Instruction is Decoded

664 ns  Instruction : LMS, 9

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

672 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 9

AR[00]= 81, DR[01]= 55, GR[10]= 91, PR[11]= 10

688 ns  FETCH STATE: Instruction is Fetched

696 ns  DECODE STATE: Instruction is Decoded

704 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

712 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 91, DR[01]= 55, GR[10]= 91, PR[11]= 11

728 ns  FETCH STATE: Instruction is Fetched

736 ns  DECODE STATE: Instruction is Decoded

744 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

752 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 91 to Memory Address...

760 ns  EXECUTE_2 STATE: Read the Data 77 from Memory Location 91 and Stored

into 10 Register

AR[00]= 91, DR[01]= 55, GR[10]= 77, PR[11]= 12

776 ns  FETCH STATE: Instruction is Fetched

784 ns  DECODE STATE: Instruction is Decoded

792 ns  Instruction : ADD, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

800 ns  EXECUTE_1 STATE: Executing ADD...Providing ALU Opcodes 55 and 77 and

Adding them...

808 ns  EXECUTE_2 STATE: Added the 01 Register Data 55 with 10 Register Data

77 and the Result is Stored into 01 Register

AR[00]= 91, DR[01]= cc, GR[10]= 77, PR[11]= 13

824 ns  FETCH STATE: Instruction is Fetched

832 ns  DECODE STATE: Instruction is Decoded

840 ns  Instruction : LLS, 1

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

848 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 1

AR[00]= 91, DR[01]= cc, GR[10]= 71, PR[11]= 14

864 ns  FETCH STATE: Instruction is Fetched

872 ns  DECODE STATE: Instruction is Decoded

880 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

888 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 91, DR[01]= cc, GR[10]= 81, PR[11]= 15

904 ns  FETCH STATE: Instruction is Fetched

912 ns  DECODE STATE: Instruction is Decoded

920 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

928 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 81, DR[01]= cc, GR[10]= 81, PR[11]= 16

944 ns  FETCH STATE: Instruction is Fetched

952 ns  DECODE STATE: Instruction is Decoded

960 ns  Instruction : WRM, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

968 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = 81  to Memory Address...

976 ns  EXECUTE_2 STATE: 01 Register Data cc is Written at Memory Address 81

WRITTEN DATA : RAM[80h]= 89, RAM[81h]= cc, RAM[82h]= 66,  RAM[FO]= xx

- Case 3:

992 ns  FETCH STATE: Instruction is Fetched

1000 ns DECODE STATE: Instruction is Decoded

1008 ns Instruction : LLS, 2

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1016 ns EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 2

AR[00]= 81, DR[01]= cc, GR[10]= 82, PR[11]= 18

1032 ns  FETCH STATE: Instruction is Fetched

1040 ns  DECODE STATE: Instruction is Decoded

1048 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1056 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 81, DR[01]= cc, GR[10]= 82, PR[11]= 19

1072 ns  FETCH STATE: Instruction is Fetched

1080 ns  DECODE STATE: Instruction is Decoded

1088 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1096 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00 register

AR[00]= 82, DR[01]= cc, GR[10]= 82, PR[11]= 1a

1112 ns  FETCH STATE: Instruction is Fetched

1120 ns  DECODE STATE: Instruction is Decoded

1128 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1136 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 82 to Memory Address...

1144 ns  EXECUTE_2 STATE: Read the Data 66 from Memory Location 82 and Stored into 10 Register

AR[00]= 82, DR[01]= cc, GR[10]= 66, PR[11]= 1b

1160 ns  FETCH STATE: Instruction is Fetched

1168 ns  DECODE STATE: Instruction is Decoded

1176 ns  Instruction : CPR, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1184 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 01

register

AR[00]= 82, DR[01]= 66, GR[10]= 66, PR[11]= 1c

1200 ns  FETCH STATE: Instruction is Fetched

1208 ns  DECODE STATE: Instruction is Decoded

1216 ns  Instruction : LLS, 2

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1224 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 2

AR[00]= 82, DR[01]= 66, GR[10]= 62, PR[11]= 1d

1240 ns  FETCH STATE: Instruction is Fetched

1248 ns  DECODE STATE: Instruction is Decoded

1256 ns  Instruction : LMS, 9

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1264 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 9

AR[00]= 82, DR[01]= 66, GR[10]= 92, PR[11]= 1e

1280 ns  FETCH STATE: Instruction is Fetched

1288 ns  DECODE STATE: Instruction is Decoded

1296 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1304 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 92, DR[01]= 66, GR[10]= 92, PR[11]= 1f

1320 ns  FETCH STATE: Instruction is Fetched

1328 ns  DECODE STATE: Instruction is Decoded

1336 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1344 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 92 to Memory Address...

1352 ns  EXECUTE_2 STATE: Read the Data 88 from Memory Location 92 and Stored into 10 Register

AR[00]= 92, DR[01]= 66, GR[10]= 88, PR[11]= 20

1368 ns  FETCH STATE: Instruction is Fetched

1376 ns  DECODE STATE: Instruction is Decoded

1384 ns  Instruction : SUB, 10, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1392 ns  EXECUTE_1 STATE: Executing SUB...Providing ALU Opcodes 88 and 66 and Adding them...

1400 ns  EXECUTE_2 STATE: Subtracted the 01 Register Data 66 from 10 Register

Data 88 and the Result is Stored into 10 Register

AR[00]= 92, DR[01]= 66, GR[10]= 22, PR[11]= 21

1416 ns  FETCH STATE: Instruction is Fetched

1424 ns  DECODE STATE: Instruction is Decoded

1432 ns  Instruction : LLS, 2

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1440 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 2

AR[00]= 92, DR[01]= 66, GR[10]= 22, PR[11]= 22

1456 ns  FETCH STATE: Instruction is Fetched

1464 ns  DECODE STATE: Instruction is Decoded

1472 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1480 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 92, DR[01]= 66, GR[10]= 82, PR[11]= 23

1496 ns  FETCH STATE: Instruction is Fetched

1504 ns  DECODE STATE: Instruction is Decoded

1512 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1520 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 82, DR[01]= 66, GR[10]= 82, PR[11]= 24

1536 ns  FETCH STATE: Instruction is Fetched

1544 ns  DECODE STATE: Instruction is Decoded

1552 ns  Instruction : WRM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1560 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = 82  to Memory Address...

1568 ns  EXECUTE_2 STATE: 10 Register Data 82 is Written at Memory Address 82

WRITTEN DATA : RAM[80h]= 89, RAM[81h]= cc, RAM[82h]= 82, RAM[FO]= xx

- Case 4:

1584 ns  FETCH STATE: Instruction is Fetched

1592 ns  DECODE STATE: Instruction is Decoded

1600 ns  Instruction : CFR;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1608 ns  EXECUTE_1 STATE: Executed CFR and Stored data  at LSBs of GR = 1

AR[00]= 82, DR[01]= 66, GR[10]= 81, PR[11]= 26

1624 ns  FETCH STATE: Instruction is Fetched

1632 ns  DECODE STATE: Instruction is Decoded

1640 ns  Instruction : CPR, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1648 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 01 register

AR[00]= 82, DR[01]= 81, GR[10]= 81, PR[11]= 27

1664 ns  FETCH STATE: Instruction is Fetched

1672 ns  DECODE STATE: Instruction is Decoded

1680 ns  Instruction : LLS, 0

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1688 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 0

AR[00]= 82, DR[01]= 81, GR[10]= 80, PR[11]= 28

1704 ns  FETCH STATE: Instruction is Fetched

1712 ns  DECODE STATE: Instruction is Decoded

1720 ns  Instruction : LMS, f

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1728 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = f

AR[00]= 82, DR[01]= 81, GR[10]= f0, PR[11]= 29

1744 ns  FETCH STATE: Instruction is Fetched

1752 ns  DECODE STATE: Instruction is Decoded

1760 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1768 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00 register

AR[00]= f0, DR[01]= 81, GR[10]= f0, PR[11]= 2a

1784 ns  FETCH STATE: Instruction is Fetched

1792 ns  DECODE STATE: Instruction is Decoded

1800 ns  Instruction : WRM, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1808 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = f0  to Memory Address...

1816 ns  EXECUTE_2 STATE: 01 Register Data 81 is Written at Memory Address f0

WRITTEN DATA : RAM[80h]= 89, RAM[81h]= cc,  RAM[82h]= 82,  RAM[FO]= 81

1832 ns  FETCH STATE: Instruction is Fetched

1840 ns  DECODE STATE: Instruction is Decoded

1848 ns  Instruction : NOP

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1856 ns  EXECUTE_1 STATE: Executed NOP and  No Operation is Perfomed

- Case 5:

1872 ns  FETCH STATE: Instruction is Fetched

1880 ns  DECODE STATE: Instruction is Decoded

1888 ns  Instruction : LLS, 0

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1896 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 0

AR[00]= f0, DR[01]= 81, GR[10]= f0, PR[11]= 2d

1912 ns  FETCH STATE: Instruction is Fetched

1920 ns  DECODE STATE: Instruction is Decoded

1928 ns  Instruction : LMS, 0

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1936 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 0

AR[00]= f0, DR[01]= 81, GR[10]= 00, PR[11]= 2e

1952 ns  FETCH STATE: Instruction is Fetched

1960 ns  DECODE STATE: Instruction is Decoded

1968 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1976 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 00, DR[01]= 81, GR[10]= 00, PR[11]= 2f

1992 ns  FETCH STATE: Instruction is Fetched

2000 ns  DECODE STATE: Instruction is Decoded

2008 ns  Instruction : JMP

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2016 ns  EXECUTE_1 STATE: Executed JMP and Jumped to Instruction stored at 00

Memory Location

V.	Synthesis and Optimizations of Whole Design (processor)

In the table below, I have shown 6 different trials that I have gone through in synthesizing and optimizing my design. I have set different constrains like Area, Clock etc. and observer the optimized cell area and optimized power for my design.

## Table V.1: Synthesis Constraints and Results

| Trial # | Constraint settings (area, clock, delay, etc.) | Synthesis results (time slack, area, power, etc.) |
| --- | --- | --- |
| 1 | Clock :2.5ns Area 1500 | SLACK: -0.02 (Violated) ; Cell Area:1389 Power: 2.8892 mW |
| 2 | Clock : 3ns Area 1300 | SLACK: 0.00 (Met) ; Cell Area:1163 Power: 2.6742 mW |
| 3 | Clock :4ns Area 1200 | SLACK: 0.00 (Met) ; Cell Area:1108 Power: 1.8995 mW |
| 4 | Clock : 5ns Area 1100 | SLACK: 0.03 (Met) ; Cell Area:1089 Power: 1.8995 mW |
| 5 | Clock : 6ns Area 1000 | SLACK: 0.05 (Met) ; Cell Area:1089 Power: 1.6743 mW |
| 6 | Clock : 7ns Area 900 | SLACK: 0.05 (Met) ; Cell Area:1089 Power: 1.6159 mW |

PR (11)

DR (10)

GR (01)

AR (00)

Figure V.1: Area versus Clock of the Final Design

PR (11)

DR (10)

GR (01)

AR (00)

Figure V.1: Power versus Clock of the Final Design

By, setting different constrains in synthesis script, I observed that when we increase the clock period, the cell area reduces and therefor the dynamic power consumption is also reduced. Moreover, when we increase the clock period the SLACK increases and if we decrease the clock period, the SLACK decreases. The Slack becomes negative when I set the clock period below 3 ns. Plus, the optimize number of cell area I observed was 1108 and the maximum dynamic power of the circuit  is 2.4745 mW.

VI.	Gate-Level Simulations/Tests of Whole Design (processor)

- Case 1:

WRITEEN DATA : RAM[80h]= 44, RAM[81h]= 55,  RAM[82h]= 66,  RAM[FO]= xx

16 ns  FETCH STATE: Instruction is Fetched

24 ns  DECODE STATE: Instruction is Decoded

32 ns  Instruction : LLS, 5

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

40 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 5

AR[00]= 00, DR[01]= 00, GR[10]= 05, PR[11]= 01

56 ns  FETCH STATE: Instruction is Fetched

64 ns  DECODE STATE: Instruction is Decoded

72 ns  Instruction : LMS, 4

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

80 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 4

AR[00]= 00, DR[01]= 00, GR[10]= 45, PR[11]= 02

96 ns  FETCH STATE: Instruction is Fetched

104 ns DECODE STATE: Instruction is Decoded

112 ns Instruction : CPR, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

120 ns EXECUTE_1 STATE: Executed CPR and Copied the Contents of 10 to 01 reg.

AR[00]= 00, DR[01]= 45, GR[10]= 45, PR[11]= 03

136 ns  FETCH STATE: Instruction is Fetched

144 ns  DECODE STATE: Instruction is Decoded

152 ns  Instruction : LLS, 0

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

160 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 0

AR[00]= 00, DR[01]= 45, GR[10]= 40, PR[11]= 04

176 ns  FETCH STATE: Instruction is Fetched

184 ns  DECODE STATE: Instruction is Decoded

192 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

200 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 00, DR[01]= 45, GR[10]= 80, PR[11]= 05

216 ns  FETCH STATE: Instruction is Fetched

224 ns  DECODE STATE: Instruction is Decoded

232 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

240 ns  EXECUTE_1 STATE: Executed CPR and Copied the Contents of 10 to 00 reg.

AR[00]= 80, DR[01]= 45, GR[10]= 80, PR[11]= 06

256 ns  FETCH STATE: Instruction is Fetched

264 ns  DECODE STATE: Instruction is Decoded

272 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

280 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 80 to Memory Address...

288 ns  EXECUTE_2 STATE: Read the Data 44 from Memory Location 80 and Stored

into 10 Register

AR[00]= 80, DR[01]= 45, GR[10]= 44, PR[11]= 07

304 ns  FETCH STATE: Instruction is Fetched

312 ns  DECODE STATE: Instruction is Decoded

320 ns  Instruction : ADD, 10, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

328 ns  EXECUTE_1 STATE: Executing ADD...Providing ALU Opcodes 44 and 45 and

Adding them...

336 ns  EXECUTE_2 STATE: Added the 10 Register Data 44 with 01 Register Data

45 and the Result is Stored into 10 Register

AR[00]= 80, DR[01]= 45, GR[10]= 89, PR[11]= 08

352 ns  FETCH STATE: Instruction is Fetched

360 ns  DECODE STATE: Instruction is Decoded

368 ns  Instruction : WRM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

376 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = 80  to Memory Address...

384 ns  EXECUTE_2 STATE: 10 Register Data 89 is Written at Memory Address 80

WRITEEN DATA : RAM[80h]= 89, RAM[81h]= 55, RAM[82h]= 66,  RAM[FO]= xx

- Case 2:

400 ns  FETCH STATE: Instruction is Fetched

408 ns  DECODE STATE: Instruction is Decoded

416 ns  Instruction : LLS, 1

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

424 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 1

AR[00]= 80, DR[01]= 45, GR[10]= 81, PR[11]= 0a

440 ns  FETCH STATE: Instruction is Fetched

448 ns  DECODE STATE: Instruction is Decoded

456 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

464 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 80, DR[01]= 45, GR[10]= 81, PR[11]= 0b

480 ns  FETCH STATE: Instruction is Fetched

488 ns  DECODE STATE: Instruction is Decoded

496 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

504 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 81, DR[01]= 45, GR[10]= 81, PR[11]= 0c

520 ns  FETCH STATE: Instruction is Fetched

528 ns  DECODE STATE: Instruction is Decoded

536 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

544 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 81 to Memory Address...

552 ns  EXECUTE_2 STATE: Read the Data 55 from Memory Location 81 and Stored

into 10 Register

AR[00]= 81, DR[01]= 45, GR[10]= 55, PR[11]= 0d

568 ns  FETCH STATE: Instruction is Fetched

576 ns  DECODE STATE: Instruction is Decoded

584 ns  Instruction : CPR, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

592 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 01

register

AR[00]= 81, DR[01]= 55, GR[10]= 55, PR[11]= 0e

608 ns  FETCH STATE: Instruction is Fetched

616 ns  DECODE STATE: Instruction is Decoded

624 ns  Instruction : LLS, 1

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

632 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 1

AR[00]= 81, DR[01]= 55, GR[10]= 51, PR[11]= 0f

648 ns  FETCH STATE: Instruction is Fetched

656 ns  DECODE STATE: Instruction is Decoded

664 ns  Instruction : LMS, 9

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

672 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 9

AR[00]= 81, DR[01]= 55, GR[10]= 91, PR[11]= 10

688 ns  FETCH STATE: Instruction is Fetched

696 ns  DECODE STATE: Instruction is Decoded

704 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

712 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 91, DR[01]= 55, GR[10]= 91, PR[11]= 11

728 ns  FETCH STATE: Instruction is Fetched

736 ns  DECODE STATE: Instruction is Decoded

744 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

752 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 91 to Memory Address...

760 ns  EXECUTE_2 STATE: Read the Data 77 from Memory Location 91 and Stored

into 10 Register

AR[00]= 91, DR[01]= 55, GR[10]= 77, PR[11]= 12

776 ns  FETCH STATE: Instruction is Fetched

784 ns  DECODE STATE: Instruction is Decoded

792 ns  Instruction : ADD, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

800 ns  EXECUTE_1 STATE: Executing ADD...Providing ALU Opcodes 55 and 77 and

Adding them...

808 ns  EXECUTE_2 STATE: Added the 01 Register Data 55 with 10 Register Data

77 and the Result is Stored into 01 Register

AR[00]= 91, DR[01]= cc, GR[10]= 77, PR[11]= 13

824 ns  FETCH STATE: Instruction is Fetched

832 ns  DECODE STATE: Instruction is Decoded

840 ns  Instruction : LLS, 1

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

848 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 1

AR[00]= 91, DR[01]= cc, GR[10]= 71, PR[11]= 14

864 ns  FETCH STATE: Instruction is Fetched

872 ns  DECODE STATE: Instruction is Decoded

880 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

888 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 91, DR[01]= cc, GR[10]= 81, PR[11]= 15

904 ns  FETCH STATE: Instruction is Fetched

912 ns  DECODE STATE: Instruction is Decoded

920 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

928 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 81, DR[01]= cc, GR[10]= 81, PR[11]= 16

944 ns  FETCH STATE: Instruction is Fetched

952 ns  DECODE STATE: Instruction is Decoded

960 ns  Instruction : WRM, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

968 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = 81  to Memory Address...

976 ns  EXECUTE_2 STATE: 01 Register Data cc is Written at Memory Address 81

WRITTEN DATA : RAM[80h]= 89, RAM[81h]= cc, RAM[82h]= 66,  RAM[FO]= xx

- Case 3:

992 ns  FETCH STATE: Instruction is Fetched

1000 ns DECODE STATE: Instruction is Decoded

1008 ns Instruction : LLS, 2

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1016 ns EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 2

AR[00]= 81, DR[01]= cc, GR[10]= 82, PR[11]= 18

1032 ns  FETCH STATE: Instruction is Fetched

1040 ns  DECODE STATE: Instruction is Decoded

1048 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1056 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 81, DR[01]= cc, GR[10]= 82, PR[11]= 19

1072 ns  FETCH STATE: Instruction is Fetched

1080 ns  DECODE STATE: Instruction is Decoded

1088 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1096 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00 register

AR[00]= 82, DR[01]= cc, GR[10]= 82, PR[11]= 1a

1112 ns  FETCH STATE: Instruction is Fetched

1120 ns  DECODE STATE: Instruction is Decoded

1128 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1136 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 82 to Memory Address...

1144 ns  EXECUTE_2 STATE: Read the Data 66 from Memory Location 82 and Stored into 10 Register

AR[00]= 82, DR[01]= cc, GR[10]= 66, PR[11]= 1b

1160 ns  FETCH STATE: Instruction is Fetched

1168 ns  DECODE STATE: Instruction is Decoded

1176 ns  Instruction : CPR, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1184 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 01

register

AR[00]= 82, DR[01]= 66, GR[10]= 66, PR[11]= 1c

1200 ns  FETCH STATE: Instruction is Fetched

1208 ns  DECODE STATE: Instruction is Decoded

1216 ns  Instruction : LLS, 2

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1224 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 2

AR[00]= 82, DR[01]= 66, GR[10]= 62, PR[11]= 1d

1240 ns  FETCH STATE: Instruction is Fetched

1248 ns  DECODE STATE: Instruction is Decoded

1256 ns  Instruction : LMS, 9

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1264 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 9

AR[00]= 82, DR[01]= 66, GR[10]= 92, PR[11]= 1e

1280 ns  FETCH STATE: Instruction is Fetched

1288 ns  DECODE STATE: Instruction is Decoded

1296 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1304 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 92, DR[01]= 66, GR[10]= 92, PR[11]= 1f

1320 ns  FETCH STATE: Instruction is Fetched

1328 ns  DECODE STATE: Instruction is Decoded

1336 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1344 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 92 to Memory Address...

1352 ns  EXECUTE_2 STATE: Read the Data 88 from Memory Location 92 and Stored into 10 Register

AR[00]= 92, DR[01]= 66, GR[10]= 88, PR[11]= 20

1368 ns  FETCH STATE: Instruction is Fetched

1376 ns  DECODE STATE: Instruction is Decoded

1384 ns  Instruction : SUB, 10, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1392 ns  EXECUTE_1 STATE: Executing SUB...Providing ALU Opcodes 88 and 66 and Adding them...

1400 ns  EXECUTE_2 STATE: Subtracted the 01 Register Data 66 from 10 Register

Data 88 and the Result is Stored into 10 Register

AR[00]= 92, DR[01]= 66, GR[10]= 22, PR[11]= 21

1416 ns  FETCH STATE: Instruction is Fetched

1424 ns  DECODE STATE: Instruction is Decoded

1432 ns  Instruction : LLS, 2

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1440 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 2

AR[00]= 92, DR[01]= 66, GR[10]= 22, PR[11]= 22

1456 ns  FETCH STATE: Instruction is Fetched

1464 ns  DECODE STATE: Instruction is Decoded

1472 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1480 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 92, DR[01]= 66, GR[10]= 82, PR[11]= 23

1496 ns  FETCH STATE: Instruction is Fetched

1504 ns  DECODE STATE: Instruction is Decoded

1512 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1520 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 82, DR[01]= 66, GR[10]= 82, PR[11]= 24

1536 ns  FETCH STATE: Instruction is Fetched

1544 ns  DECODE STATE: Instruction is Decoded

1552 ns  Instruction : WRM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1560 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = 82  to Memory Address...

1568 ns  EXECUTE_2 STATE: 10 Register Data 82 is Written at Memory Address 82

WRITTEN DATA : RAM[80h]= 89, RAM[81h]= cc, RAM[82h]= 82, RAM[FO]= xx

- Case 4:

1584 ns  FETCH STATE: Instruction is Fetched

1592 ns  DECODE STATE: Instruction is Decoded

1600 ns  Instruction : CFR;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1608 ns  EXECUTE_1 STATE: Executed CFR and Stored data  at LSBs of GR = 1

AR[00]= 82, DR[01]= 66, GR[10]= 81, PR[11]= 26

1624 ns  FETCH STATE: Instruction is Fetched

1632 ns  DECODE STATE: Instruction is Decoded

1640 ns  Instruction : CPR, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1648 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 01 register

AR[00]= 82, DR[01]= 81, GR[10]= 81, PR[11]= 27

1664 ns  FETCH STATE: Instruction is Fetched

1672 ns  DECODE STATE: Instruction is Decoded

1680 ns  Instruction : LLS, 0

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1688 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 0

AR[00]= 82, DR[01]= 81, GR[10]= 80, PR[11]= 28

1704 ns  FETCH STATE: Instruction is Fetched

1712 ns  DECODE STATE: Instruction is Decoded

1720 ns  Instruction : LMS, f

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1728 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = f

AR[00]= 82, DR[01]= 81, GR[10]= f0, PR[11]= 29

1744 ns  FETCH STATE: Instruction is Fetched

1752 ns  DECODE STATE: Instruction is Decoded

1760 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1768 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00 register

AR[00]= f0, DR[01]= 81, GR[10]= f0, PR[11]= 2a

1784 ns  FETCH STATE: Instruction is Fetched

1792 ns  DECODE STATE: Instruction is Decoded

1800 ns  Instruction : WRM, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1808 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = f0  to Memory Address...

1816 ns  EXECUTE_2 STATE: 01 Register Data 81 is Written at Memory Address f0

WRITTEN DATA : RAM[80h]= 89, RAM[81h]= cc,  RAM[82h]= 82,  RAM[FO]= 81

1832 ns  FETCH STATE: Instruction is Fetched

1840 ns  DECODE STATE: Instruction is Decoded

1848 ns  Instruction : NOP

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1856 ns  EXECUTE_1 STATE: Executed NOP and  No Operation is Perfomed

- Case 5:

1872 ns  FETCH STATE: Instruction is Fetched

1880 ns  DECODE STATE: Instruction is Decoded

1888 ns  Instruction : LLS, 0

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1896 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 0

AR[00]= f0, DR[01]= 81, GR[10]= f0, PR[11]= 2d

1912 ns  FETCH STATE: Instruction is Fetched

1920 ns  DECODE STATE: Instruction is Decoded

1928 ns  Instruction : LMS, 0

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1936 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 0

AR[00]= f0, DR[01]= 81, GR[10]= 00, PR[11]= 2e

1952 ns  FETCH STATE: Instruction is Fetched

1960 ns  DECODE STATE: Instruction is Decoded

1968 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1976 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 00, DR[01]= 81, GR[10]= 00, PR[11]= 2f

1992 ns  FETCH STATE: Instruction is Fetched

2000 ns  DECODE STATE: Instruction is Decoded

2008 ns  Instruction : JMP

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2016 ns  EXECUTE_1 STATE: Executed JMP and Jumped to Instruction stored at 00

Memory Location

VII.	Conclusion

The Scalar Processor was successfully completed with all the functionalities implemented and verified. This project was very helpful to me in understanding the basic concept of Verilog design. I also studied tradeoff between speed, area and power consumption. The area and timing constraints were met to design a highly optimized circuit with low power consumption. Also, the speed of operation of the circuit is satisfactory. However, the pipelining was not implemented in this project but the same can be implemented to make the Scalar Processor much faster. Moreover, in this project, many logical  and immediate instructions could be implemented. To encapsulate, the project involved designing the basic microprocessor with memory, and ALU and Control Unit which supports subset of MIPS instructions.

Appendix A

A.1	Contents from EDA Tool Configurations and Setup Files

1. .cshrc

# @(#)cshrc

set filec

set history=100

set prompt="$cwd>[\!] "

limit coredumpsize 0

set path=(~ . /bin /usr/bin /usr/local/bin /usr/kerberos/bin

/usr/sbin /sbin /etc /usr/etc /usr/X11R6/bin /apps/silvaco/bin

/apps/vnc /apps/synopsys /apps/synopsys/installer

/apps/synopsys/CORE/bin /apps/synopsys/SYNTH/bin

/apps/synopsys/SYNTH/linux/bin

/apps/synopsys/SYNTH/linux/syn/bin /apps/synopsys/TCAD/bin

/apps/synopsys/VCSMX_NEW/bin)

setenv MANPATH "/usr/share/man:"

umask 002

setenv MYPATH $PATH

setenv SYNOPSYS /apps/synopsys

setenv SYNOPSYS_SIM $SYNOPSYS/VCSMX_NEW

setenv VCS_HOME $SYNOPSYS_SIM

setenv CLS_CSD_COMPATIBILITY_LOCKING NO

setenv SKIP_CDS_DIALOG

setenv VCS_ARCH_OVERRIDE linux

setenv SNPSLMD $SYNOPSYS/SYNTH

set path=($path $SNPSLMD/linux/bin )

set path=($path $SNPSLMD/linux/syn/bin )

source $SYNOPSYS_SIM/bin/environ.csh

if ( ! $?DISPLAY ) then

set tty = `tty|sed "s,/dev/,,"`

set who = `who | /bin/grep $tty`

set where = `echo $who | awk '{print $6}'`

set loc = `echo $where | sed "s/[(,),:]//g"`

setenv DISPLAY ${loc}:0.0

unset tty who where loc

if ( -f ${locker_desc} ) ${cat} ${locker_desc}

endif

unset base_dir info_dir locker_desc

cat uname

2. .login

if ( $?ENVIRONMENT == 0 ) then

umask 22 EE 271 Final Project Report Summer 2014

Page 25 of 254

set history=100

set noclobber ignoreeof

set ignoreeof

setenv VISUAL /bin/vi

setenv EDITOR /bin/ed

stty erase '^H' kill '^U' intr '^C' echoe tostop

endif

echo "Your HOME is $HOME"

3. .synopsys dc .setup

designer="EE271 Students"

company="San Jose State University, EE Dept."

search_path="/apps/synopsys/SYNTH/libraries/syn"

search_path=search_path + "/apps/synopsys/CORE/libraries/syn"

search_path=search_path + "./src" + "./db"

link_library={"*","class.db","and_or.db","dw_foundation.sldb"}

target_library={"class.db","and_or.db"}

EE271 Final

symbol_library={"class.sdb","generic.sdb"}

synthetic_library={"dw_foundation.sldb","standard.sldb"}

define_design_lib WORK -path ./work

alias rc "report_constraint -all_violators"

view_script_submenu_items={"Clean Sweep","remove_design

_designs"}

edifin_lib_in_port_symbol = "ipin"

edifin_lib_out_port_symbol = "opin"

edifin_lib_inout_port_symbol = "iopin"

edifin_lib_in_osc_symbol = "iooff"

edifin_lib_out_osc_symbol = "ooff"

edifin_lib_inout_osc_symbol = "ioff"

edifin_lib_logic_1_symbol = "vdd"

edifin_lib_logic_0_symbol = "gnd"

edifin_lib_ripper_bus = "bus_end"

edifin_lib_route_grid = 1024

edifin_lib_templates =

{{A,landscape,Asize},{A,portrait,Asize.book},{B,landscape,Bsize}

,{C,landscape,Csize},{D,landscape,Dsize},{E,landscape,Esize},{F,

landscape,Fsize}}

edifin_ground_net_name = "gnd!"

edifin_ground_net_property_name = ""

edifin_ground_net_property_value = ""

edifout_ground_name = "gnd"

edifout_ground_net_name = "gnd!"

edifout_ground_net_property_name = ""

edifout_ground_net_property_value = "" EE 271 Final Project Report Summer 2014

Page 26 of 254

edifout_ground_pin_name = "gnd!"

edifin_power_net_name = "vdd!"

edifin_power_net_property_name = ""

edifin_power_net_property_value = ""

edifout_power_name = "vdd"

edifout_power_net_name = "vdd!"

edifout_power_net_property_name = ""

edifout_power_net_property_value = ""

edifout_power_pin_name = "vdd!"

edifout_power_and_ground_representation = "net"

edifin_autoconnect_ports = "true"

single_group_per_sheet = "true"

use_port_name_for_oscs = "false"

write_name_nets_same_as_ports = "true"

edifout_netlist_only = "false"

edifout_target_system = "cadence"

edifout_instantiate_ports = "true"

A.2	Scripts and/or Commands Used for Simulation and Synthesis

- Commands Used for Simulation of project:

- vcs +v2k 271control.v scalar_test.v ; ./simv | tee PRE_SYNTHESIS.txt = to simulate the file

- gtkwave scalar.vcd & = to check gtkwave of code.

- dc_shell -xg -f synthesis.script | tee SYNTHESIS.txt = to check the different design constrains.

- vcs +v2k -y /export/apps/toshiba/sjsu/verilog/tc240c +libext+.tsbvlibp scalar_test.v scalar_netlist.v ; ./simv | tee postsy.txt; = to synthesis the code with particular library.

- Script for Scalar Processor:

set link_library {/apps/toshiba/sjsu/synopsys/tc240c/tc240c.db_WCCOM25}

set target_library {/apps/toshiba/sjsu/synopsys/tc240c/tc240c.db_WCCOM25}

set symbol_library {/apps/toshiba/sjsu/synopsys/tc240c/tc240c.workview.sdb}

set synthetic_library {dw_foundation.sldb standard.sldb}

set_min_library /apps/toshiba/sjsu/synopsys/tc240c/tc240c.db_WCCOM25 -min_version /apps/toshiba/sjsu/synopsys/tc240c/tc240c.db_WCCOM25

read_verilog 271control.v

current_design SCALAR_PROCESSOR

link

check_design

create_clock CLK -name CLK -period 3.0000000

set_propagated_clock CLK

set_clock_uncertainty .25 CLK

set_max_delay 1 -from [all_inputs]

set_max_delay 1 -to [all_outputs]

#set_max_delay 2 -to sum

set_load 160 sum

set_max_area 900

compile -map_effort high

report_cell

report_net

update_timing

report_timing -max_paths 10

report_area 5000

report_power

write -hierarchy -format verilog -output scalar_netlist.v

quit

Appendix B

Completed Verilog Source Codes and Testbenches

- SCALAR PROCESSOR (CONTROLLER)

//File Name: 271control.v

`timescale 1ns/10ps

//------------------- DEFINITION OF STATES ---------------------

`define IDEL 3'b000

`define FETCH  3'b001

`define DECODE 3'b010

`define UPDATE 3'b011

`define EXECUTE_1 3'b100

`define EXECUTE_2 3'b101

//---------------- DEFINITION OF INSTRUCTIONS ------------------

`define NOP    4'b0000

`define JMP    4'b0001

`define RDM    4'b0010

`define WRM    4'b0011

`define CPR    4'b0100

`define ADD    4'b0101

`define SUB    4'b0110

`define LLS    4'b0111

`define LMS    4'b1000

`define CFR    4'b1001

//---------------SCALAR PROCESSOR VERILOG CODE -----------------

module SCALAR_PROCESSOR(dat,addr, rd, wrt, clk, rst);

//-------------------- OUTPUT SIGNALS --------------------------

output rd, wrt;

output [7:0] addr;

//-------------------- INPUT SIGNALS ---------------------------

input  clk, rst;

//-------------------- INOUT SIGNALS ---------------------------

inout [7:0] dat ;

//---------------------- STATE NETS ----------------------------

reg [2:0] state;

reg [2:0] n_state;

//---------------- ALU REGISTERS & WIRES -----------------------

reg [7:0]  OP1, OP2;

reg OPRN,sign,carry,zero;

wire [7:0] ALU_OUT;

wire cin,cout,ov;

//---------------SCALAR PROCESSOR INTERNAL REGISTERS -----------

reg [7:0]  addr;

reg rd, wrt;

reg [3:0] opcode;

reg [1:0] D,S;

reg [7:0] ARRAY [3:0];

reg [7:0] AR, DR, GR, PR,dat_T;

reg [3:0] FR;

//--------------------ALU MODULE INSTANTIATE -------------------

ALU alu23 (.sum(ALU_OUT),.a(OP1),.b(OP2),.cin(OPRN),.cout(cout),.ov(ov));

//--------------------- SCALAR PROCESSOR -----------------------

assign  dat = ((rd==1'b0)&&(wrt==1'b1))? dat_T:{8{1'bz} };

always @(*)

begin

AR = ARRAY [0];

DR = ARRAY [1];

GR = ARRAY [2];

PR = ARRAY [3];

end

always @ (ALU_OUT)

begin

if (ALU_OUT ==0)

zero = 1'b1;

else zero = 1'b0;

if (ALU_OUT[7] ==1'b1)

sign = 1'b1;

else sign =1'b0;

if ((!OPRN && cout) | (OPRN && !cout))

carry=1'b1;

else carry=1'b0;

end

always @ (posedge (clk) or posedge (rst))

begin

if(rst)

state <= `IDEL;

else state <= n_state;

end

always @ (*)

begin

case(state)

`IDEL  : n_state = `FETCH;

`FETCH : n_state = `DECODE;

`DECODE: n_state = `UPDATE;

`UPDATE: n_state = `EXECUTE_1;

`EXECUTE_1:

begin

if (opcode == `NOP || opcode==`LLS || opcode==`LMS || opcode==`CPR || opcode==`CFR || opcode==`JMP)

n_state = `IDEL;

else n_state = `EXECUTE_2;

end

`EXECUTE_2:

begin

n_state = `IDEL;

end

endcase

end

always @ (posedge clk or posedge rst)

begin

if (rst)

begin

AR <= 0;

GR <= 0;

DR <= 0;

PR <= 0;

FR <= 0;

OP1 <= 0;

OP2 <= 0;

ARRAY[3] <=0;

ARRAY[2] <=0;

ARRAY[1] <=0;

ARRAY[0] <=0;

addr <= 0;

rd <= 0;

wrt <= 0;

dat_T <=0;

D <= 0;

S <= 0;

opcode <= 0;

OPRN <= 0;

end

else

begin

rd <= 0;

wrt <= 0;

case(n_state)

//------------------------ FETCH STATE -------------------------

`FETCH:

begin

addr <= ARRAY[3];

rd <= 1'b1;

wrt <= 1'b0;

end

//------------------------ DECODE STATE ------------------------

`DECODE:

begin

rd <=1'b1;

wrt <=1'b0;

{opcode,D,S}  <= dat;

end

//------------------------ UPDATE STATE ------------------------

`UPDATE:

begin

rd <=1'b0;

wrt <=1'b0;

case(opcode)

`NOP:

begin

$write($time,"  NOP\n");

ARRAY [3] <= ARRAY[3]+1;

end

`JMP:

begin

$write($time,"  JMP \n");

end

`RDM:

begin

$write($time,"  RDM, %02b;\n", D);

ARRAY [3] <= ARRAY[3]+1;

end

`WRM:

begin

$write($time,"  WRM, %02b;\n", D);

ARRAY [3] <= ARRAY[3]+1;

end

`CPR:

begin

$write($time, "  CPR, %02b, %02b;\n", D ,S);

ARRAY [3] <= ARRAY[3]+1;

end

`ADD:

begin

$write($time,"  ADD, %02b, %02b;\n",D,S);

ARRAY [3] <= ARRAY[3]+1;

end

`SUB:

begin

$write($time,"  SUB, %02b, %02b;\n", D ,S);

ARRAY [3] <= ARRAY[3]+1;

end

`LLS:

begin

$write($time,"  LLS, %02h\n",{D,S});

ARRAY [3] <= ARRAY[3]+1;

end

`LMS:

begin

$write($time,"  LMS, %02h\n",{D,S});

ARRAY [3] <= ARRAY[3]+1;

end

`CFR:

begin

$write($time,"  CFR;\n");

ARRAY [3] <= ARRAY[3]+1;

end

default: $write("");

endcase

end

//-------------------- EXECUTE_1 STATE -------------------------

`EXECUTE_1:

begin

case(opcode)

`NOP:

begin

$write($time,”Executed NOP and  No Operation is Perfomed\n\n");

end

`JMP:

begin

ARRAY[3] <= ARRAY[0];

rd <= 1'b0;

wrt <= 1'b0;

$write($time,"  Executed JMP and Jumped to Instruction stored at %08H Memory Location\n\n",ARRAY[0]);

end

`RDM:

begin

addr <= ARRAY[0];

rd <= 1'b1;

wrt <= 1'b0;

$write($time,"  Executing RDM...Setting AR = %08h to Memory Address...  \n",ARRAY[0]);

end

`WRM:

begin

addr <= ARRAY[0];

dat_T <= ARRAY[D];

rd <= 1'b0;

wrt<=1'b1;

$write($time,"  Executing WRM...Setting AR = %08h  to Memory Address...  \n",ARRAY[0]);

end

`CPR:

begin

ARRAY[D] <= ARRAY[S];

rd <= 1'b0;

wrt <= 1'b0;

$write($time,"  Executed CPR and  Copied the Contents of %02b to %02b register\n",S,D);

end

`ADD:

begin

OP1 <= ARRAY[D];

OP2 <= ARRAY[S];

OPRN <= 1'b0;

rd <= 1'b0;

wrt <= 1'b0;

$write($time,"  Executing ADD...Providing ALU Opcodes %08h and %08h and Adding them...   \n",ARRAY[D],ARRAY[S]);

end

`SUB:

begin

OP1 <= ARRAY[D];

OP2 <= ARRAY[S];

OPRN <= 1'b1;

rd <= 1'b0;

wrt <= 1'b0;

$write($time,"  Executing SUB...Providing ALU Opcodes %08h and %08h and Adding them...   \n",ARRAY[D],ARRAY[S]);

end

`LLS:

begin

ARRAY[2][3:0] <= {D,S};

rd <= 1'b0;

wrt <= 1'b0;

$write($time,"  Executed LLS and Stored data at LSBs of GR is %02h\n",{D,S});

end

`LMS:

begin

ARRAY[2][7:4] <= {D,S};

rd <= 1'b0;

wrt <= 1'b0;

$write($time,"  Executed LMS and Stored data at MSBs of GR is %02h\n",{D,S});

end

`CFR:

begin

ARRAY[2][3:0] <= FR;

rd <= 1'b0;

wrt <= 1'b0;

$write($time,"  Executed CFR and Stored data  at LSBs of GR = %02h\n",FR);

End

default:$write(" INVALID INSTRUCTION ");

endcase

end

//-------------------- EXECUTE_2 STATE ---------------------

`EXECUTE_2:

begin

case(opcode)

`JMP:

begin

ARRAY[3] <= ARRAY[0];

rd <= 1'b0;

wrt <= 1'b0;

end

`RDM:

begin

ARRAY[D] <= dat;

rd <= 1'b1;

wrt <= 1'b0;

$write($time,"  Read the Data %08h from Memory Location %08h and Stored into %02b Register\n",dat,addr,D);

end

`WRM:

begin

$write($time,"  %02b Register Data %08h is Written at Memory Address %08h\n",D,ARRAY[D],addr);

end

`CPR:

begin

rd <= 1'b0;

wrt <= 1'b0;

end

`ADD:

begin

ARRAY[D] <= ALU_OUT;

FR <= {zero,sign,carry,ov};

rd <= 1'b0;

wrt <= 1'b0;

$write($time,"  Added the %02b Register Data %08h with %02b Register Data %08h and the Result is Stored into %02b Register\n",D,ARRAY[D],S,ARRAY[S],D);

end

`SUB:

begin

ARRAY[D] <= ALU_OUT;

FR <= {zero,sign,carry,ov};

rd <= 1'b0;

wrt <= 1'b0;

$write($time,"  Subtracted the %02b Register Data %08h from %02b Register Data %08h and the Result is Stored into %02b Register\n",S,ARRAY[S],D,ARRAY[D],D);

end

`LLS:

begin

rd <= 1'b0;

wrt <= 1'b0;

end

`LMS:

begin

rd <= 1'b0;

wrt <= 1'b0;

end

`CFR:

begin

rd <= 1'b0;

wrt <= 1'b0;

end

default: $write("");

endcase

end

//-------------------- IDEL STATE -------------------------

`IDEL:

begin

rd <=1'b0;

wrt <=1'b0;

case(opcode)

`RDM:

begin

$write("AR[00]= %08h, DR[01]= %08h, GR[10]= %08h, PR[11]= %08h\n\n",ARRAY[0],ARRAY[1],ARRAY[2],ARRAY[3]);

end

`CPR:

begin

$write("AR[00]= %08h, DR[01]= %08h, GR[10]= %08h, PR[11]= %08h\n\n",ARRAY[0],ARRAY[1],ARRAY[2],ARRAY[3]);

end

`ADD:

begin

$write("AR[00]= %08h, DR[01]= %08h, GR[10]= %08h, PR[11]= %08h\n\n",ARRAY[0],ARRAY[1],ARRAY[2],ARRAY[3]);

end

`SUB:

begin

$write("AR[00]= %08h, DR[01]= %08h, GR[10]= %08h, PR[11]= %08h\n\n",ARRAY[0],ARRAY[1],ARRAY[2],ARRAY[3]);

end

`LLS:

begin

$write("AR[00]= %08h, DR[01]= %08h, GR[10]= %08h, PR[11]= %08h\n\n",ARRAY[0],ARRAY[1],ARRAY[2],ARRAY[3]);

end

`LMS:

begin

$write("AR[00]= %08h, DR[01]= %08h, GR[10]= %08h, PR[11]= %08h\n\n",ARRAY[0],ARRAY[1],ARRAY[2],ARRAY[3]);

end

`CFR:

begin

$write("AR[00]= %08h, DR[01]= %08h, GR[10]= %08h, PR[11]= %08h\n\n",ARRAY[0],ARRAY[1],ARRAY[2],ARRAY[3]);

end

default: $write("");

endcase

end

endcase

end

end

endmodule

- Arithmetic and Logic Unit (ALU)

module ALU(sum,cout,a, b,cin,ov);

input [7:0] a;

input [7:0] b;

input cin;

output [7:0] sum;

output cout,ov;

reg [7:0] d;

wire cin7;

RCA_8 n1_inst (sum, cout, a, d, cin, cin7);

assign ov = cin7^cout

always @ (*)

begin

if(!cin)

begin

d = b;

end

else if (cin)

begin

d = ~b;

end

end

endmodule

- 8 BIT RIPPLE CARRY ADDER

module RCA_8 (sum, cout, a, b, cin,cin7);

output [7:0] sum;

output cout,ov;

input [7:0] a, b;

input cin;

wire cin1, cin2, cin3,cin4,cin5,cin6;

add_full U1 (sum[0], cin1, a[0], b[0], cin);

add_full U2 (sum[1], cin2, a[1], b[1], cin1);

add_full U3 (sum[2], cin3, a[2], b[2], cin2);

add_full U4 (sum[3], cin4, a[3], b[3], cin3);

add_full U5 (sum[4], cin5, a[4], b[4], cin4);

add_full U6 (sum[5], cin6, a[5], b[5], cin5);

add_full U7 (sum[6], cin7, a[6], b[6], cin6);

add_full U8 (sum[7], cout, a[7], b[7], cin7);

endmodule

- FULL ADDER

module add_full (sum, cout, a, b, cin);

input a, b, cin;

output cout, sum;

wire w1, w2, w3;

add_half U1 (w1, w2, a, b);

add_half U2 (sum, w3, cin, w1);

assign cout= w2|w3;

endmodule

- HALF ADDER

module add_half (sum, cout, a, b);

input a, b;

output cout, sum;

assign sum = a^b;

assign cout= a&b;

endmodule

- Testbench for Scalar Processor

//File name: Scalar_test.v

`timescale 1ns/10ps;

`define AR   2'b00

`define DR   2'b01

`define GR   2'b10

`define PR   2'b11

`define NOP    4'b0000

`define JMP    4'b0001

`define RDM    4'b0010

`define WRM    4'b0011

`define CPR    4'b0100

`define ADD    4'b0101

`define SUB    4'b0110

`define LLS    4'b0111

`define LMS    4'b1000

`define CFR    4'b1001

`define IDEL 3'b000

`define FETCH  3'b001

`define DECODE 3'b010

`define UPDATE 3'b011

`define EXECUTE_1 3'b100

`define EXECUTE_2 3'b101

module SCALAR_TEST;

wire [7:0] addr;

wire rd, wrt, clk;

wire [7:0] dat;

// reset

reg rst;

reg [2:0] state, n_state;

//MEMORY STORAGE

reg [7:0] RAM [0:255];

reg [3:0] opcode;

reg [1:0] D,S;

//Integers for RESET and Display

integer u;

// CLOCK INSTANCE

clk_GENERATOR clk_gen_inst(.clk(clk));

// SCALAR PROCESSOR INSTANCE

SCALAR_PROCESSORS 123 (.dat(dat,.addr(addr,.rd(rd),.wrt(wrt),.clk(clk),.rst(rst));

initial begin

$dumpfile ("scalar.vcd");

$dumpvars (0,SCALAR_TEST);

end

initial

begin

rst=1'b1;

#11 rst=1'b0;

//----------------------INSTRUCTION MEMORY ---------------------

//    1)  ADD AN 8-BIT DATA AT MEMORY LOCATION 80H TO AN IMMEDIATE DATA 45H AND STORE THE RESULT TO MEMORY LOCATION 80H    //

RAM[0] = {`LLS,4'h5};

RAM[1] = {`LMS,4'h4};

RAM[2] = {`CPR,`DR,`GR};

RAM[3] = {`LLS,4'h0};

RAM[4] = {`LMS,4'h8};

RAM[5] = {`CPR,`AR,`GR};

RAM[6] = {`RDM,`GR,2'b00};

RAM[7] = {`ADD,`GR,`DR};

RAM[8] = {`WRM,`GR,2'b00};

//    2)  ADD AN 8-BIT DATA AT MEMORY LOCATION 81H AND 91H AND STORE THE RESULT TO MEMORY LOCATION 81H    //

RAM[9]  = {`LLS,4'h1};

RAM[10]  = {`LMS,4'h8};

RAM[11] = {`CPR,`AR,`GR};

RAM[12] = {`RDM,`GR,2'b00};

RAM[13] = {`CPR,`DR,`GR};

RAM[14] = {`LLS,4'h1};

RAM[15] = {`LMS,4'h9};

RAM[16] = {`CPR,`AR,`GR};

RAM[17] = {`RDM,`GR,2'b00};

RAM[18] = {`ADD,`DR,`GR};

RAM[19] = {`LLS,4'h1};

RAM[20] = {`LMS,4'h8};

RAM[21] = {`CPR,`AR,`GR};

RAM[22] = {`WRM,`DR,2'b00};

//     3)   SUBSTRACT AN 8-BIT DATA AT MEMORY LOCATION 82H FROM ANOTHER 8-BIT DATA AT MEMORY LOCATION 91H AND STORE THE RESULT TO MEMORY LOCATION 81H    //

RAM[23] = {`LLS,4'h2};

RAM[24] = {`LMS,4'h8};

RAM[25] = {`CPR,`AR,`GR};

RAM[26] = {`RDM,`GR,2'b00};

RAM[27] = {`CPR,`DR,`GR};

RAM[28] = {`LLS,4'h2};

RAM[29] = {`LMS,4'h9};

RAM[30] = {`CPR,`AR,`GR};

RAM[31] = {`RDM,`GR,2'b00};

RAM[32] = {`SUB,`GR,`DR};

RAM[33] = {`LLS,4'h2};

RAM[34] = {`LMS,4'h8};

RAM[35] = {`CPR,`AR,`GR};

RAM[36] = {`WRM,`GR,2'b00};

//     4)    STORE THE CONTENTS OF 4-BIT FLAG REGISTER INTO LEAST SIGNIFICANT 4-BIT OF MEMORY LOCATION F0H       //

RAM[37] = {`CFR,4'h0};

RAM[38] = {`CPR,`DR,`GR};

RAM[39] = {`LLS,4'h0};

RAM[40] = {`LMS,4'hF};

RAM[41] = {`CPR,`AR,`GR};

RAM[42] = {`WRM,`DR,2'b00};

RAM[43] = {`NOP,4'h0};

//     5)   USE THE JMP INSTRUCTION TO REPEAT THE SEPS FROM 1) TO 4) ONE MORE TIME   //

RAM[44] = {`LLS,4'h0};

RAM[45] = {`LMS,4'h0};

RAM[46] = {`CPR,`AR,`GR};

RAM[47] = {`JMP,4'h0};

//----------------------DATA MEMORY ---------------------------

RAM[8'h80] = 8'h44;

RAM[8'h81] = 8'h55;

RAM[8'h82] = 8'h66;

RAM[8'h91] = 8'h77;

RAM[8'h92] = 8'h88;

for (u=0;u<=255;u=u+1)

begin

$display("Address=  %02h, Data at %02h = %02h ",u,u,RAM[u]);

end

$monitor ("                         WRITEEN DATA : RAM[80h]= %h, RAM[81h]= %h,  RAM[82h]= %h,  RAM[FO]= %h\n",RAM[8'h80],RAM[8'h81],RAM[8'h82],RAM[8'hf0]);

#3874 $finish;

end

//------------------------ MEMORY BANK -------------------------

always @ (posedge (clk) or posedge (rst))

begin

if(rst)

state <= `IDEL;

else state <= n_state;

end

always @ (*)

begin

case(state)

`IDEL  : n_state = `FETCH;

`FETCH : n_state = `DECODE;

`DECODE: n_state = `UPDATE;

`UPDATE: n_state = `EXECUTE_1;

`EXECUTE_1:begin

if (opcode == `NOP || opcode==`LLS || opcode==`LMS || opcode==`CPR || opcode==`CFR || opcode==`JMP)

n_state = `IDEL;

else n_state = `EXECUTE_2;

end

`EXECUTE_2:begin

n_state = `IDEL;

end

endcase

end

always @ (posedge clk )

begin

case(n_state)

//---------------------------- FETCH STATE ---------------------

`FETCH:

begin

$write($time," ns  FETCH STATE: Instruction is Fetched\n");

end

//-----------------------------DECODE STATE --------------------   `DECODE:

begin

$write($time," ns  DECODE STATE: Instruction is Decoded\n");

{opcode,D,S} <= dat;

end

//------------------------- UPDATE STATE -----------------------

`UPDATE:

begin

case(opcode)

`NOP:

begin

$write($time," ns  Instruction : NOP\n");

$write("UPDATE STATE: PR Register is Incremented to Fetch the next Instruction. . .\n");

end

`JMP:

begin

$write($time," ns  Instruction : JMP \n");

$write("UPDATE STATE: PR Register is Incremented to Fetch the next Instruction. . .\n");

end

`RDM:

begin

$write($time," ns  Instruction : RDM,%02h ;\n",D);

$write("UPDATE STATE: PR Register is Incremented to Fetch the next Instruction. . .\n");

end

`WRM:

begin

$write($time," ns  Instruction : WRM,%02h ;\n",S);

$write("UPDATE STATE: PR Register is Incremented to Fetch the next Instruction. . .\n");

end

`CPR:

begin

$write($time, " ns  Instruction : CPR,%02h,%02h ;\n ",D,S);

$write("                         UPDATE STATE: PR Register is Incremented to Fetch the next Instruction. . .\n");

end

`ADD:

begin

$write($time," ns  Instruction : ADD,%02h,%02h ;\n ",D,S);

$write("UPDATE STATE: PR Register is Incremented to Fetch the next Instruction. . .\n");

end

`SUB:

begin

$write($time," ns  Instruction : SUB,%02h,%02h ;\n ",D,S);

$write("UPDATE STATE: PR Register is Incremented to Fetch the next Instruction. . .\n");

end

`LLS:

begin

$write($time," ns  Instruction : LLS,%02h ;\n ",dat[3:0]);

$write("UPDATE STATE: PR Register is Incremented to Fetch the next Instruction. . .\n");

end

`LMS:

begin

$write($time," ns  Instruction : LMS,%02h ;\n",dat[3:0]);

$write("UPDATE STATE: PR Register is Incremented to Fetch the next Instruction. . .\n");

end

`CFR:

begin

$write($time," ns  Instruction : CFR;\n");

$write("UPDATE STATE: PR Register is Incremented to Fetch the next Instruction. . .\n");

//ARRAY [3] <= ARRAY[3]+1;

end

default: $write("");

endcase

end

//------------------------- EXECUTE_1 STATE --------------------

`EXECUTE_1:

begin

case(opcode)

`NOP:

begin

$write($time," ns  EXECUTE_1 STATE: Executed NOP and  No Operation is Perfomed\n\n");

end

`JMP:

begin

$write($time," ns  EXECUTE_1 STATE: Executed JMP and Jumped to Instruction stored at Memory Location 00H\n\n");

end

`RDM:

begin

$write($time," ns  EXECUTE_1 STATE: Executing RDM...Setting AR to Memory Address...  \n");

end

`WRM:

begin

$write($time," ns  EXECUTE_1 STATE: Executing WRM...Setting AR  to Memory Address...  \n");

end

`CPR:

begin

$write($time," ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of register %02h to %02h ;\n ",S,D);

end

`ADD:

begin

$write($time," ns  EXECUTE_1 STATE: Executing ADD...Providing ALU inputs %02h and %02h Register Contents and Adding them...\n",S,D);

end

`SUB:

begin

$write($time," ns  EXECUTE_1 STATE: Executing SUB...Providing ALU inputs %02h and %02h Register Contents and Subtracting them...\n",S,D);

end

`LLS:

begin

$write($time," ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR \n");

end

`LMS:

begin

$write($time," ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR \n");

end

`CFR:

begin

$write($time," ns  EXECUTE_1 STATE: Executed CFR and Stored data FR to LSBs of GR \n");

end

default:$write($time," ns EXECUTE_1 STATE: INVALID INSTRUCTION ");

endcase

end

`EXECUTE_2:

begin

case(opcode)

`RDM:

begin

$write($time," ns  EXECUTE_2 STATE: Read the Data %08h from Memory Location %08h and Stored into %02b Register\n",dat,addr,D);

end

`WRM:

begin

$write($time," ns  EXECUTE_2 STATE: %02b Register Data is Written at Memory Address %08h\n",D,addr);

end

`ADD:

begin

$write($time," ns  EXECUTE_2 STATE: Added the %02b Register Data with %02b Register Data and the Result is Stored into %02b Register\n",D,S,D);

end

`SUB:

begin

$write($time," ns  EXECUTE_2 STATE: Subtracted the %02b Register Data from %02b Register Data and the Result is Stored into %02b Register\n",S,D,D);

end

endcase

end

`IDEL:

begin

end

endcase

end

assign dat = ((rd==1'b1)&&(wrt==1'b0))? RAM [addr]:{8{1'bz} };

always @ (posedge clk)

begin

if ((rd==1'b0)&&(wrt==1'b1)) // write operation

RAM [addr] <=  dat;

end

endmodule

//-----------------------CLOCK GENERATOR MODULE ----------------

module clk_GENERATOR(clk);

output clk;

reg clk;

initial

begin

clk = 1'b1;

end

always

begin

#4 clk <= ~clk;

end

endmodule

Appendix C

Reports and Circuits from EDA Tools

C.1	Reports (contents) from RTL (Pre-synthesis) Simulations (VCS or NCVERILOG)

Chronologic VCS simulator copyright 1991-2014

Contains Synopsys proprietary information.

Compiler version I-2014.03-2; Runtime version I-2014.03-2;  Dec  5 03:56 2014

Address=  00, Data at 00 = 75

Address=  01, Data at 01 = 84

Address=  02, Data at 02 = 46

Address=  03, Data at 03 = 70

Address=  04, Data at 04 = 88

Address=  05, Data at 05 = 42

Address=  06, Data at 06 = 28

Address=  07, Data at 07 = 59

Address=  08, Data at 08 = 38

Address=  09, Data at 09 = 71

Address=  0a, Data at 0a = 88

Address=  0b, Data at 0b = 42

Address=  0c, Data at 0c = 28

Address=  0d, Data at 0d = 46

Address=  0e, Data at 0e = 71

Address=  0f, Data at 0f = 89

Address=  10, Data at 10 = 42

Address=  11, Data at 11 = 28

Address=  12, Data at 12 = 56

Address=  13, Data at 13 = 71

Address=  14, Data at 14 = 88

Address=  15, Data at 15 = 42

Address=  16, Data at 16 = 34

Address=  17, Data at 17 = 72

Address=  18, Data at 18 = 88

Address=  19, Data at 19 = 42

Address=  1a, Data at 1a = 28

Address=  1b, Data at 1b = 46

Address=  1c, Data at 1c = 72

Address=  1d, Data at 1d = 89

Address=  1e, Data at 1e = 42

Address=  1f, Data at 1f = 28

Address=  20, Data at 20 = 69

Address=  21, Data at 21 = 72

Address=  22, Data at 22 = 88

Address=  23, Data at 23 = 42

Address=  24, Data at 24 = 38

Address=  25, Data at 25 = 90

Address=  26, Data at 26 = 46

Address=  27, Data at 27 = 70

Address=  28, Data at 28 = 8f

Address=  29, Data at 29 = 42

Address=  2a, Data at 2a = 34

Address=  2b, Data at 2b = 00

Address=  2c, Data at 2c = 70

Address=  2d, Data at 2d = 80

Address=  2e, Data at 2e = 42

Address=  2f, Data at 2f = 10

Address=  30, Data at 30 = xx

Address=  31, Data at 31 = xx

Address=  32, Data at 32 = xx

Address=  33, Data at 33 = xx

Address=  34, Data at 34 = xx

Address=  35, Data at 35 = xx

Address=  36, Data at 36 = xx

Address=  37, Data at 37 = xx

Address=  38, Data at 38 = xx

Address=  39, Data at 39 = xx

Address=  3a, Data at 3a = xx

Address=  3b, Data at 3b = xx

Address=  3c, Data at 3c = xx

Address=  3d, Data at 3d = xx

Address=  3e, Data at 3e = xx

Address=  3f, Data at 3f = xx

Address=  40, Data at 40 = xx

Address=  41, Data at 41 = xx

Address=  42, Data at 42 = xx

Address=  43, Data at 43 = xx

Address=  44, Data at 44 = xx

Address=  45, Data at 45 = xx

Address=  46, Data at 46 = xx

Address=  47, Data at 47 = xx

Address=  48, Data at 48 = xx

Address=  49, Data at 49 = xx

Address=  4a, Data at 4a = xx

Address=  4b, Data at 4b = xx

Address=  4c, Data at 4c = xx

Address=  4d, Data at 4d = xx

Address=  4e, Data at 4e = xx

Address=  4f, Data at 4f = xx

Address=  50, Data at 50 = xx

Address=  51, Data at 51 = xx

Address=  52, Data at 52 = xx

Address=  53, Data at 53 = xx

Address=  54, Data at 54 = xx

Address=  55, Data at 55 = xx

Address=  56, Data at 56 = xx

Address=  57, Data at 57 = xx

Address=  58, Data at 58 = xx

Address=  59, Data at 59 = xx

Address=  5a, Data at 5a = xx

Address=  5b, Data at 5b = xx

Address=  5c, Data at 5c = xx

Address=  5d, Data at 5d = xx

Address=  5e, Data at 5e = xx

Address=  5f, Data at 5f = xx

Address=  60, Data at 60 = xx

Address=  61, Data at 61 = xx

Address=  62, Data at 62 = xx

Address=  63, Data at 63 = xx

Address=  64, Data at 64 = xx

Address=  65, Data at 65 = xx

Address=  66, Data at 66 = xx

Address=  67, Data at 67 = xx

Address=  68, Data at 68 = xx

Address=  69, Data at 69 = xx

Address=  6a, Data at 6a = xx

Address=  6b, Data at 6b = xx

Address=  6c, Data at 6c = xx

Address=  6d, Data at 6d = xx

Address=  6e, Data at 6e = xx

Address=  6f, Data at 6f = xx

Address=  70, Data at 70 = xx

Address=  71, Data at 71 = xx

Address=  72, Data at 72 = xx

Address=  73, Data at 73 = xx

Address=  74, Data at 74 = xx

Address=  75, Data at 75 = xx

Address=  76, Data at 76 = xx

Address=  77, Data at 77 = xx

Address=  78, Data at 78 = xx

Address=  79, Data at 79 = xx

Address=  7a, Data at 7a = xx

Address=  7b, Data at 7b = xx

Address=  7c, Data at 7c = xx

Address=  7d, Data at 7d = xx

Address=  7e, Data at 7e = xx

Address=  7f, Data at 7f = xx

Address=  80, Data at 80 = 44

Address=  81, Data at 81 = 55

Address=  82, Data at 82 = 66

Address=  83, Data at 83 = xx

Address=  84, Data at 84 = xx

Address=  85, Data at 85 = xx

Address=  86, Data at 86 = xx

Address=  87, Data at 87 = xx

Address=  88, Data at 88 = xx

Address=  89, Data at 89 = xx

Address=  8a, Data at 8a = xx

Address=  8b, Data at 8b = xx

Address=  8c, Data at 8c = xx

Address=  8d, Data at 8d = xx

Address=  8e, Data at 8e = xx

Address=  8f, Data at 8f = xx

Address=  90, Data at 90 = xx

Address=  91, Data at 91 = 77

Address=  92, Data at 92 = 88

Address=  93, Data at 93 = xx

Address=  94, Data at 94 = xx

Address=  95, Data at 95 = xx

Address=  96, Data at 96 = xx

Address=  97, Data at 97 = xx

Address=  98, Data at 98 = xx

Address=  99, Data at 99 = xx

Address=  9a, Data at 9a = xx

Address=  9b, Data at 9b = xx

Address=  9c, Data at 9c = xx

Address=  9d, Data at 9d = xx

Address=  9e, Data at 9e = xx

Address=  9f, Data at 9f = xx

Address=  a0, Data at a0 = xx

Address=  a1, Data at a1 = xx

Address=  a2, Data at a2 = xx

Address=  a3, Data at a3 = xx

Address=  a4, Data at a4 = xx

Address=  a5, Data at a5 = xx

Address=  a6, Data at a6 = xx

Address=  a7, Data at a7 = xx

Address=  a8, Data at a8 = xx

Address=  a9, Data at a9 = xx

Address=  aa, Data at aa = xx

Address=  ab, Data at ab = xx

Address=  ac, Data at ac = xx

Address=  ad, Data at ad = xx

Address=  ae, Data at ae = xx

Address=  af, Data at af = xx

Address=  b0, Data at b0 = xx

Address=  b1, Data at b1 = xx

Address=  b2, Data at b2 = xx

Address=  b3, Data at b3 = xx

Address=  b4, Data at b4 = xx

Address=  b5, Data at b5 = xx

Address=  b6, Data at b6 = xx

Address=  b7, Data at b7 = xx

Address=  b8, Data at b8 = xx

Address=  b9, Data at b9 = xx

Address=  ba, Data at ba = xx

Address=  bb, Data at bb = xx

Address=  bc, Data at bc = xx

Address=  bd, Data at bd = xx

Address=  be, Data at be = xx

Address=  bf, Data at bf = xx

Address=  c0, Data at c0 = xx

Address=  c1, Data at c1 = xx

Address=  c2, Data at c2 = xx

Address=  c3, Data at c3 = xx

Address=  c4, Data at c4 = xx

Address=  c5, Data at c5 = xx

Address=  c6, Data at c6 = xx

Address=  c7, Data at c7 = xx

Address=  c8, Data at c8 = xx

Address=  c9, Data at c9 = xx

Address=  ca, Data at ca = xx

Address=  cb, Data at cb = xx

Address=  cc, Data at cc = xx

Address=  cd, Data at cd = xx

Address=  ce, Data at ce = xx

Address=  cf, Data at cf = xx

Address=  d0, Data at d0 = xx

Address=  d1, Data at d1 = xx

Address=  d2, Data at d2 = xx

Address=  d3, Data at d3 = xx

Address=  d4, Data at d4 = xx

Address=  d5, Data at d5 = xx

Address=  d6, Data at d6 = xx

Address=  d7, Data at d7 = xx

Address=  d8, Data at d8 = xx

Address=  d9, Data at d9 = xx

Address=  da, Data at da = xx

Address=  db, Data at db = xx

Address=  dc, Data at dc = xx

Address=  dd, Data at dd = xx

Address=  de, Data at de = xx

Address=  df, Data at df = xx

Address=  e0, Data at e0 = xx

Address=  e1, Data at e1 = xx

Address=  e2, Data at e2 = xx

Address=  e3, Data at e3 = xx

Address=  e4, Data at e4 = xx

Address=  e5, Data at e5 = xx

Address=  e6, Data at e6 = xx

Address=  e7, Data at e7 = xx

Address=  e8, Data at e8 = xx

Address=  e9, Data at e9 = xx

Address=  ea, fData at ea = xx

Address=  eb, Data at eb = xx

Address=  ec, Data at ec = xx

Address=  ed, Data at ed = xx

Address=  ee, Data at ee = xx

Address=  ef, Data at ef = xx

Address=  f0, Data at f0 = xx

Address=  f1, Data at f1 = xx

Address=  f2, Data at f2 = xx

Address=  f3, Data at f3 = xx

Address=  f4, Data at f4 = xx

Address=  f5, Data at f5 = xx

Address=  f6, Data at f6 = xx

Address=  f7, Data at f7 = xx

Address=  f8, Data at f8 = xx

Address=  f9, Data at f9 = xx

Address=  fa, Data at fa = xx

Address=  fb, Data at fb = xx

Address=  fc, Data at fc = xx

Address=  fd, Data at fd = xx

Address=  fe, Data at fe = xx

Address=  ff, Data at ff = xx

WRITEEN DATA : RAM[80h]= 44, RAM[81h]= 55,  RAM[82h]= 66,  RAM[FO]= xx

- Case 1:

16 ns  FETCH STATE: Instruction is Fetched

24 ns  DECODE STATE: Instruction is Decoded

32 ns  Instruction : LLS, 5

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

40 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 5

AR[00]= 00, DR[01]= 00, GR[10]= 05, PR[11]= 01

56 ns  FETCH STATE: Instruction is Fetched

64 ns  DECODE STATE: Instruction is Decoded

72 ns  Instruction : LMS, 4

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

80 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 4

AR[00]= 00, DR[01]= 00, GR[10]= 45, PR[11]= 02

96 ns  FETCH STATE: Instruction is Fetched

104 ns DECODE STATE: Instruction is Decoded

112 ns Instruction : CPR, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

120 ns EXECUTE_1 STATE: Executed CPR and Copied the Contents of 10 to 01 reg.

AR[00]= 00, DR[01]= 45, GR[10]= 45, PR[11]= 03

136 ns  FETCH STATE: Instruction is Fetched

144 ns  DECODE STATE: Instruction is Decoded

152 ns  Instruction : LLS, 0

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

160 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 0

AR[00]= 00, DR[01]= 45, GR[10]= 40, PR[11]= 04

176 ns  FETCH STATE: Instruction is Fetched

184 ns  DECODE STATE: Instruction is Decoded

192 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

200 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 00, DR[01]= 45, GR[10]= 80, PR[11]= 05

216 ns  FETCH STATE: Instruction is Fetched

224 ns  DECODE STATE: Instruction is Decoded

232 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

240 ns  EXECUTE_1 STATE: Executed CPR and Copied the Contents of 10 to 00 reg.

AR[00]= 80, DR[01]= 45, GR[10]= 80, PR[11]= 06

256 ns  FETCH STATE: Instruction is Fetched

264 ns  DECODE STATE: Instruction is Decoded

272 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

280 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 80 to Memory Address...

288 ns  EXECUTE_2 STATE: Read the Data 44 from Memory Location 80 and Stored

into 10 Register

AR[00]= 80, DR[01]= 45, GR[10]= 44, PR[11]= 07

304 ns  FETCH STATE: Instruction is Fetched

312 ns  DECODE STATE: Instruction is Decoded

320 ns  Instruction : ADD, 10, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

328 ns  EXECUTE_1 STATE: Executing ADD...Providing ALU Opcodes 44 and 45 and

Adding them...

336 ns  EXECUTE_2 STATE: Added the 10 Register Data 44 with 01 Register Data

45 and the Result is Stored into 10 Register

AR[00]= 80, DR[01]= 45, GR[10]= 89, PR[11]= 08

352 ns  FETCH STATE: Instruction is Fetched

360 ns  DECODE STATE: Instruction is Decoded

368 ns  Instruction : WRM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

376 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = 80  to Memory Address...

384 ns  EXECUTE_2 STATE: 10 Register Data 89 is Written at Memory Address 80

WRITEEN DATA : RAM[80h]= 89, RAM[81h]= 55, RAM[82h]= 66,  RAM[FO]= xx

- Case 2:

400 ns  FETCH STATE: Instruction is Fetched

408 ns  DECODE STATE: Instruction is Decoded

416 ns  Instruction : LLS, 1

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

424 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 1

AR[00]= 80, DR[01]= 45, GR[10]= 81, PR[11]= 0a

440 ns  FETCH STATE: Instruction is Fetched

448 ns  DECODE STATE: Instruction is Decoded

456 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

464 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 80, DR[01]= 45, GR[10]= 81, PR[11]= 0b

480 ns  FETCH STATE: Instruction is Fetched

488 ns  DECODE STATE: Instruction is Decoded

496 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

504 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 81, DR[01]= 45, GR[10]= 81, PR[11]= 0c

520 ns  FETCH STATE: Instruction is Fetched

528 ns  DECODE STATE: Instruction is Decoded

536 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

544 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 81 to Memory Address...

552 ns  EXECUTE_2 STATE: Read the Data 55 from Memory Location 81 and Stored

into 10 Register

AR[00]= 81, DR[01]= 45, GR[10]= 55, PR[11]= 0d

568 ns  FETCH STATE: Instruction is Fetched

576 ns  DECODE STATE: Instruction is Decoded

584 ns  Instruction : CPR, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

592 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 01

register

AR[00]= 81, DR[01]= 55, GR[10]= 55, PR[11]= 0e

608 ns  FETCH STATE: Instruction is Fetched

616 ns  DECODE STATE: Instruction is Decoded

624 ns  Instruction : LLS, 1

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

632 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 1

AR[00]= 81, DR[01]= 55, GR[10]= 51, PR[11]= 0f

648 ns  FETCH STATE: Instruction is Fetched

656 ns  DECODE STATE: Instruction is Decoded

664 ns  Instruction : LMS, 9

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

672 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 9

AR[00]= 81, DR[01]= 55, GR[10]= 91, PR[11]= 10

688 ns  FETCH STATE: Instruction is Fetched

696 ns  DECODE STATE: Instruction is Decoded

704 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

712 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 91, DR[01]= 55, GR[10]= 91, PR[11]= 11

728 ns  FETCH STATE: Instruction is Fetched

736 ns  DECODE STATE: Instruction is Decoded

744 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

752 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 91 to Memory Address...

760 ns  EXECUTE_2 STATE: Read the Data 77 from Memory Location 91 and Stored

into 10 Register

AR[00]= 91, DR[01]= 55, GR[10]= 77, PR[11]= 12

776 ns  FETCH STATE: Instruction is Fetched

784 ns  DECODE STATE: Instruction is Decoded

792 ns  Instruction : ADD, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

800 ns  EXECUTE_1 STATE: Executing ADD...Providing ALU Opcodes 55 and 77 and

Adding them...

808 ns  EXECUTE_2 STATE: Added the 01 Register Data 55 with 10 Register Data

77 and the Result is Stored into 01 Register

AR[00]= 91, DR[01]= cc, GR[10]= 77, PR[11]= 13

824 ns  FETCH STATE: Instruction is Fetched

832 ns  DECODE STATE: Instruction is Decoded

840 ns  Instruction : LLS, 1

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

848 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 1

AR[00]= 91, DR[01]= cc, GR[10]= 71, PR[11]= 14

864 ns  FETCH STATE: Instruction is Fetched

872 ns  DECODE STATE: Instruction is Decoded

880 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

888 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 91, DR[01]= cc, GR[10]= 81, PR[11]= 15

904 ns  FETCH STATE: Instruction is Fetched

912 ns  DECODE STATE: Instruction is Decoded

920 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

928 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 81, DR[01]= cc, GR[10]= 81, PR[11]= 16

944 ns  FETCH STATE: Instruction is Fetched

952 ns  DECODE STATE: Instruction is Decoded

960 ns  Instruction : WRM, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

968 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = 81  to Memory Address...

976 ns  EXECUTE_2 STATE: 01 Register Data cc is Written at Memory Address 81

WRITTEN DATA : RAM[80h]= 89, RAM[81h]= cc, RAM[82h]= 66,  RAM[FO]= xx

- Case 3:

992 ns  FETCH STATE: Instruction is Fetched

1000 ns DECODE STATE: Instruction is Decoded

1008 ns Instruction : LLS, 2

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1016 ns EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 2

AR[00]= 81, DR[01]= cc, GR[10]= 82, PR[11]= 18

1032 ns  FETCH STATE: Instruction is Fetched

1040 ns  DECODE STATE: Instruction is Decoded

1048 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1056 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 81, DR[01]= cc, GR[10]= 82, PR[11]= 19

1072 ns  FETCH STATE: Instruction is Fetched

1080 ns  DECODE STATE: Instruction is Decoded

1088 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1096 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00 register

AR[00]= 82, DR[01]= cc, GR[10]= 82, PR[11]= 1a

1112 ns  FETCH STATE: Instruction is Fetched

1120 ns  DECODE STATE: Instruction is Decoded

1128 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1136 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 82 to Memory Address...

1144 ns  EXECUTE_2 STATE: Read the Data 66 from Memory Location 82 and Stored into 10 Register

AR[00]= 82, DR[01]= cc, GR[10]= 66, PR[11]= 1b

1160 ns  FETCH STATE: Instruction is Fetched

1168 ns  DECODE STATE: Instruction is Decoded

1176 ns  Instruction : CPR, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1184 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 01

register

AR[00]= 82, DR[01]= 66, GR[10]= 66, PR[11]= 1c

1200 ns  FETCH STATE: Instruction is Fetched

1208 ns  DECODE STATE: Instruction is Decoded

1216 ns  Instruction : LLS, 2

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1224 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 2

AR[00]= 82, DR[01]= 66, GR[10]= 62, PR[11]= 1d

1240 ns  FETCH STATE: Instruction is Fetched

1248 ns  DECODE STATE: Instruction is Decoded

1256 ns  Instruction : LMS, 9

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1264 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 9

AR[00]= 82, DR[01]= 66, GR[10]= 92, PR[11]= 1e

1280 ns  FETCH STATE: Instruction is Fetched

1288 ns  DECODE STATE: Instruction is Decoded

1296 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1304 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 92, DR[01]= 66, GR[10]= 92, PR[11]= 1f

1320 ns  FETCH STATE: Instruction is Fetched

1328 ns  DECODE STATE: Instruction is Decoded

1336 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1344 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 92 to Memory Address...

1352 ns  EXECUTE_2 STATE: Read the Data 88 from Memory Location 92 and Stored into 10 Register

AR[00]= 92, DR[01]= 66, GR[10]= 88, PR[11]= 20

1368 ns  FETCH STATE: Instruction is Fetched

1376 ns  DECODE STATE: Instruction is Decoded

1384 ns  Instruction : SUB, 10, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1392 ns  EXECUTE_1 STATE: Executing SUB...Providing ALU Opcodes 88 and 66 and Adding them...

1400 ns  EXECUTE_2 STATE: Subtracted the 01 Register Data 66 from 10 Register

Data 88 and the Result is Stored into 10 Register

AR[00]= 92, DR[01]= 66, GR[10]= 22, PR[11]= 21

1416 ns  FETCH STATE: Instruction is Fetched

1424 ns  DECODE STATE: Instruction is Decoded

1432 ns  Instruction : LLS, 2

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1440 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 2

AR[00]= 92, DR[01]= 66, GR[10]= 22, PR[11]= 22

1456 ns  FETCH STATE: Instruction is Fetched

1464 ns  DECODE STATE: Instruction is Decoded

1472 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1480 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 92, DR[01]= 66, GR[10]= 82, PR[11]= 23

1496 ns  FETCH STATE: Instruction is Fetched

1504 ns  DECODE STATE: Instruction is Decoded

1512 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1520 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 82, DR[01]= 66, GR[10]= 82, PR[11]= 24

1536 ns  FETCH STATE: Instruction is Fetched

1544 ns  DECODE STATE: Instruction is Decoded

1552 ns  Instruction : WRM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1560 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = 82  to Memory Address...

1568 ns  EXECUTE_2 STATE: 10 Register Data 82 is Written at Memory Address 82

WRITTEN DATA : RAM[80h]= 89, RAM[81h]= cc, RAM[82h]= 82, RAM[FO]= xx

- Case 4:

1584 ns  FETCH STATE: Instruction is Fetched

1592 ns  DECODE STATE: Instruction is Decoded

1600 ns  Instruction : CFR;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1608 ns  EXECUTE_1 STATE: Executed CFR and Stored data  at LSBs of GR = 1

AR[00]= 82, DR[01]= 66, GR[10]= 81, PR[11]= 26

1624 ns  FETCH STATE: Instruction is Fetched

1632 ns  DECODE STATE: Instruction is Decoded

1640 ns  Instruction : CPR, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1648 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 01 register

AR[00]= 82, DR[01]= 81, GR[10]= 81, PR[11]= 27

1664 ns  FETCH STATE: Instruction is Fetched

1672 ns  DECODE STATE: Instruction is Decoded

1680 ns  Instruction : LLS, 0

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1688 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 0

AR[00]= 82, DR[01]= 81, GR[10]= 80, PR[11]= 28

1704 ns  FETCH STATE: Instruction is Fetched

1712 ns  DECODE STATE: Instruction is Decoded

1720 ns  Instruction : LMS, f

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1728 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = f

AR[00]= 82, DR[01]= 81, GR[10]= f0, PR[11]= 29

1744 ns  FETCH STATE: Instruction is Fetched

1752 ns  DECODE STATE: Instruction is Decoded

1760 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1768 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00 register

AR[00]= f0, DR[01]= 81, GR[10]= f0, PR[11]= 2a

1784 ns  FETCH STATE: Instruction is Fetched

1792 ns  DECODE STATE: Instruction is Decoded

1800 ns  Instruction : WRM, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1808 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = f0  to Memory Address...

1816 ns  EXECUTE_2 STATE: 01 Register Data 81 is Written at Memory Address f0

WRITTEN DATA : RAM[80h]= 89, RAM[81h]= cc,  RAM[82h]= 82,  RAM[FO]= 81

1832 ns  FETCH STATE: Instruction is Fetched

1840 ns  DECODE STATE: Instruction is Decoded

1848 ns  Instruction : NOP

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1856 ns  EXECUTE_1 STATE: Executed NOP and  No Operation is Perfomed

- Case 5:

1872 ns  FETCH STATE: Instruction is Fetched

1880 ns  DECODE STATE: Instruction is Decoded

1888 ns  Instruction : LLS, 0

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1896 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 0

AR[00]= f0, DR[01]= 81, GR[10]= f0, PR[11]= 2d

1912 ns  FETCH STATE: Instruction is Fetched

1920 ns  DECODE STATE: Instruction is Decoded

1928 ns  Instruction : LMS, 0

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1936 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 0

AR[00]= f0, DR[01]= 81, GR[10]= 00, PR[11]= 2e

1952 ns  FETCH STATE: Instruction is Fetched

1960 ns  DECODE STATE: Instruction is Decoded

1968 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1976 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 00, DR[01]= 81, GR[10]= 00, PR[11]= 2f

1992 ns  FETCH STATE: Instruction is Fetched

2000 ns  DECODE STATE: Instruction is Decoded

2008 ns  Instruction : JMP

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2016 ns  EXECUTE_1 STATE: Executed JMP and Jumped to Instruction stored at 00

Memory Location

2032 ns  FETCH STATE: Instruction is Fetched

2040 ns  DECODE STATE: Instruction is Decoded

2048 ns  Instruction : LLS, 5

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2056 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 5

AR[00]= 00, DR[01]= 81, GR[10]= 05, PR[11]= 01

2072 ns  FETCH STATE: Instruction is Fetched

2080 ns  DECODE STATE: Instruction is Decoded

2088 ns  Instruction : LMS, 4

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2096 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 4

AR[00]= 00, DR[01]= 81, GR[10]= 45, PR[11]= 02

2112 ns  FETCH STATE: Instruction is Fetched

2120 ns  DECODE STATE: Instruction is Decoded

2128 ns  Instruction : CPR, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2136 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 01 register

AR[00]= 00, DR[01]= 45, GR[10]= 45, PR[11]= 03

2152 ns  FETCH STATE: Instruction is Fetched

2160 ns  DECODE STATE: Instruction is Decoded

2168 ns  Instruction : LLS, 0

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2176 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 0

AR[00]= 00, DR[01]= 45, GR[10]= 40, PR[11]= 04

2192 ns  FETCH STATE: Instruction is Fetched

2200 ns  DECODE STATE: Instruction is Decoded

2208 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2216 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 00, DR[01]= 45, GR[10]= 80, PR[11]= 05

2232 ns  FETCH STATE: Instruction is Fetched

2240 ns  DECODE STATE: Instruction is Decoded

2248 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2256 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 80, DR[01]= 45, GR[10]= 80, PR[11]= 06

2272 ns  FETCH STATE: Instruction is Fetched

2280 ns  DECODE STATE: Instruction is Decoded

2288 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2296 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 80 to Memory Address...

2304 ns  EXECUTE_2 STATE: Read the Data 89 from Memory Location 80 and Stored into 10 Register

AR[00]= 80, DR[01]= 45, GR[10]= 89, PR[11]= 07

2320 ns  FETCH STATE: Instruction is Fetched

2328 ns  DECODE STATE: Instruction is Decoded

2336 ns  Instruction : ADD, 10, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2344 ns  EXECUTE_1 STATE: Executing ADD...Providing ALU Opcodes 89 and 45 and

Adding them...

2352 ns  EXECUTE_2 STATE: Added the 10 Register Data 89 with 01 Register Data 45 and the Result is Stored into 10 Register

AR[00]= 80, DR[01]= 45, GR[10]= ce, PR[11]= 08

2368 ns  FETCH STATE: Instruction is Fetched

2376 ns  DECODE STATE: Instruction is Decoded

2384 ns  Instruction : WRM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2392 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = 80  to Memory

Address...

2400 ns  EXECUTE_2 STATE: 10 Register Data ce is Written at Memory Address 80

WRITTEN DATA : RAM[80h]= ce, RAM[81h]= cc, RAM[82h]= 82, RAM[FO]= 81

2416 ns  FETCH STATE: Instruction is Fetched

2424 ns  DECODE STATE: Instruction is Decoded

2432 ns  Instruction : LLS, 1

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2440 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 1

AR[00]= 80, DR[01]= 45, GR[10]= c1, PR[11]= 0a

2456 ns  FETCH STATE: Instruction is Fetched

2464 ns  DECODE STATE: Instruction is Decoded

2472 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2480 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 80, DR[01]= 45, GR[10]= 81, PR[11]= 0b

2496 ns  FETCH STATE: Instruction is Fetched

2504 ns  DECODE STATE: Instruction is Decoded

2512 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2520 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 81, DR[01]= 45, GR[10]= 81, PR[11]= 0c

2536 ns  FETCH STATE: Instruction is Fetched

2544 ns  DECODE STATE: Instruction is Decoded

2552 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2560 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 81 to Memory Address...

2568 ns  EXECUTE_2 STATE: Read the Data cc from Memory Location 81 and Stored

into 10 Register

AR[00]= 81, DR[01]= 45, GR[10]= cc, PR[11]= 0d

2584 ns  FETCH STATE: Instruction is Fetched

2592 ns  DECODE STATE: Instruction is Decoded

2600 ns  Instruction : CPR, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2608 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 01

register

AR[00]= 81, DR[01]= cc, GR[10]= cc, PR[11]= 0e

2624 ns  FETCH STATE: Instruction is Fetched

2632 ns  DECODE STATE: Instruction is Decoded

2640 ns  Instruction : LLS, 1

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2648 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 1

AR[00]= 81, DR[01]= cc, GR[10]= c1, PR[11]= 0f

2664 ns  FETCH STATE: Instruction is Fetched

2672 ns  DECODE STATE: Instruction is Decoded

2680 ns  Instruction : LMS, 9

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2688 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 9

AR[00]= 81, DR[01]= cc, GR[10]= 91, PR[11]= 10

2704 ns  FETCH STATE: Instruction is Fetched

2712 ns  DECODE STATE: Instruction is Decoded

2720 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2728 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 91, DR[01]= cc, GR[10]= 91, PR[11]= 11

2744 ns  FETCH STATE: Instruction is Fetched

2752 ns  DECODE STATE: Instruction is Decoded

2760 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2768 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 91 to Memory Address...

2776 ns  EXECUTE_2 STATE: Read the Data 77 from Memory Location 91 and Stored

into 10 Register

AR[00]= 91, DR[01]= cc, GR[10]= 77, PR[11]= 12

2792 ns  FETCH STATE: Instruction is Fetched

2800 ns  DECODE STATE: Instruction is Decoded

2808 ns  Instruction : ADD, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2816 ns  EXECUTE_1 STATE: Executing ADD...Providing ALU Opcodes cc and 77 and

Adding them...

2824 ns  EXECUTE_2 STATE: Added the 01 Register Data cc with 10 Register Data

77 and the Result is Stored into 01 Register

AR[00]= 91, DR[01]= 43, GR[10]= 77, PR[11]= 13

2840 ns  FETCH STATE: Instruction is Fetched

2848 ns  DECODE STATE: Instruction is Decoded

2856 ns  Instruction : LLS, 1

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2864 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 1

AR[00]= 91, DR[01]= 43, GR[10]= 71, PR[11]= 14

2880 ns  FETCH STATE: Instruction is Fetched

2888 ns  DECODE STATE: Instruction is Decoded

2896 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2904 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 91, DR[01]= 43, GR[10]= 81, PR[11]= 15

2920 ns  FETCH STATE: Instruction is Fetched

2928 ns  DECODE STATE: Instruction is Decoded

2936 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2944 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 81, DR[01]= 43, GR[10]= 81, PR[11]= 16

2960 ns  FETCH STATE: Instruction is Fetched

2968 ns  DECODE STATE: Instruction is Decoded

2976 ns  Instruction : WRM, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2984 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = 81  to Memory Address...

2992 ns  EXECUTE_2 STATE: 01 Register Data 43 is Written at Memory Address 81

WRITTEN DATA : RAM[80h]= ce,RAM[81h]= 43, RAM[82h]= 82,  RAM[FO]= 81

3008 ns  FETCH STATE: Instruction is Fetched

3016 ns  DECODE STATE: Instruction is Decoded

3024 ns  Instruction : LLS, 2

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3032 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 2

AR[00]= 81, DR[01]= 43, GR[10]= 82, PR[11]= 18

3048 ns  FETCH STATE: Instruction is Fetched

3056 ns  DECODE STATE: Instruction is Decoded

3064 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3072 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 81, DR[01]= 43, GR[10]= 82, PR[11]= 19

3088 ns  FETCH STATE: Instruction is Fetched

3096 ns  DECODE STATE: Instruction is Decoded

3104 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3112 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 82, DR[01]= 43, GR[10]= 82, PR[11]= 1a

3128 ns  FETCH STATE: Instruction is Fetched

3136 ns  DECODE STATE: Instruction is Decoded

3144 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3152 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 82 to Memory Address...

3160 ns  EXECUTE_2 STATE: Read the Data 82 from Memory Location 82 and Stored into 10 Register

AR[00]= 82, DR[01]= 43, GR[10]= 82, PR[11]= 1b

3176 ns  FETCH STATE: Instruction is Fetched

3184 ns  DECODE STATE: Instruction is Decoded

3192 ns  Instruction : CPR, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3200 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 01

register

AR[00]= 82, DR[01]= 82, GR[10]= 82, PR[11]= 1c

3216 ns  FETCH STATE: Instruction is Fetched

3224 ns  DECODE STATE: Instruction is Decoded

3232 ns  Instruction : LLS, 2

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3240 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 2

AR[00]= 82, DR[01]= 82, GR[10]= 82, PR[11]= 1d

3256 ns  FETCH STATE: Instruction is Fetched

3264 ns  DECODE STATE: Instruction is Decoded

3272 ns  Instruction : LMS, 9

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3280 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 9

AR[00]= 82, DR[01]= 82, GR[10]= 92, PR[11]= 1e

3296 ns  FETCH STATE: Instruction is Fetched

3304 ns  DECODE STATE: Instruction is Decoded

3312 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3320 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 92, DR[01]= 82, GR[10]= 92, PR[11]= 1f

3336 ns  FETCH STATE: Instruction is Fetched

3344 ns  DECODE STATE: Instruction is Decoded

3352 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3360 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 92 to Memory Address...

3368 ns  EXECUTE_2 STATE: Read the Data 88 from Memory Location 92 and Stored

into 10 Register

AR[00]= 92, DR[01]= 82, GR[10]= 88, PR[11]= 20

3384 ns  FETCH STATE: Instruction is Fetched

3392 ns  DECODE STATE: Instruction is Decoded

3400 ns  Instruction : SUB, 10, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3408 ns  EXECUTE_1 STATE: Executing SUB...Providing ALU Opcodes 88 and 82 and

Adding them...

3416 ns  EXECUTE_2 STATE: Subtracted the 01 Register Data 82 from 10 Register

Data 88 and the Result is Stored into 10 Register

AR[00]= 92, DR[01]= 82, GR[10]= 06, PR[11]= 21

3432 ns  FETCH STATE: Instruction is Fetched

3440 ns  DECODE STATE: Instruction is Decoded

3448 ns  Instruction : LLS, 2

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3456 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 2

AR[00]= 92, DR[01]= 82, GR[10]= 02, PR[11]= 22

3472 ns  FETCH STATE: Instruction is Fetched

3480 ns  DECODE STATE: Instruction is Decoded

3488 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next

Instruction.

3496 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 92, DR[01]= 82, GR[10]= 82, PR[11]= 23

3512 ns  FETCH STATE: Instruction is Fetched

3520 ns  DECODE STATE: Instruction is Decoded

3528 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3536 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 82, DR[01]= 82, GR[10]= 82, PR[11]= 24

3552 ns  FETCH STATE: Instruction is Fetched

3560 ns  DECODE STATE: Instruction is Decoded

3568 ns  Instruction : WRM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction

3576 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = 82  to Memory

Address...

3584 ns  EXECUTE_2 STATE: 10 Register Data 82 is Written at Memory Address 82

3600 ns  FETCH STATE: Instruction is Fetched

3608 ns  DECODE STATE: Instruction is Decoded

3616 ns  Instruction : CFR;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3624 ns  EXECUTE_1 STATE: Executed CFR and Stored data  at LSBs of GR = 0

AR[00]= 82, DR[01]= 82, GR[10]= 80, PR[11]= 26

3640 ns  FETCH STATE: Instruction is Fetched

3648 ns  DECODE STATE: Instruction is Decoded

3656 ns  Instruction : CPR, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3664 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 01

register

AR[00]= 82, DR[01]= 80, GR[10]= 80, PR[11]= 27

3680 ns  FETCH STATE: Instruction is Fetched

3688 ns  DECODE STATE: Instruction is Decoded

3696 ns  Instruction : LLS, 0

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3704 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 0

AR[00]= 82, DR[01]= 80, GR[10]= 80, PR[11]= 28

3720 ns  FETCH STATE: Instruction is Fetched

3728 ns  DECODE STATE: Instruction is Decoded

3736 ns  Instruction : LMS, f

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3744 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = f

AR[00]= 82, DR[01]= 80, GR[10]= f0, PR[11]= 29

3760 ns  FETCH STATE: Instruction is Fetched

3768 ns  DECODE STATE: Instruction is Decoded

3776 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3784 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00 register

AR[00]= f0, DR[01]= 80, GR[10]= f0, PR[11]= 2a

3800 ns  FETCH STATE: Instruction is Fetched

3808 ns  DECODE STATE: Instruction is Decoded

3816 ns  Instruction : WRM, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3824 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = f0  to Memory Address...

3832 ns  EXECUTE_2 STATE: 01 Register Data 80 is Written at Memory Address f0

WRITTEN DATA : RAM[80h]= ce, RAM[81h]= 43,RAM[82h]= 82,  RAM[FO]= 80

3848 ns  FETCH STATE: Instruction is Fetched

3856 ns  DECODE STATE: Instruction is Decoded

3864 ns  Instruction : NOP

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3872 ns  EXECUTE_1 STATE: Executed NOP and  No Operation is Perfomed

3888 ns  FETCH STATE: Instruction is Fetched

$finish called from file "scalar_test.v", line 133.

$finish at simulation time               388900

V C S   S i m u l a t i o n   R e p o r t

Time: 3889000 ps

CPU Time:      0.270 seconds;       Data structure size:   0.0Mb

Fri Dec  5 03:56:12 2014

C.2	Reports (contents) from Netlist (Post-synthesis) Simulations (VCS or NCVERILOG)

Chronologic VCS simulator copyright 1991-2014

Contains Synopsys proprietary information.

Compiler version I-2014.03-2; Runtime version I-2014.03-2;  Dec  5 15:24 2014

Address=  00, Data at 00 = 75

Address=  01, Data at 01 = 84

Address=  02, Data at 02 = 46

Address=  03, Data at 03 = 70

Address=  04, Data at 04 = 88

Address=  05, Data at 05 = 42

Address=  06, Data at 06 = 28

Address=  07, Data at 07 = 59

Address=  08, Data at 08 = 38

Address=  09, Data at 09 = 71

Address=  0a, Data at 0a = 88

Address=  0b, Data at 0b = 42

Address=  0c, Data at 0c = 28

Address=  0d, Data at 0d = 46

Address=  0e, Data at 0e = 71

Address=  0f, Data at 0f = 89

Address=  10, Data at 10 = 42

Address=  11, Data at 11 = 28

Address=  12, Data at 12 = 56

Address=  13, Data at 13 = 71

Address=  14, Data at 14 = 88

Address=  15, Data at 15 = 42

Address=  16, Data at 16 = 34

Address=  17, Data at 17 = 72

Address=  18, Data at 18 = 88

Address=  19, Data at 19 = 42

Address=  1a, Data at 1a = 28

Address=  1b, Data at 1b = 46

Address=  1c, Data at 1c = 72

Address=  1d, Data at 1d = 89

Address=  1e, Data at 1e = 42

Address=  1f, Data at 1f = 28

Address=  20, Data at 20 = 69

Address=  21, Data at 21 = 72

Address=  22, Data at 22 = 88

Address=  23, Data at 23 = 42

Address=  24, Data at 24 = 38

Address=  25, Data at 25 = 90

Address=  26, Data at 26 = 46

Address=  27, Data at 27 = 70

Address=  28, Data at 28 = 8f

Address=  29, Data at 29 = 42

Address=  2a, Data at 2a = 34

Address=  2b, Data at 2b = 00

Address=  2c, Data at 2c = 70

Address=  2d, Data at 2d = 80

Address=  2e, Data at 2e = 42

Address=  2f, Data at 2f = 10

Address=  30, Data at 30 = xx

Address=  31, Data at 31 = xx

Address=  32, Data at 32 = xx

Address=  33, Data at 33 = xx

Address=  34, Data at 34 = xx

Address=  35, Data at 35 = xx

Address=  36, Data at 36 = xx

Address=  37, Data at 37 = xx

Address=  38, Data at 38 = xx

Address=  39, Data at 39 = xx

Address=  3a, Data at 3a = xx

Address=  3b, Data at 3b = xx

Address=  3c, Data at 3c = xx

Address=  3d, Data at 3d = xx

Address=  3e, Data at 3e = xx

Address=  3f, Data at 3f = xx

Address=  40, Data at 40 = xx

Address=  41, Data at 41 = xx

Address=  42, Data at 42 = xx

Address=  43, Data at 43 = xx

Address=  44, Data at 44 = xx

Address=  45, Data at 45 = xx

Address=  46, Data at 46 = xx

Address=  47, Data at 47 = xx

Address=  48, Data at 48 = xx

Address=  49, Data at 49 = xx

Address=  4a, Data at 4a = xx

Address=  4b, Data at 4b = xx

Address=  4c, Data at 4c = xx

Address=  4d, Data at 4d = xx

Address=  4e, Data at 4e = xx

Address=  4f, Data at 4f = xx

Address=  50, Data at 50 = xx

Address=  51, Data at 51 = xx

Address=  52, Data at 52 = xx

Address=  53, Data at 53 = xx

Address=  54, Data at 54 = xx

Address=  55, Data at 55 = xx

Address=  56, Data at 56 = xx

Address=  57, Data at 57 = xx

Address=  58, Data at 58 = xx

Address=  59, Data at 59 = xx

Address=  5a, Data at 5a = xx

Address=  5b, Data at 5b = xx

Address=  5c, Data at 5c = xx

Address=  5d, Data at 5d = xx

Address=  5e, Data at 5e = xx

Address=  5f, Data at 5f = xx

Address=  60, Data at 60 = xx

Address=  61, Data at 61 = xx

Address=  62, Data at 62 = xx

Address=  63, Data at 63 = xx

Address=  64, Data at 64 = xx

Address=  65, Data at 65 = xx

Address=  66, Data at 66 = xx

Address=  67, Data at 67 = xx

Address=  68, Data at 68 = xx

Address=  69, Data at 69 = xx

Address=  6a, Data at 6a = xx

Address=  6b, Data at 6b = xx

Address=  6c, Data at 6c = xx

Address=  6d, Data at 6d = xx

Address=  6e, Data at 6e = xx

Address=  6f, Data at 6f = xx

Address=  70, Data at 70 = xx

Address=  71, Data at 71 = xx

Address=  72, Data at 72 = xx

Address=  73, Data at 73 = xx

Address=  74, Data at 74 = xx

Address=  75, Data at 75 = xx

Address=  76, Data at 76 = xx

Address=  77, Data at 77 = xx

Address=  78, Data at 78 = xx

Address=  79, Data at 79 = xx

Address=  7a, Data at 7a = xx

Address=  7b, Data at 7b = xx

Address=  7c, Data at 7c = xx

Address=  7d, Data at 7d = xx

Address=  7e, Data at 7e = xx

Address=  7f, Data at 7f = xx

Address=  80, Data at 80 = 44

Address=  81, Data at 81 = 55

Address=  82, Data at 82 = 66

Address=  83, Data at 83 = xx

Address=  84, Data at 84 = xx

Address=  85, Data at 85 = xx

Address=  86, Data at 86 = xx

Address=  87, Data at 87 = xx

Address=  88, Data at 88 = xx

Address=  89, Data at 89 = xx

Address=  8a, Data at 8a = xx

Address=  8b, Data at 8b = xx

Address=  8c, Data at 8c = xx

Address=  8d, Data at 8d = xx

Address=  8e, Data at 8e = xx

Address=  8f, Data at 8f = xx

Address=  90, Data at 90 = xx

Address=  91, Data at 91 = 77

Address=  92, Data at 92 = 88

Address=  93, Data at 93 = xx

Address=  94, Data at 94 = xx

Address=  95, Data at 95 = xx

Address=  96, Data at 96 = xx

Address=  97, Data at 97 = xx

Address=  98, Data at 98 = xx

Address=  99, Data at 99 = xx

Address=  9a, Data at 9a = xx

Address=  9b, Data at 9b = xx

Address=  9c, Data at 9c = xx

Address=  9d, Data at 9d = xx

Address=  9e, Data at 9e = xx

Address=  9f, Data at 9f = xx

Address=  a0, Data at a0 = xx

Address=  a1, Data at a1 = xx

Address=  a2, Data at a2 = xx

Address=  a3, Data at a3 = xx

Address=  a4, Data at a4 = xx

Address=  a5, Data at a5 = xx

Address=  a6, Data at a6 = xx

Address=  a7, Data at a7 = xx

Address=  a8, Data at a8 = xx

Address=  a9, Data at a9 = xx

Address=  aa, Data at aa = xx

Address=  ab, Data at ab = xx

Address=  ac, Data at ac = xx

Address=  ad, Data at ad = xx

Address=  ae, Data at ae = xx

Address=  af, Data at af = xx

Address=  b0, Data at b0 = xx

Address=  b1, Data at b1 = xx

Address=  b2, Data at b2 = xx

Address=  b3, Data at b3 = xx

Address=  b4, Data at b4 = xx

Address=  b5, Data at b5 = xx

Address=  b6, Data at b6 = xx

Address=  b7, Data at b7 = xx

Address=  b8, Data at b8 = xx

Address=  b9, Data at b9 = xx

Address=  ba, Data at ba = xx

Address=  bb, Data at bb = xx

Address=  bc, Data at bc = xx

Address=  bd, Data at bd = xx

Address=  be, Data at be = xx

Address=  bf, Data at bf = xx

Address=  c0, Data at c0 = xx

Address=  c1, Data at c1 = xx

Address=  c2, Data at c2 = xx

Address=  c3, Data at c3 = xx

Address=  c4, Data at c4 = xx

Address=  c5, Data at c5 = xx

Address=  c6, Data at c6 = xx

Address=  c7, Data at c7 = xx

Address=  c8, Data at c8 = xx

Address=  c9, Data at c9 = xx

Address=  ca, Data at ca = xx

Address=  cb, Data at cb = xx

Address=  cc, Data at cc = xx

Address=  cd, Data at cd = xx

Address=  ce, Data at ce = xx

Address=  cf, Data at cf = xx

Address=  d0, Data at d0 = xx

Address=  d1, Data at d1 = xx

Address=  d2, Data at d2 = xx

Address=  d3, Data at d3 = xx

Address=  d4, Data at d4 = xx

Address=  d5, Data at d5 = xx

Address=  d6, Data at d6 = xx

Address=  d7, Data at d7 = xx

Address=  d8, Data at d8 = xx

Address=  d9, Data at d9 = xx

Address=  da, Data at da = xx

Address=  db, Data at db = xx

Address=  dc, Data at dc = xx

Address=  dd, Data at dd = xx

Address=  de, Data at de = xx

Address=  df, Data at df = xx

Address=  e0, Data at e0 = xx

Address=  e1, Data at e1 = xx

Address=  e2, Data at e2 = xx

Address=  e3, Data at e3 = xx

Address=  e4, Data at e4 = xx

Address=  e5, Data at e5 = xx

Address=  e6, Data at e6 = xx

Address=  e7, Data at e7 = xx

Address=  e8, Data at e8 = xx

Address=  e9, Data at e9 = xx

Address=  ea, Data at ea = xx

Address=  eb, Data at eb = xx

Address=  ec, Data at ec = xx

Address=  ed, Data at ed = xx

Address=  ee, Data at ee = xx

Address=  ef, Data at ef = xx

Address=  f0, Data at f0 = xx

Address=  f1, Data at f1 = xx

Address=  f2, Data at f2 = xx

Address=  f3, Data at f3 = xx

Address=  f4, Data at f4 = xx

Address=  f5, Data at f5 = xx

Address=  f6, Data at f6 = xx

Address=  f7, Data at f7 = xx

Address=  f8, Data at f8 = xx

Address=  f9, Data at f9 = xx

Address=  fa, Data at fa = xx

Address=  fb, Data at fb = xx

Address=  fc, Data at fc = xx

Address=  fd, Data at fd = xx

Address=  fe, Data at fe = xx

Address=  ff, Data at ff = xx

WRITEEN DATA : RAM[80h]= 44, RAM[81h]= 55,  RAM[82h]= 66,  RAM[FO]= xx

- Case 1:

16 ns  FETCH STATE: Instruction is Fetched

24 ns  DECODE STATE: Instruction is Decoded

32 ns  Instruction : LLS, 5

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

40 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 5

AR[00]= 00, DR[01]= 00, GR[10]= 05, PR[11]= 01

56 ns  FETCH STATE: Instruction is Fetched

64 ns  DECODE STATE: Instruction is Decoded

72 ns  Instruction : LMS, 4

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

80 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 4

AR[00]= 00, DR[01]= 00, GR[10]= 45, PR[11]= 02

96 ns  FETCH STATE: Instruction is Fetched

104 ns DECODE STATE: Instruction is Decoded

112 ns Instruction : CPR, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

120 ns EXECUTE_1 STATE: Executed CPR and Copied the Contents of 10 to 01 reg.

AR[00]= 00, DR[01]= 45, GR[10]= 45, PR[11]= 03

136 ns  FETCH STATE: Instruction is Fetched

144 ns  DECODE STATE: Instruction is Decoded

152 ns  Instruction : LLS, 0

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

160 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 0

AR[00]= 00, DR[01]= 45, GR[10]= 40, PR[11]= 04

176 ns  FETCH STATE: Instruction is Fetched

184 ns  DECODE STATE: Instruction is Decoded

192 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

200 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 00, DR[01]= 45, GR[10]= 80, PR[11]= 05

216 ns  FETCH STATE: Instruction is Fetched

224 ns  DECODE STATE: Instruction is Decoded

232 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

240 ns  EXECUTE_1 STATE: Executed CPR and Copied the Contents of 10 to 00 reg.

AR[00]= 80, DR[01]= 45, GR[10]= 80, PR[11]= 06

256 ns  FETCH STATE: Instruction is Fetched

264 ns  DECODE STATE: Instruction is Decoded

272 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

280 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 80 to Memory Address...

288 ns  EXECUTE_2 STATE: Read the Data 44 from Memory Location 80 and Stored

into 10 Register

AR[00]= 80, DR[01]= 45, GR[10]= 44, PR[11]= 07

304 ns  FETCH STATE: Instruction is Fetched

312 ns  DECODE STATE: Instruction is Decoded

320 ns  Instruction : ADD, 10, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

328 ns  EXECUTE_1 STATE: Executing ADD...Providing ALU Opcodes 44 and 45 and

Adding them...

336 ns  EXECUTE_2 STATE: Added the 10 Register Data 44 with 01 Register Data

45 and the Result is Stored into 10 Register

AR[00]= 80, DR[01]= 45, GR[10]= 89, PR[11]= 08

352 ns  FETCH STATE: Instruction is Fetched

360 ns  DECODE STATE: Instruction is Decoded

368 ns  Instruction : WRM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

376 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = 80  to Memory Address...

384 ns  EXECUTE_2 STATE: 10 Register Data 89 is Written at Memory Address 80

WRITEEN DATA : RAM[80h]= 89, RAM[81h]= 55, RAM[82h]= 66,  RAM[FO]= xx

- Case 2:

400 ns  FETCH STATE: Instruction is Fetched

408 ns  DECODE STATE: Instruction is Decoded

416 ns  Instruction : LLS, 1

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

424 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 1

AR[00]= 80, DR[01]= 45, GR[10]= 81, PR[11]= 0a

440 ns  FETCH STATE: Instruction is Fetched

448 ns  DECODE STATE: Instruction is Decoded

456 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

464 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 80, DR[01]= 45, GR[10]= 81, PR[11]= 0b

480 ns  FETCH STATE: Instruction is Fetched

488 ns  DECODE STATE: Instruction is Decoded

496 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

504 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 81, DR[01]= 45, GR[10]= 81, PR[11]= 0c

520 ns  FETCH STATE: Instruction is Fetched

528 ns  DECODE STATE: Instruction is Decoded

536 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

544 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 81 to Memory Address...

552 ns  EXECUTE_2 STATE: Read the Data 55 from Memory Location 81 and Stored

into 10 Register

AR[00]= 81, DR[01]= 45, GR[10]= 55, PR[11]= 0d

568 ns  FETCH STATE: Instruction is Fetched

576 ns  DECODE STATE: Instruction is Decoded

584 ns  Instruction : CPR, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

592 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 01

register

AR[00]= 81, DR[01]= 55, GR[10]= 55, PR[11]= 0e

608 ns  FETCH STATE: Instruction is Fetched

616 ns  DECODE STATE: Instruction is Decoded

624 ns  Instruction : LLS, 1

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

632 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 1

AR[00]= 81, DR[01]= 55, GR[10]= 51, PR[11]= 0f

648 ns  FETCH STATE: Instruction is Fetched

656 ns  DECODE STATE: Instruction is Decoded

664 ns  Instruction : LMS, 9

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

672 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 9

AR[00]= 81, DR[01]= 55, GR[10]= 91, PR[11]= 10

688 ns  FETCH STATE: Instruction is Fetched

696 ns  DECODE STATE: Instruction is Decoded

704 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

712 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 91, DR[01]= 55, GR[10]= 91, PR[11]= 11

728 ns  FETCH STATE: Instruction is Fetched

736 ns  DECODE STATE: Instruction is Decoded

744 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

752 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 91 to Memory Address...

760 ns  EXECUTE_2 STATE: Read the Data 77 from Memory Location 91 and Stored

into 10 Register

AR[00]= 91, DR[01]= 55, GR[10]= 77, PR[11]= 12

776 ns  FETCH STATE: Instruction is Fetched

784 ns  DECODE STATE: Instruction is Decoded

792 ns  Instruction : ADD, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

800 ns  EXECUTE_1 STATE: Executing ADD...Providing ALU Opcodes 55 and 77 and

Adding them...

808 ns  EXECUTE_2 STATE: Added the 01 Register Data 55 with 10 Register Data

77 and the Result is Stored into 01 Register

AR[00]= 91, DR[01]= cc, GR[10]= 77, PR[11]= 13

824 ns  FETCH STATE: Instruction is Fetched

832 ns  DECODE STATE: Instruction is Decoded

840 ns  Instruction : LLS, 1

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

848 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 1

AR[00]= 91, DR[01]= cc, GR[10]= 71, PR[11]= 14

864 ns  FETCH STATE: Instruction is Fetched

872 ns  DECODE STATE: Instruction is Decoded

880 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

888 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 91, DR[01]= cc, GR[10]= 81, PR[11]= 15

904 ns  FETCH STATE: Instruction is Fetched

912 ns  DECODE STATE: Instruction is Decoded

920 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

928 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 81, DR[01]= cc, GR[10]= 81, PR[11]= 16

944 ns  FETCH STATE: Instruction is Fetched

952 ns  DECODE STATE: Instruction is Decoded

960 ns  Instruction : WRM, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

968 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = 81  to Memory Address...

976 ns  EXECUTE_2 STATE: 01 Register Data cc is Written at Memory Address 81

WRITTEN DATA : RAM[80h]= 89, RAM[81h]= cc, RAM[82h]= 66,  RAM[FO]= xx

- Case 3:

992 ns  FETCH STATE: Instruction is Fetched

1000 ns DECODE STATE: Instruction is Decoded

1008 ns Instruction : LLS, 2

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1016 ns EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 2

AR[00]= 81, DR[01]= cc, GR[10]= 82, PR[11]= 18

1032 ns  FETCH STATE: Instruction is Fetched

1040 ns  DECODE STATE: Instruction is Decoded

1048 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1056 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 81, DR[01]= cc, GR[10]= 82, PR[11]= 19

1072 ns  FETCH STATE: Instruction is Fetched

1080 ns  DECODE STATE: Instruction is Decoded

1088 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1096 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00 register

AR[00]= 82, DR[01]= cc, GR[10]= 82, PR[11]= 1a

1112 ns  FETCH STATE: Instruction is Fetched

1120 ns  DECODE STATE: Instruction is Decoded

1128 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1136 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 82 to Memory Address...

1144 ns  EXECUTE_2 STATE: Read the Data 66 from Memory Location 82 and Stored into 10 Register

AR[00]= 82, DR[01]= cc, GR[10]= 66, PR[11]= 1b

1160 ns  FETCH STATE: Instruction is Fetched

1168 ns  DECODE STATE: Instruction is Decoded

1176 ns  Instruction : CPR, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1184 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 01

register

AR[00]= 82, DR[01]= 66, GR[10]= 66, PR[11]= 1c

1200 ns  FETCH STATE: Instruction is Fetched

1208 ns  DECODE STATE: Instruction is Decoded

1216 ns  Instruction : LLS, 2

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1224 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 2

AR[00]= 82, DR[01]= 66, GR[10]= 62, PR[11]= 1d

1240 ns  FETCH STATE: Instruction is Fetched

1248 ns  DECODE STATE: Instruction is Decoded

1256 ns  Instruction : LMS, 9

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1264 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 9

AR[00]= 82, DR[01]= 66, GR[10]= 92, PR[11]= 1e

1280 ns  FETCH STATE: Instruction is Fetched

1288 ns  DECODE STATE: Instruction is Decoded

1296 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1304 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 92, DR[01]= 66, GR[10]= 92, PR[11]= 1f

1320 ns  FETCH STATE: Instruction is Fetched

1328 ns  DECODE STATE: Instruction is Decoded

1336 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1344 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 92 to Memory Address...

1352 ns  EXECUTE_2 STATE: Read the Data 88 from Memory Location 92 and Stored into 10 Register

AR[00]= 92, DR[01]= 66, GR[10]= 88, PR[11]= 20

1368 ns  FETCH STATE: Instruction is Fetched

1376 ns  DECODE STATE: Instruction is Decoded

1384 ns  Instruction : SUB, 10, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1392 ns  EXECUTE_1 STATE: Executing SUB...Providing ALU Opcodes 88 and 66 and Adding them...

1400 ns  EXECUTE_2 STATE: Subtracted the 01 Register Data 66 from 10 Register

Data 88 and the Result is Stored into 10 Register

AR[00]= 92, DR[01]= 66, GR[10]= 22, PR[11]= 21

1416 ns  FETCH STATE: Instruction is Fetched

1424 ns  DECODE STATE: Instruction is Decoded

1432 ns  Instruction : LLS, 2

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1440 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 2

AR[00]= 92, DR[01]= 66, GR[10]= 22, PR[11]= 22

1456 ns  FETCH STATE: Instruction is Fetched

1464 ns  DECODE STATE: Instruction is Decoded

1472 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1480 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 92, DR[01]= 66, GR[10]= 82, PR[11]= 23

1496 ns  FETCH STATE: Instruction is Fetched

1504 ns  DECODE STATE: Instruction is Decoded

1512 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1520 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 82, DR[01]= 66, GR[10]= 82, PR[11]= 24

1536 ns  FETCH STATE: Instruction is Fetched

1544 ns  DECODE STATE: Instruction is Decoded

1552 ns  Instruction : WRM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1560 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = 82  to Memory Address...

1568 ns  EXECUTE_2 STATE: 10 Register Data 82 is Written at Memory Address 82

WRITTEN DATA : RAM[80h]= 89, RAM[81h]= cc, RAM[82h]= 82, RAM[FO]= xx

- Case 4:

1584 ns  FETCH STATE: Instruction is Fetched

1592 ns  DECODE STATE: Instruction is Decoded

1600 ns  Instruction : CFR;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1608 ns  EXECUTE_1 STATE: Executed CFR and Stored data  at LSBs of GR = 1

AR[00]= 82, DR[01]= 66, GR[10]= 81, PR[11]= 26

1624 ns  FETCH STATE: Instruction is Fetched

1632 ns  DECODE STATE: Instruction is Decoded

1640 ns  Instruction : CPR, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1648 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 01 register

AR[00]= 82, DR[01]= 81, GR[10]= 81, PR[11]= 27

1664 ns  FETCH STATE: Instruction is Fetched

1672 ns  DECODE STATE: Instruction is Decoded

1680 ns  Instruction : LLS, 0

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1688 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 0

AR[00]= 82, DR[01]= 81, GR[10]= 80, PR[11]= 28

1704 ns  FETCH STATE: Instruction is Fetched

1712 ns  DECODE STATE: Instruction is Decoded

1720 ns  Instruction : LMS, f

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1728 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = f

AR[00]= 82, DR[01]= 81, GR[10]= f0, PR[11]= 29

1744 ns  FETCH STATE: Instruction is Fetched

1752 ns  DECODE STATE: Instruction is Decoded

1760 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1768 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00 register

AR[00]= f0, DR[01]= 81, GR[10]= f0, PR[11]= 2a

1784 ns  FETCH STATE: Instruction is Fetched

1792 ns  DECODE STATE: Instruction is Decoded

1800 ns  Instruction : WRM, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1808 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = f0  to Memory Address...

1816 ns  EXECUTE_2 STATE: 01 Register Data 81 is Written at Memory Address f0

WRITTEN DATA : RAM[80h]= 89, RAM[81h]= cc,  RAM[82h]= 82,  RAM[FO]= 81

1832 ns  FETCH STATE: Instruction is Fetched

1840 ns  DECODE STATE: Instruction is Decoded

1848 ns  Instruction : NOP

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1856 ns  EXECUTE_1 STATE: Executed NOP and  No Operation is Perfomed

- Case 5:

1872 ns  FETCH STATE: Instruction is Fetched

1880 ns  DECODE STATE: Instruction is Decoded

1888 ns  Instruction : LLS, 0

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1896 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 0

AR[00]= f0, DR[01]= 81, GR[10]= f0, PR[11]= 2d

1912 ns  FETCH STATE: Instruction is Fetched

1920 ns  DECODE STATE: Instruction is Decoded

1928 ns  Instruction : LMS, 0

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1936 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 0

AR[00]= f0, DR[01]= 81, GR[10]= 00, PR[11]= 2e

1952 ns  FETCH STATE: Instruction is Fetched

1960 ns  DECODE STATE: Instruction is Decoded

1968 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

1976 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 00, DR[01]= 81, GR[10]= 00, PR[11]= 2f

1992 ns  FETCH STATE: Instruction is Fetched

2000 ns  DECODE STATE: Instruction is Decoded

2008 ns  Instruction : JMP

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2016 ns  EXECUTE_1 STATE: Executed JMP and Jumped to Instruction stored at 00

Memory Location

2032 ns  FETCH STATE: Instruction is Fetched

2040 ns  DECODE STATE: Instruction is Decoded

2048 ns  Instruction : LLS, 5

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2056 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 5

AR[00]= 00, DR[01]= 81, GR[10]= 05, PR[11]= 01

2072 ns  FETCH STATE: Instruction is Fetched

2080 ns  DECODE STATE: Instruction is Decoded

2088 ns  Instruction : LMS, 4

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2096 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 4

AR[00]= 00, DR[01]= 81, GR[10]= 45, PR[11]= 02

2112 ns  FETCH STATE: Instruction is Fetched

2120 ns  DECODE STATE: Instruction is Decoded

2128 ns  Instruction : CPR, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2136 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 01 register

AR[00]= 00, DR[01]= 45, GR[10]= 45, PR[11]= 03

2152 ns  FETCH STATE: Instruction is Fetched

2160 ns  DECODE STATE: Instruction is Decoded

2168 ns  Instruction : LLS, 0

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2176 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 0

AR[00]= 00, DR[01]= 45, GR[10]= 40, PR[11]= 04

2192 ns  FETCH STATE: Instruction is Fetched

2200 ns  DECODE STATE: Instruction is Decoded

2208 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2216 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 00, DR[01]= 45, GR[10]= 80, PR[11]= 05

2232 ns  FETCH STATE: Instruction is Fetched

2240 ns  DECODE STATE: Instruction is Decoded

2248 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2256 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 80, DR[01]= 45, GR[10]= 80, PR[11]= 06

2272 ns  FETCH STATE: Instruction is Fetched

2280 ns  DECODE STATE: Instruction is Decoded

2288 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2296 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 80 to Memory Address...

2304 ns  EXECUTE_2 STATE: Read the Data 89 from Memory Location 80 and Stored into 10 Register

AR[00]= 80, DR[01]= 45, GR[10]= 89, PR[11]= 07

2320 ns  FETCH STATE: Instruction is Fetched

2328 ns  DECODE STATE: Instruction is Decoded

2336 ns  Instruction : ADD, 10, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2344 ns  EXECUTE_1 STATE: Executing ADD...Providing ALU Opcodes 89 and 45 and

Adding them...

2352 ns  EXECUTE_2 STATE: Added the 10 Register Data 89 with 01 Register Data 45 and the Result is Stored into 10 Register

AR[00]= 80, DR[01]= 45, GR[10]= ce, PR[11]= 08

2368 ns  FETCH STATE: Instruction is Fetched

2376 ns  DECODE STATE: Instruction is Decoded

2384 ns  Instruction : WRM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2392 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = 80  to Memory

Address...

2400 ns  EXECUTE_2 STATE: 10 Register Data ce is Written at Memory Address 80

WRITTEN DATA : RAM[80h]= ce, RAM[81h]= cc, RAM[82h]= 82, RAM[FO]= 81

2416 ns  FETCH STATE: Instruction is Fetched

2424 ns  DECODE STATE: Instruction is Decoded

2432 ns  Instruction : LLS, 1

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2440 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 1

AR[00]= 80, DR[01]= 45, GR[10]= c1, PR[11]= 0a

2456 ns  FETCH STATE: Instruction is Fetched

2464 ns  DECODE STATE: Instruction is Decoded

2472 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2480 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 80, DR[01]= 45, GR[10]= 81, PR[11]= 0b

2496 ns  FETCH STATE: Instruction is Fetched

2504 ns  DECODE STATE: Instruction is Decoded

2512 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2520 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 81, DR[01]= 45, GR[10]= 81, PR[11]= 0c

2536 ns  FETCH STATE: Instruction is Fetched

2544 ns  DECODE STATE: Instruction is Decoded

2552 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2560 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 81 to Memory Address...

2568 ns  EXECUTE_2 STATE: Read the Data cc from Memory Location 81 and Stored

into 10 Register

AR[00]= 81, DR[01]= 45, GR[10]= cc, PR[11]= 0d

2584 ns  FETCH STATE: Instruction is Fetched

2592 ns  DECODE STATE: Instruction is Decoded

2600 ns  Instruction : CPR, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2608 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 01

register

AR[00]= 81, DR[01]= cc, GR[10]= cc, PR[11]= 0e

2624 ns  FETCH STATE: Instruction is Fetched

2632 ns  DECODE STATE: Instruction is Decoded

2640 ns  Instruction : LLS, 1

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2648 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 1

AR[00]= 81, DR[01]= cc, GR[10]= c1, PR[11]= 0f

2664 ns  FETCH STATE: Instruction is Fetched

2672 ns  DECODE STATE: Instruction is Decoded

2680 ns  Instruction : LMS, 9

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2688 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 9

AR[00]= 81, DR[01]= cc, GR[10]= 91, PR[11]= 10

2704 ns  FETCH STATE: Instruction is Fetched

2712 ns  DECODE STATE: Instruction is Decoded

2720 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2728 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 91, DR[01]= cc, GR[10]= 91, PR[11]= 11

2744 ns  FETCH STATE: Instruction is Fetched

2752 ns  DECODE STATE: Instruction is Decoded

2760 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2768 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 91 to Memory Address...

2776 ns  EXECUTE_2 STATE: Read the Data 77 from Memory Location 91 and Stored

into 10 Register

AR[00]= 91, DR[01]= cc, GR[10]= 77, PR[11]= 12

2792 ns  FETCH STATE: Instruction is Fetched

2800 ns  DECODE STATE: Instruction is Decoded

2808 ns  Instruction : ADD, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2816 ns  EXECUTE_1 STATE: Executing ADD...Providing ALU Opcodes cc and 77 and

Adding them...

2824 ns  EXECUTE_2 STATE: Added the 01 Register Data cc with 10 Register Data

77 and the Result is Stored into 01 Register

AR[00]= 91, DR[01]= 43, GR[10]= 77, PR[11]= 13

2840 ns  FETCH STATE: Instruction is Fetched

2848 ns  DECODE STATE: Instruction is Decoded

2856 ns  Instruction : LLS, 1

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2864 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 1

AR[00]= 91, DR[01]= 43, GR[10]= 71, PR[11]= 14

2880 ns  FETCH STATE: Instruction is Fetched

2888 ns  DECODE STATE: Instruction is Decoded

2896 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2904 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 91, DR[01]= 43, GR[10]= 81, PR[11]= 15

2920 ns  FETCH STATE: Instruction is Fetched

2928 ns  DECODE STATE: Instruction is Decoded

2936 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2944 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 81, DR[01]= 43, GR[10]= 81, PR[11]= 16

2960 ns  FETCH STATE: Instruction is Fetched

2968 ns  DECODE STATE: Instruction is Decoded

2976 ns  Instruction : WRM, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

2984 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = 81  to Memory Address...

2992 ns  EXECUTE_2 STATE: 01 Register Data 43 is Written at Memory Address 81

WRITTEN DATA : RAM[80h]= ce,RAM[81h]= 43, RAM[82h]= 82,  RAM[FO]= 81

3008 ns  FETCH STATE: Instruction is Fetched

3016 ns  DECODE STATE: Instruction is Decoded

3024 ns  Instruction : LLS, 2

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3032 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 2

AR[00]= 81, DR[01]= 43, GR[10]= 82, PR[11]= 18

3048 ns  FETCH STATE: Instruction is Fetched

3056 ns  DECODE STATE: Instruction is Decoded

3064 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3072 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 81, DR[01]= 43, GR[10]= 82, PR[11]= 19

3088 ns  FETCH STATE: Instruction is Fetched

3096 ns  DECODE STATE: Instruction is Decoded

3104 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3112 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 82, DR[01]= 43, GR[10]= 82, PR[11]= 1a

3128 ns  FETCH STATE: Instruction is Fetched

3136 ns  DECODE STATE: Instruction is Decoded

3144 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3152 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 82 to Memory Address...

3160 ns  EXECUTE_2 STATE: Read the Data 82 from Memory Location 82 and Stored into 10 Register

AR[00]= 82, DR[01]= 43, GR[10]= 82, PR[11]= 1b

3176 ns  FETCH STATE: Instruction is Fetched

3184 ns  DECODE STATE: Instruction is Decoded

3192 ns  Instruction : CPR, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3200 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 01

register

AR[00]= 82, DR[01]= 82, GR[10]= 82, PR[11]= 1c

3216 ns  FETCH STATE: Instruction is Fetched

3224 ns  DECODE STATE: Instruction is Decoded

3232 ns  Instruction : LLS, 2

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3240 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 2

AR[00]= 82, DR[01]= 82, GR[10]= 82, PR[11]= 1d

3256 ns  FETCH STATE: Instruction is Fetched

3264 ns  DECODE STATE: Instruction is Decoded

3272 ns  Instruction : LMS, 9

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3280 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 9

AR[00]= 82, DR[01]= 82, GR[10]= 92, PR[11]= 1e

3296 ns  FETCH STATE: Instruction is Fetched

3304 ns  DECODE STATE: Instruction is Decoded

3312 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3320 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 92, DR[01]= 82, GR[10]= 92, PR[11]= 1f

3336 ns  FETCH STATE: Instruction is Fetched

3344 ns  DECODE STATE: Instruction is Decoded

3352 ns  Instruction : RDM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3360 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 92 to Memory Address...

3368 ns  EXECUTE_2 STATE: Read the Data 88 from Memory Location 92 and Stored

into 10 Register

AR[00]= 92, DR[01]= 82, GR[10]= 88, PR[11]= 20

3384 ns  FETCH STATE: Instruction is Fetched

3392 ns  DECODE STATE: Instruction is Decoded

3400 ns  Instruction : SUB, 10, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3408 ns  EXECUTE_1 STATE: Executing SUB...Providing ALU Opcodes 88 and 82 and

Adding them...

3416 ns  EXECUTE_2 STATE: Subtracted the 01 Register Data 82 from 10 Register

Data 88 and the Result is Stored into 10 Register

AR[00]= 92, DR[01]= 82, GR[10]= 06, PR[11]= 21

3432 ns  FETCH STATE: Instruction is Fetched

3440 ns  DECODE STATE: Instruction is Decoded

3448 ns  Instruction : LLS, 2

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3456 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 2

AR[00]= 92, DR[01]= 82, GR[10]= 02, PR[11]= 22

3472 ns  FETCH STATE: Instruction is Fetched

3480 ns  DECODE STATE: Instruction is Decoded

3488 ns  Instruction : LMS, 8

UPDATE STATE:PR Register is Incremented to Fetch the next

Instruction.

3496 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 8

AR[00]= 92, DR[01]= 82, GR[10]= 82, PR[11]= 23

3512 ns  FETCH STATE: Instruction is Fetched

3520 ns  DECODE STATE: Instruction is Decoded

3528 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3536 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00

register

AR[00]= 82, DR[01]= 82, GR[10]= 82, PR[11]= 24

3552 ns  FETCH STATE: Instruction is Fetched

3560 ns  DECODE STATE: Instruction is Decoded

3568 ns  Instruction : WRM, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction

3576 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = 82  to Memory

Address...

3584 ns  EXECUTE_2 STATE: 10 Register Data 82 is Written at Memory Address 82

3600 ns  FETCH STATE: Instruction is Fetched

3608 ns  DECODE STATE: Instruction is Decoded

3616 ns  Instruction : CFR;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3624 ns  EXECUTE_1 STATE: Executed CFR and Stored data  at LSBs of GR = 0

AR[00]= 82, DR[01]= 82, GR[10]= 80, PR[11]= 26

3640 ns  FETCH STATE: Instruction is Fetched

3648 ns  DECODE STATE: Instruction is Decoded

3656 ns  Instruction : CPR, 01, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3664 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 01

register

AR[00]= 82, DR[01]= 80, GR[10]= 80, PR[11]= 27

3680 ns  FETCH STATE: Instruction is Fetched

3688 ns  DECODE STATE: Instruction is Decoded

3696 ns  Instruction : LLS, 0

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3704 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 0

AR[00]= 82, DR[01]= 80, GR[10]= 80, PR[11]= 28

3720 ns  FETCH STATE: Instruction is Fetched

3728 ns  DECODE STATE: Instruction is Decoded

3736 ns  Instruction : LMS, f

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3744 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = f

AR[00]= 82, DR[01]= 80, GR[10]= f0, PR[11]= 29

3760 ns  FETCH STATE: Instruction is Fetched

3768 ns  DECODE STATE: Instruction is Decoded

3776 ns  Instruction : CPR, 00, 10;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3784 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 10 to 00 register

AR[00]= f0, DR[01]= 80, GR[10]= f0, PR[11]= 2a

3800 ns  FETCH STATE: Instruction is Fetched

3808 ns  DECODE STATE: Instruction is Decoded

3816 ns  Instruction : WRM, 01;

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3824 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = f0  to Memory Address...

3832 ns  EXECUTE_2 STATE: 01 Register Data 80 is Written at Memory Address f0

WRITTEN DATA : RAM[80h]= ce, RAM[81h]= 43,RAM[82h]= 82,  RAM[FO]= 80

3848 ns  FETCH STATE: Instruction is Fetched

3856 ns  DECODE STATE: Instruction is Decoded

3864 ns  Instruction : NOP

UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

3872 ns  EXECUTE_1 STATE: Executed NOP and  No Operation is Perfomed

3888 ns  FETCH STATE: Instruction is Fetched

$finish called from file "scalar_test.v", line 133.

$finish at simulation time               388900

V C S   S i m u l a t i o n   R e p o r t

Time: 3889000 ps

CPU Time:      0.270 seconds;       Data structure size:   0.0Mb

Fri Dec  5 03:56:12 2014

C.3	Reports (contents) from Synthesis (Design Compiler)

Warning: DC is only available in Tcl and XG mode. The -tcl_mode and -xg_mode options are no longer required.

DC Professional (TM)

DC Expert (TM)

DC Ultra (TM)

FloorPlan Manager (TM)

HDL Compiler (TM)

VHDL Compiler (TM)

Library Compiler (TM)

DesignWare Developer (TM)

DFT Compiler (TM)

BSD Compiler

Power Compiler (TM)

Version C-2009.06-SP5 for linux -- Jan 15, 2010

Copyright (c) 1988-2009 by Synopsys, Inc.

ALL RIGHTS RESERVED

This software and the associated documentation are confidential and

proprietary to Synopsys, Inc. Your use or disclosure of this software

is subject to the terms and conditions of a written license agreement

between you, or your company, and Synopsys, Inc.

The above trademark notice does not imply that you are licensed to use

all of the listed products. You are licensed to use only those products

for which you have lawfully obtained a valid license key.

Initializing...

set link_library {/apps/toshiba/sjsu/synopsys/tc240c/tc240c.db_WCCOM25}

/apps/toshiba/sjsu/synopsys/tc240c/tc240c.db_WCCOM25

set target_library {/apps/toshiba/sjsu/synopsys/tc240c/tc240c.db_WCCOM25}

/apps/toshiba/sjsu/synopsys/tc240c/tc240c.db_WCCOM25

set symbol_library {/apps/toshiba/sjsu/synopsys/tc240c/tc240c.workview.sdb}

/apps/toshiba/sjsu/synopsys/tc240c/tc240c.workview.sdb

set synthetic_library {dw_foundation.sldb standard.sldb}

dw_foundation.sldb standard.sldb

set_min_library /apps/toshiba/sjsu/synopsys/tc240c/tc240c.db_WCCOM25 -min_version /apps/toshiba/sjsu/synopsys/tc240c/tc240c.db_WCCOM25

Loading db file '/apps/toshiba/sjsu/synopsys/tc240c/tc240c.db_WCCOM25'

Error: Minimum version must be a different library. (TIM-98)

0

read_verilog 271control.v

Loading db file '/apps/synopsys/SYNTH/libraries/syn/gtech.db'

Loading db file '/apps/synopsys/SYNTH/libraries/syn/standard.sldb'

Loading link library 'tc240c'

Warning: Function '=' leaked 1 allocations for 16 bytes. (EQN-21)

Loading link library 'gtech'

Loading verilog file '/home/pa/pate8552/271new/271control.v'

Detecting input file type automatically (-rtl or -netlist).

Running DC verilog reader

Reading with Presto HDL Compiler (equivalent to -rtl option).

Running PRESTO HDLC

Loading db file '/apps/synopsys/SYNTH/libraries/syn/dw_foundation.sldb'

Warning: The following synthetic libraries should be added to

the list of link libraries:

'dw_foundation.sldb'. (UISN-26)

Compiling source file /home/pa/pate8552/271new/271control.v

Warning:  Little argument or return value checking implemented for system task or function '$time'. (VER-209)

Warning:  /home/pa/pate8552/271new/271control.v:173: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:180: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:187: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:187: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:194: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:194: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:201: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:201: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:208: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:215: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:240: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:248: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:256: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:265: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:265: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:273: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:273: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:283: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:283: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:293: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:301: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:309: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:331: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:331: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:331: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:339: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:339: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:339: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:350: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:350: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:350: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:350: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:350: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:359: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:359: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:359: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:359: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:359: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:398: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:398: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:398: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:398: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:403: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:403: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:403: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:403: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:408: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:408: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:408: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:408: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:413: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:413: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:413: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:413: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:418: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:418: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:418: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:418: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:423: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:423: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:423: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:423: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:428: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:428: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:428: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:428: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:173: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:180: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:187: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:187: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:194: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:194: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:201: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:201: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:208: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:215: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:240: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:248: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:256: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:265: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:265: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:273: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:273: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:283: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:283: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:293: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:301: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:309: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:331: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:331: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:331: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:339: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:339: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:339: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:350: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:350: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:350: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:350: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:350: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:359: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:359: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:359: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:359: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:359: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:398: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:398: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:398: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:398: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:403: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:403: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:403: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:403: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:408: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:408: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:408: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:408: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:413: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:413: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:413: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:413: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:418: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:418: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:418: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:418: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:423: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:423: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:423: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:423: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:428: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:428: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:428: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:428: Invalid escape sequence '\0' in call to '$display'. (VER-941)

Warning:  /home/pa/pate8552/271new/271control.v:75: 'OPRN' is being read, but does not appear in the sensitivity list of the block. (ELAB-292)

Warning:  /home/pa/pate8552/271new/271control.v:75: 'cout' is being read, but does not appear in the sensitivity list of the block. (ELAB-292)

Statistics for case statements in always block at line 87 in file

'/home/pa/pate8552/271new/271control.v'

===============================================

|           Line           |  full/ parallel  |

===============================================

|            89            |     no/auto      |

===============================================

$display output: 00 ns  FETCH STATE: Instruction is Fetched

$display output: 00 ns  DECODE STATE: Instruction is Decoded

$display output: 00 ns  Instruction : NOP

$display output:          UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

$display output: 00 ns  Instruction : JMP

$display output:          UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

$display output: 00 ns  Instruction : RDM, 2b;

?$display output:          UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

$display output: 00 ns  Instruction : WRM, 2b;

?$display output:          UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

$display output: 00 ns  Instruction : CPR, 2b, 2b;

??$display output:          UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

$display output: 00 ns  Instruction : ADD, 2b, 2b;

??$display output:          UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

$display output: 00 ns  Instruction : SUB, 2b, 2b;

??$display output:          UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

$display output: 00 ns  Instruction : LLS, 2h

?$display output:          UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

$display output: 00 ns  Instruction : LMS, 2h

?$display output:          UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

$display output: 00 ns  Instruction : CFR;

$display output:          UPDATE STATE:PR Register is Incremented to Fetch the next Instruction.

$display output: $display output: 00 ns  EXECUTE_1 STATE: Executed NOP and  No Operation is Perfomed

$display output: 00 ns  EXECUTE_1 STATE: Executed JMP and Jumped to Instruction stored at 8H Memory Location

?$display output: 00 ns  EXECUTE_1 STATE: Executing RDM...Setting AR = 8h to Memory Address...

?$display output: 00 ns  EXECUTE_1 STATE: Executing WRM...Setting AR = 8h  to Memory Address...

?$display output: 00 ns  EXECUTE_1 STATE: Executed CPR and  Copied the Contents of 2b to 2b register

??$display output: 00 ns  EXECUTE_1 STATE: Executing ADD...Providing ALU Opcodes 8h and 8h and Adding them...

??$display output: 00 ns  EXECUTE_1 STATE: Executing SUB...Providing ALU Opcodes 8h and 8h and Adding them...

??$display output: 00 ns  EXECUTE_1 STATE: Executed LLS and Stored data at LSBs of GR = 2h

?$display output: 00 ns  EXECUTE_1 STATE: Executed LMS and Stored data at MSBs of GR = 2h

?$display output: 00 ns  EXECUTE_1 STATE: Executed CFR and Stored data  at LSBs of GR = 2h

?$display output: 00 ns EXECUTE_1 STATE: INVALID INSTRUCTION $display output: 00 ns  EXECUTE_2 STATE: Read the Data 8h from Memory Location 8h and Stored into 2b Register

???$display output: 00 ns  EXECUTE_2 STATE: 2b Register Data 8h is Written at Memory Address 8h

???$display output: 00 ns  EXECUTE_2 STATE: Added the 2b Register Data 8h with 2b Register Data 8h and the Result is Stored into 2b Register

?????$display output: 00 ns  EXECUTE_2 STATE: Subtracted the 2b Register Data 8h from 2b Register Data 8h and the Result is Stored into 2b Register

?????$display output: $display output:          AR[00]= 8h, DR[01]= 8h, GR[10]= 8h, PR[11]= 8h

????$display output:          AR[00]= 8h, DR[01]= 8h, GR[10]= 8h, PR[11]= 8h

????$display output:          AR[00]= 8h, DR[01]= 8h, GR[10]= 8h, PR[11]= 8h

????$display output:          AR[00]= 8h, DR[01]= 8h, GR[10]= 8h, PR[11]= 8h

????$display output:          AR[00]= 8h, DR[01]= 8h, GR[10]= 8h, PR[11]= 8h

????$display output:          AR[00]= 8h, DR[01]= 8h, GR[10]= 8h, PR[11]= 8h

????$display output:          AR[00]= 8h, DR[01]= 8h, GR[10]= 8h, PR[11]= 8h

????$display output:

Statistics for case statements in always block at line 105 in file

'/home/pa/pate8552/271new/271control.v'

===============================================

|           Line           |  full/ parallel  |

===============================================

|           135            |     no/auto      |

|           157            |    auto/auto     |

|           233            |    auto/auto     |

|           321            |    auto/auto     |

|           394            |    auto/auto     |

===============================================

Inferred memory devices in process

in routine SCALAR_PROCESSOR line 80 in file

'/home/pa/pate8552/271new/271control.v'.

===============================================================================

|    Register Name    |   Type    | Width | Bus | MB | AR | AS | SR | SS | ST |

===============================================================================

|      state_reg      | Flip-flop |   3   |  Y  | N  | Y  | N  | N  | N  | N  |

===============================================================================

Inferred memory devices in process

in routine SCALAR_PROCESSOR line 87 in file

'/home/pa/pate8552/271new/271control.v'.

===========================================================================

|    Register Name    | Type  | Width | Bus | MB | AR | AS | SR | SS | ST |

===========================================================================

|     n_state_reg     | Latch |   3   |  Y  | N  | N  | N  | -  | -  | -  |

===========================================================================

Inferred memory devices in process

in routine SCALAR_PROCESSOR line 105 in file

'/home/pa/pate8552/271new/271control.v'.

===============================================================================

|    Register Name    |   Type    | Width | Bus | MB | AR | AS | SR | SS | ST |

===============================================================================

|       FR_reg        | Flip-flop |   4   |  Y  | N  | Y  | N  | N  | N  | N  |

|       OP2_reg       | Flip-flop |   8   |  Y  | N  | Y  | N  | N  | N  | N  |

|      ARRAY_reg      | Flip-flop |  32   |  N  | N  | Y  | N  | N  | N  | N  |

|        S_reg        | Flip-flop |   2   |  Y  | N  | Y  | N  | N  | N  | N  |

|       rd_reg        | Flip-flop |   1   |  N  | N  | Y  | N  | N  | N  | N  |

|       OP1_reg       | Flip-flop |   8   |  Y  | N  | Y  | N  | N  | N  | N  |

|      addr_reg       | Flip-flop |   8   |  Y  | N  | Y  | N  | N  | N  | N  |

|       wrt_reg       | Flip-flop |   1   |  N  | N  | Y  | N  | N  | N  | N  |

|      OPRN_reg       | Flip-flop |   1   |  N  | N  | Y  | N  | N  | N  | N  |

|        D_reg        | Flip-flop |   2   |  Y  | N  | Y  | N  | N  | N  | N  |

|     opcode_reg      | Flip-flop |   4   |  Y  | N  | Y  | N  | N  | N  | N  |

|      dat_T_reg      | Flip-flop |   8   |  Y  | N  | Y  | N  | N  | N  | N  |

===============================================================================

Inferred tri-state devices in process

in routine SCALAR_PROCESSOR line 55 in file

'/home/pa/pate8552/271new/271control.v'.

=================================================

| Register Name |       Type       | Width | MB |

=================================================

|    dat_tri    | Tri-State Buffer |   8   | N  |

=================================================

Statistics for MUX_OPs

===============================================================

|   block name/line    | Inputs | Outputs | # sel inputs | MB |

===============================================================

| SCALAR_PROCESSOR/258 |   4    |    8    |      2       | N  |

| SCALAR_PROCESSOR/266 |   4    |    8    |      2       | N  |

===============================================================

Presto compilation completed successfully.

Current design is now '/home/pa/pate8552/271new/SCALAR_PROCESSOR.db:SCALAR_PROCESSOR'

Loaded 5 designs.

Current design is 'SCALAR_PROCESSOR'.

SCALAR_PROCESSOR ALU RCA_8 add_full add_half

current_design SCALAR_PROCESSOR

Current design is 'SCALAR_PROCESSOR'.

{SCALAR_PROCESSOR}

link

Linking design 'SCALAR_PROCESSOR'

Using the following designs and libraries:

--------------------------------------------------------------------------

tc240c (library)            /apps/toshiba/sjsu/synopsys/tc240c/tc240c.db_WCCOM25

1

check_design

Warning: In design 'SCALAR_PROCESSOR', cell 'C2376' does not drive any nets. (LINT-1)

Warning: In design 'SCALAR_PROCESSOR', cell 'C2377' does not drive any nets. (LINT-1)

Warning: In design 'SCALAR_PROCESSOR', cell 'C2378' does not drive any nets. (LINT-1)

Warning: In design 'SCALAR_PROCESSOR', cell 'C2379' does not drive any nets. (LINT-1)

Warning: In design 'SCALAR_PROCESSOR', cell 'C2380' does not drive any nets. (LINT-1)

Warning: In design 'SCALAR_PROCESSOR', cell 'C2381' does not drive any nets. (LINT-1)

Warning: In design 'SCALAR_PROCESSOR', cell 'C2382' does not drive any nets. (LINT-1)

Warning: In design 'SCALAR_PROCESSOR', cell 'C2383' does not drive any nets. (LINT-1)

Warning: In design 'SCALAR_PROCESSOR', cell 'C2384' does not drive any nets. (LINT-1)

Warning: Net 'dat[0]' has a single tri-state driver.  (LINT-63)

Warning: Net 'dat[1]' has a single tri-state driver.  (LINT-63)

Warning: Net 'dat[2]' has a single tri-state driver.  (LINT-63)

Warning: Net 'dat[3]' has a single tri-state driver.  (LINT-63)

Warning: Net 'dat[4]' has a single tri-state driver.  (LINT-63)

Warning: Net 'dat[5]' has a single tri-state driver.  (LINT-63)

Warning: Net 'dat[6]' has a single tri-state driver.  (LINT-63)

Warning: Net 'dat[7]' has a single tri-state driver.  (LINT-63)

Information: Design 'SCALAR_PROCESSOR' has multiply instantiated designs. Use the '-multiple_designs' switch for more information. (LINT-78)

1

create_clock CLK -name CLK -period 4.0000000

Warning: Can't find object 'CLK' in design 'SCALAR_PROCESSOR'. (UID-95)

Error: Value for list 'source_objects' must have 1 elements. (CMD-036)

0

set_propagated_clock CLK

Warning: Can't find object 'CLK' in design 'SCALAR_PROCESSOR'. (UID-95)

Error: Value for list 'object_list' must have 1 elements. (CMD-036)

0

set_clock_uncertainty .25 CLK

Warning: Can't find object 'CLK' in design 'SCALAR_PROCESSOR'. (UID-95)

Error: Value for list 'object_list' must have 1 elements. (CMD-036)

0

set_max_delay 1.1 -from [all_inputs]

1

set_max_delay 1.1 -to [all_outputs]

1

#set_max_delay 2 -to sum

set_load 160 sum

Warning: Can't find object 'sum' in design 'SCALAR_PROCESSOR'. (UID-95)

Error: Value for list 'objects' must have 1 elements. (CMD-036)

0

set_max_area 500

1

compile -map_effort high

Warning: The following synthetic libraries should be added to

the list of link libraries:

'dw_foundation.sldb'. (UISN-26)

Information: Checking out the license 'DesignWare'. (SEC-104)

Information: Evaluating DesignWare library utilization. (UISN-27)

============================================================================

| DesignWare Building Block Library       |      Version       | Available |

============================================================================

| Basic DW Building Blocks                | C-2009.06-DWBB_0912 |    *     |

| Licensed DW Building Blocks             | C-2009.06-DWBB_0912 |    *     |

============================================================================

Warning: Net 'dat[0]' has a single tri-state driver.  (LINT-63)

Warning: Net 'dat[1]' has a single tri-state driver.  (LINT-63)

Warning: Net 'dat[2]' has a single tri-state driver.  (LINT-63)

Warning: Net 'dat[3]' has a single tri-state driver.  (LINT-63)

Warning: Net 'dat[4]' has a single tri-state driver.  (LINT-63)

Warning: Net 'dat[5]' has a single tri-state driver.  (LINT-63)

Warning: Net 'dat[6]' has a single tri-state driver.  (LINT-63)

Warning: Net 'dat[7]' has a single tri-state driver.  (LINT-63)

Information: There are 9 potential problems in your design. Please run 'check_design' for more information. (LINT-99)

Warning: Operating condition WCCOM25 set on design SCALAR_PROCESSOR has different process,

voltage and temperatures parameters than the parameters at which target library

tc240c is characterized. Delays may be inaccurate as a result. (OPT-998)

Beginning Pass 1 Mapping

------------------------

Processing 'add_half_0'

Processing 'add_full_0'

Processing 'RCA_8'

Processing 'ALU'

Processing 'SCALAR_PROCESSOR'

Updating timing information

Information: Updating design information... (UID-85)

Beginning Implementation Selection

----------------------------------

Processing 'SCALAR_PROCESSOR_DW01_inc_0'

Beginning Mapping Optimizations  (High effort)

-------------------------------

ELAPSED            WORST NEG TOTAL NEG  DESIGN

TIME      AREA      SLACK     SLACK   RULE COST         ENDPOINT

--------- --------- --------- --------- --------- -------------------------

0:00:02    1597.0      0.04       0.8       0.0

0:00:02    1095.0      0.53       5.4       0.0

0:00:02    1099.5      0.22       2.6       0.0

0:00:02    1103.5      0.11       1.1       0.0

0:00:02    1112.5      0.11       0.9       0.0

0:00:02    1121.5      0.07       0.6       0.0

0:00:02    1119.5      0.06       0.6       0.0

0:00:02    1116.5      0.04       0.4       0.0

0:00:02    1111.5      0.04       0.3       0.0

0:00:02    1113.0      0.03       0.2       0.0

0:00:02    1113.5      0.03       0.2       0.0

0:00:02    1114.0      0.03       0.1       0.0

0:00:02    1114.5      0.03       0.1       0.0

0:00:03    1113.0      0.03       0.1       0.0

0:00:03    1115.5      0.03       0.1       0.0

0:00:03    1115.5      0.03       0.1       0.0

0:00:03    1115.5      0.03       0.1       0.0

0:00:03    1115.5      0.03       0.1       0.0

0:00:03    1115.5      0.03       0.1       0.0

0:00:03    1115.5      0.03       0.1       0.0

0:00:03    1115.5      0.03       0.1       0.0

Beginning Delay Optimization Phase

----------------------------------

ELAPSED            WORST NEG TOTAL NEG  DESIGN

TIME      AREA      SLACK     SLACK   RULE COST         ENDPOINT

--------- --------- --------- --------- --------- -------------------------

0:00:03    1115.5      0.03       0.1       0.0

0:00:03    1113.0      0.00       0.0       0.0

Beginning Area-Recovery Phase  (max_area 500)

-----------------------------

ELAPSED            WORST NEG TOTAL NEG  DESIGN

TIME      AREA      SLACK     SLACK   RULE COST         ENDPOINT

--------- --------- --------- --------- --------- -------------------------

0:00:03    1113.0      0.00       0.0       0.0

0:00:03    1113.0      0.00       0.0       0.0

0:00:03    1112.0      0.00       0.0       0.0

0:00:03    1112.0      0.00       0.0       0.0

0:00:03    1112.0      0.00       0.0       0.0

0:00:03    1112.0      0.00       0.0       0.0

0:00:03    1112.0      0.00       0.0       0.0

0:00:03    1112.0      0.00       0.0       0.0

0:00:03    1112.0      0.00       0.0       0.0

0:00:03    1112.0      0.00       0.0       0.0

0:00:03    1112.0      0.00       0.0       0.0

0:00:03    1112.0      0.00       0.0       0.0

0:00:03    1111.0      0.00       0.0       0.0

0:00:03    1111.0      0.00       0.0       0.0

0:00:03    1111.0      0.00       0.0       0.0

0:00:03    1111.0      0.00       0.0       0.0

0:00:03    1111.0      0.00       0.0       0.0

0:00:03    1111.0      0.00       0.0       0.0

0:00:03    1111.0      0.00       0.0       0.0

0:00:03    1110.5      0.00       0.0       0.0

0:00:03    1110.0      0.00       0.0       0.0

0:00:03    1109.5      0.00       0.0       0.0

0:00:03    1108.5      0.00       0.0       0.0

0:00:03    1108.0      0.00       0.0       0.0

0:00:03    1108.0      0.00       0.0       0.0

0:00:03    1108.0      0.00       0.0       0.0

0:00:03    1108.0      0.00       0.0       0.0

0:00:03    1108.0      0.00       0.0       0.0

0:00:03    1108.0      0.00       0.0       0.0

0:00:03    1108.0      0.00       0.0       0.0

0:00:03    1108.0      0.00       0.0       0.0

0:00:03    1108.0      0.00       0.0       0.0

Loading db file '/apps/toshiba/sjsu/synopsys/tc240c/tc240c.db_WCCOM25'

Optimization Complete

---------------------

1

report_cell

Information: Updating design information... (UID-85)

****************************************

Report : cell

Design : SCALAR_PROCESSOR

Version: C-2009.06-SP5

Date   : Fri Dec  5 03:58:23 2014

****************************************

Attributes:

BO - reference allows boundary optimization

b - black box (unknown)

h - hierarchical

mo - map_only

n - noncombinational

r - removable

so - sizing only

u - contains unmapped logic

Cell                      Reference       Library             Area  Attributes

--------------------------------------------------------------------------------

ARRAY_reg[0][0]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[0][1]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[0][2]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[0][3]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[0][4]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[0][5]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[0][6]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[0][7]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[1][0]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[1][1]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[1][2]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[1][3]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[1][4]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[1][5]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[1][6]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[1][7]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[2][0]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[2][1]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[2][2]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[2][3]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[2][4]           CFD2QX2         tc240c          7.500000  n, so

ARRAY_reg[2][5]           CFD2QX2         tc240c          7.500000  n, so

ARRAY_reg[2][6]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[2][7]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[3][0]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[3][1]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[3][2]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[3][3]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[3][4]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[3][5]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[3][6]           CFD2QXL         tc240c          5.000000  n, so

ARRAY_reg[3][7]           CFD2QXL         tc240c          5.000000  n, so

D_reg[0]                  CFD2QXL         tc240c          5.000000  n, so

D_reg[1]                  CFD2QXL         tc240c          5.000000  n, so

FR_reg[0]                 CFD2QXL         tc240c          5.000000  n, so

FR_reg[1]                 CFD2QXL         tc240c          5.000000  n, so

FR_reg[2]                 CFD2QXL         tc240c          5.000000  n, so

FR_reg[3]                 CFD2QXL         tc240c          5.000000  n, so

OP1_reg[0]                CFD2QXL         tc240c          5.000000  n, so

OP1_reg[1]                CFD2QXL         tc240c          5.000000  n, so

OP1_reg[2]                CFD2QXL         tc240c          5.000000  n, so

OP1_reg[3]                CFD2QXL         tc240c          5.000000  n, so

OP1_reg[4]                CFD2QXL         tc240c          5.000000  n, so

OP1_reg[5]                CFD2QXL         tc240c          5.000000  n, so

OP1_reg[6]                CFD2QXL         tc240c          5.000000  n, so

OP1_reg[7]                CFD2QXL         tc240c          5.000000  n, so

OP2_reg[0]                CFD2QXL         tc240c          5.000000  n, so

OP2_reg[1]                CFD2QXL         tc240c          5.000000  n, so

OP2_reg[2]                CFD2QXL         tc240c          5.000000  n, so

OP2_reg[3]                CFD2QXL         tc240c          5.000000  n, so

OP2_reg[4]                CFD2QXL         tc240c          5.000000  n, so

OP2_reg[5]                CFD2QXL         tc240c          5.000000  n, so

OP2_reg[6]                CFD2QXL         tc240c          5.000000  n, so

OP2_reg[7]                CFD2QXL         tc240c          5.000000  n, so

OPRN_reg                  CFD2QXL         tc240c          5.000000  n, so

S_reg[0]                  CFD2QXL         tc240c          5.000000  n, so

S_reg[1]                  CFD2QXL         tc240c          5.000000  n, so

U74                       CAN4X1          tc240c          2.000000

U92                       CND2IX1         tc240c          1.500000

U124                      CAOR2X1         tc240c          2.500000

U126                      CAOR2X1         tc240c          2.500000

U127                      CAOR2X1         tc240c          2.500000

U130                      CAOR2X1         tc240c          2.500000

U135                      CAOR2X1         tc240c          2.500000

U139                      CAOR2X1         tc240c          2.500000

U143                      CAOR2X1         tc240c          2.500000

U147                      CAOR2X1         tc240c          2.500000

U151                      CAOR2X1         tc240c          2.500000

U155                      CAOR2X1         tc240c          2.500000

U159                      CAOR2X1         tc240c          2.500000

U163                      CAOR2X1         tc240c          2.500000

U170                      CAN3X2          tc240c          2.000000

U172                      CAOR2X1         tc240c          2.500000

U173                      CAOR2X1         tc240c          2.500000

U174                      CAOR2X1         tc240c          2.500000

U175                      CAOR2X1         tc240c          2.500000

U177                      COR8X1          tc240c          4.000000

U178                      COR2X1          tc240c          1.500000

U180                      CAOR2X1         tc240c          2.500000

U184                      CAOR2X1         tc240c          2.500000

U188                      CAOR2X1         tc240c          2.500000

U189                      CAOR2X1         tc240c          2.500000

U190                      CAOR2X1         tc240c          2.500000

U191                      CAOR2X1         tc240c          2.500000

U192                      CAOR2X1         tc240c          2.500000

U193                      CAOR2X1         tc240c          2.500000

U194                      CAOR2X1         tc240c          2.500000

U195                      CAOR2X1         tc240c          2.500000

U196                      CAOR2X1         tc240c          2.500000

U197                      CAOR2X1         tc240c          2.500000

U198                      CAOR2X1         tc240c          2.500000

U199                      CAOR2X1         tc240c          2.500000

U200                      CAOR2X1         tc240c          2.500000

U201                      CAOR2X1         tc240c          2.500000

U202                      CAOR2X1         tc240c          2.500000

U203                      CAOR2X1         tc240c          2.500000

U204                      CAOR2X1         tc240c          2.500000

U209                      CAOR2X1         tc240c          2.500000

U210                      CAOR2X1         tc240c          2.500000

U211                      CAOR2X1         tc240c          2.500000

U212                      CAOR2X1         tc240c          2.500000

U219                      CAOR2X1         tc240c          2.500000

U223                      CAOR1X1         tc240c          2.000000

U245                      COND1XL         tc240c          1.500000

U246                      COND1XL         tc240c          1.500000

U247                      COAN1X1         tc240c          2.000000

U248                      CND3XL          tc240c          1.500000

U249                      CND5X1          tc240c          3.500000

U250                      COAN1X1         tc240c          2.000000

U251                      CND3XL          tc240c          1.500000

U252                      COAN1X1         tc240c          2.000000

U253                      COND1XL         tc240c          1.500000

U254                      CANR2X1         tc240c          2.000000

U255                      CANR2X1         tc240c          2.000000

U256                      CANR2X2         tc240c          4.000000

U257                      COR2X1          tc240c          1.500000

U258                      CND2X1          tc240c          1.000000

U259                      CANR2X1         tc240c          2.000000

U260                      COR2X1          tc240c          1.500000

U261                      CND2X1          tc240c          1.000000

U262                      CANR2X1         tc240c          2.000000

U263                      CND2X1          tc240c          1.000000

U264                      CND2XL          tc240c          1.000000

U265                      CND2X1          tc240c          1.000000

U266                      COAN1X1         tc240c          2.000000

U267                      CND2X1          tc240c          1.000000

U268                      CND2X1          tc240c          1.000000

U269                      CND2XL          tc240c          1.000000

U270                      CND2X1          tc240c          1.000000

U271                      CND2X1          tc240c          1.000000

U272                      CND2XL          tc240c          1.000000

U273                      CND2X1          tc240c          1.000000

U274                      CND2X1          tc240c          1.000000

U275                      CND2XL          tc240c          1.000000

U276                      CND2X1          tc240c          1.000000

U277                      COAN1X1         tc240c          2.000000

U278                      CND2X1          tc240c          1.000000

U279                      CANR2X1         tc240c          2.000000

U280                      CND2X1          tc240c          1.000000

U281                      CANR2X1         tc240c          2.000000

U282                      COAN1X1         tc240c          2.000000

U283                      CND2X1          tc240c          1.000000

U284                      COAN1X1         tc240c          2.000000

U285                      CND2X1          tc240c          1.000000

U286                      COND1XL         tc240c          1.500000

U287                      CAN2X1          tc240c          1.500000

U288                      COR2X1          tc240c          1.500000

U289                      COR2X1          tc240c          1.500000

U290                      CAN2X1          tc240c          1.500000

U291                      CANR2X1         tc240c          2.000000

U292                      CANR2X1         tc240c          2.000000

U293                      CND2X1          tc240c          1.000000

U294                      CAN2X1          tc240c          1.500000

U295                      COR2X1          tc240c          1.500000

U296                      CANR2X1         tc240c          2.000000

U297                      CAN2X1          tc240c          1.500000

U298                      COR2X1          tc240c          1.500000

U299                      COAN1X1         tc240c          2.000000

U300                      CND2X1          tc240c          1.000000

U301                      CND2X1          tc240c          1.000000

U302                      COR2X1          tc240c          1.500000

U303                      CIVXL           tc240c          1.000000

U304                      CAN2X1          tc240c          1.500000

U305                      COR2X1          tc240c          1.500000

U306                      COAN1X1         tc240c          2.000000

U307                      CND2X1          tc240c          1.000000

U308                      CND2X1          tc240c          1.000000

U309                      COR2X1          tc240c          1.500000

U310                      CIVXL           tc240c          1.000000

U311                      CNR3XL          tc240c          1.500000

U312                      CNR2IX4         tc240c          3.500000

U313                      CNR2X1          tc240c          1.000000

U314                      CIVX2           tc240c          1.000000

U315                      CND3XL          tc240c          1.500000

U316                      CANR4CX1        tc240c          2.000000

U317                      CNR2X1          tc240c          1.000000

U318                      CNR2X1          tc240c          1.000000

U319                      CIVX2           tc240c          1.000000

U320                      CAN2X1          tc240c          1.500000

U321                      CANR11X1        tc240c          2.000000

U322                      CANR1XL         tc240c          1.500000

U323                      CND4X1          tc240c          2.000000

U324                      CAN2X1          tc240c          1.500000

U325                      CND3XL          tc240c          1.500000

U326                      CANR1XL         tc240c          1.500000

U327                      CND3XL          tc240c          1.500000

U328                      CNR2X1          tc240c          1.000000

U329                      CIVX2           tc240c          1.000000

U330                      CIVX2           tc240c          1.000000

U331                      CIVX2           tc240c          1.000000

U332                      CIVX2           tc240c          1.000000

U333                      CANR2X1         tc240c          2.000000

U334                      CANR2X1         tc240c          2.000000

U335                      CND2X1          tc240c          1.000000

U336                      CANR2X1         tc240c          2.000000

U337                      CND2X1          tc240c          1.000000

U338                      CND2X1          tc240c          1.000000

U339                      CANR2X1         tc240c          2.000000

U340                      CND2X1          tc240c          1.000000

U341                      CND2X1          tc240c          1.000000

U342                      COND4CX1        tc240c          2.000000

U343                      CND2X1          tc240c          1.000000

U344                      CANR2X1         tc240c          2.000000

U345                      COND4CX1        tc240c          2.000000

U346                      CND2X1          tc240c          1.000000

U347                      CANR2X1         tc240c          2.000000

U348                      COND4CX1        tc240c          2.000000

U349                      CND2X1          tc240c          1.000000

U350                      CANR2X1         tc240c          2.000000

U351                      COND4CX1        tc240c          2.000000

U352                      CND2X1          tc240c          1.000000

U353                      CANR2X1         tc240c          2.000000

U354                      CAN4X1          tc240c          2.000000

U355                      COND11X1        tc240c          2.000000

U356                      CANR11X1        tc240c          2.000000

U357                      COND4CX1        tc240c          2.000000

U358                      CNR3XL          tc240c          1.500000

U359                      CNR2IX1         tc240c          1.500000

U360                      CNR4X1          tc240c          2.000000

U361                      CNR3XL          tc240c          1.500000

U362                      CNR3XL          tc240c          1.500000

U363                      CNR3XL          tc240c          1.500000

U364                      CNR3XL          tc240c          1.500000

U365                      CNR2X1          tc240c          1.000000

U366                      CMX4XL          tc240c          5.500000  mo

U367                      CMX4XL          tc240c          5.500000  mo

U368                      CMX4XL          tc240c          5.500000  mo

U369                      CMX4XL          tc240c          5.500000  mo

U370                      CANR3X1         tc240c          2.000000

U371                      COND11X1        tc240c          2.000000

U372                      CND3XL          tc240c          1.500000

U373                      CANR2X1         tc240c          2.000000

U374                      CANR2X1         tc240c          2.000000

U375                      CANR2X1         tc240c          2.000000

U376                      CNR2X1          tc240c          1.000000

U377                      CMX4XL          tc240c          5.500000  mo

U378                      CMX4XL          tc240c          5.500000  mo

U379                      CMX4XL          tc240c          5.500000  mo

U380                      CMX4XL          tc240c          5.500000  mo

U381                      COND2X1         tc240c          2.000000

U382                      CENX1           tc240c          2.000000

U383                      CND3XL          tc240c          1.500000

U384                      CND2X1          tc240c          1.000000

U385                      CND3XL          tc240c          1.500000

U386                      COND4CX1        tc240c          2.000000

U387                      CND2X1          tc240c          1.000000

U388                      COND1XL         tc240c          1.500000

U389                      CANR2X1         tc240c          2.000000

U390                      COND1XL         tc240c          1.500000

U391                      CANR2X1         tc240c          2.000000

U392                      COND1XL         tc240c          1.500000

U393                      CANR2X1         tc240c          2.000000

U394                      COND1XL         tc240c          1.500000

U395                      CANR2X1         tc240c          2.000000

U396                      COND1XL         tc240c          1.500000

U397                      CANR2X1         tc240c          2.000000

U398                      COND1XL         tc240c          1.500000

U399                      CND2X1          tc240c          1.000000

U400                      CAOR2XL         tc240c          2.500000

U401                      CAOR2XL         tc240c          2.500000

U402                      CAOR2XL         tc240c          2.500000

U403                      CAOR2XL         tc240c          2.500000

U404                      CAOR2XL         tc240c          2.500000

U405                      CAOR2XL         tc240c          2.500000

U406                      CAOR2XL         tc240c          2.500000

U407                      CAOR2XL         tc240c          2.500000

U408                      COND1XL         tc240c          1.500000

U409                      CND2X1          tc240c          1.000000

U410                      COND1XL         tc240c          1.500000

U411                      CND2X1          tc240c          1.000000

U412                      COND1XL         tc240c          1.500000

U413                      CND2X1          tc240c          1.000000

U414                      CMX4XL          tc240c          5.500000  mo

U415                      CMX4XL          tc240c          5.500000  mo

U416                      CMX4XL          tc240c          5.500000  mo

U417                      CMX4XL          tc240c          5.500000  mo

U418                      CMX4XL          tc240c          5.500000  mo

U419                      CMX4XL          tc240c          5.500000  mo

U420                      CMX4XL          tc240c          5.500000  mo

U421                      CMX4XL          tc240c          5.500000  mo

U422                      COND2X1         tc240c          2.000000

U423                      COND2X1         tc240c          2.000000

U424                      COND2X1         tc240c          2.000000

U425                      COND2X1         tc240c          2.000000

U426                      CANR2XL         tc240c          2.000000

U427                      CANR2XL         tc240c          2.000000

U428                      CANR2XL         tc240c          2.000000

U429                      CANR2XL         tc240c          2.000000

U430                      CANR2XL         tc240c          2.000000

U431                      CANR2XL         tc240c          2.000000

U432                      CANR2XL         tc240c          2.000000

U433                      COND1XL         tc240c          1.500000

U434                      COND1XL         tc240c          1.500000

U435                      COND1XL         tc240c          1.500000

U436                      COND1XL         tc240c          1.500000

U437                      COND1XL         tc240c          1.500000

U438                      COND1XL         tc240c          1.500000

U439                      COND1XL         tc240c          1.500000

U440                      COND1XL         tc240c          1.500000

U441                      CANR2X1         tc240c          2.000000

U442                      COND3X1         tc240c          2.000000

U443                      CANR2X1         tc240c          2.000000

U444                      COND3X1         tc240c          2.000000

U445                      CANR2X1         tc240c          2.000000

U446                      CANR2X1         tc240c          2.000000

U447                      CIVX3           tc240c          1.500000

U448                      CIVX2           tc240c          1.000000

U449                      CIVX2           tc240c          1.000000

U450                      CIVX2           tc240c          1.000000

U451                      CIVX2           tc240c          1.000000

U452                      CIVX2           tc240c          1.000000

U453                      CIVX2           tc240c          1.000000

U454                      CIVX2           tc240c          1.000000

U455                      CIVX2           tc240c          1.000000

U456                      CIVX2           tc240c          1.000000

U457                      CIVX2           tc240c          1.000000

U458                      CIVX2           tc240c          1.000000

U459                      CIVX2           tc240c          1.000000

U460                      CIVX2           tc240c          1.000000

U461                      CIVX2           tc240c          1.000000

U462                      CIVX2           tc240c          1.000000

U463                      CIVX2           tc240c          1.000000

U464                      CIVX2           tc240c          1.000000

U465                      CIVX2           tc240c          1.000000

U466                      CIVX2           tc240c          1.000000

U467                      CIVX2           tc240c          1.000000

U468                      CIVX2           tc240c          1.000000

U469                      CIVX2           tc240c          1.000000

U470                      CIVX2           tc240c          1.000000

U471                      CIVX2           tc240c          1.000000

U472                      CIVX2           tc240c          1.000000

U473                      CIVX2           tc240c          1.000000

U474                      CIVX2           tc240c          1.000000

U475                      CIVX2           tc240c          1.000000

U476                      CIVX2           tc240c          1.000000

U477                      CIVX2           tc240c          1.000000

U478                      CIVX2           tc240c          1.000000

U479                      CIVX2           tc240c          1.000000

U480                      CIVX2           tc240c          1.000000

U481                      CIVX2           tc240c          1.000000

U482                      CIVX2           tc240c          1.000000

U483                      CIVX2           tc240c          1.000000

U484                      CIVX2           tc240c          1.000000

U485                      CIVX2           tc240c          1.000000

U486                      CIVX2           tc240c          1.000000

U487                      CIVX2           tc240c          1.000000

U488                      CIVX2           tc240c          1.000000

U489                      CIVX2           tc240c          1.000000

U490                      CIVX2           tc240c          1.000000

U491                      CIVX2           tc240c          1.000000

addr_reg[0]               CFD2QXL         tc240c          5.000000  n, so

addr_reg[1]               CFD2QXL         tc240c          5.000000  n, so

addr_reg[2]               CFD2QXL         tc240c          5.000000  n, so

addr_reg[3]               CFD2QXL         tc240c          5.000000  n, so

addr_reg[4]               CFD2QXL         tc240c          5.000000  n, so

addr_reg[5]               CFD2QXL         tc240c          5.000000  n, so

addr_reg[6]               CFD2QXL         tc240c          5.000000  n, so

addr_reg[7]               CFD2QXL         tc240c          5.000000  n, so

alu23                     ALU                             86.000000 h

dat_T_reg[0]              CFD2QXL         tc240c          5.000000  n, so

dat_T_reg[1]              CFD2QXL         tc240c          5.000000  n, so

dat_T_reg[2]              CFD2QXL         tc240c          5.000000  n, so

dat_T_reg[3]              CFD2QXL         tc240c          5.000000  n, so

dat_T_reg[4]              CFD2QXL         tc240c          5.000000  n, so

dat_T_reg[5]              CFD2QXL         tc240c          5.000000  n, so

dat_T_reg[6]              CFD2QXL         tc240c          5.000000  n, so

dat_T_reg[7]              CFD2QXL         tc240c          5.000000  n, so

dat_tri[0]                CTSX2           tc240c          3.000000  n

dat_tri[1]                CTSX2           tc240c          3.000000  n

dat_tri[2]                CTSX2           tc240c          3.000000  n

dat_tri[3]                CTSX2           tc240c          3.000000  n

dat_tri[4]                CTSX2           tc240c          3.000000  n

dat_tri[5]                CTSX2           tc240c          3.000000  n

dat_tri[6]                CTSX2           tc240c          3.000000  n

dat_tri[7]                CTSX2           tc240c          3.000000  n

n_state_reg[0]            CLDP1QXL        tc240c          2.500000  n

n_state_reg[1]            CLDP1QXL        tc240c          2.500000  n

n_state_reg[2]            CLDP1QXL        tc240c          2.500000  n

opcode_reg[0]             CFD2QXL         tc240c          5.000000  n, so

opcode_reg[1]             CFD2QXL         tc240c          5.000000  n, so

opcode_reg[2]             CFD2QXL         tc240c          5.000000  n, so

opcode_reg[3]             CFD2QXL         tc240c          5.000000  n, so

r330                      SCALAR_PROCESSOR_DW01_inc_0     24.000000 BO, h

rd_reg                    CFD2QX1         tc240c          7.000000  n, so

state_reg[0]              CFD2QXL         tc240c          5.000000  n, so

state_reg[1]              CFD2QXL         tc240c          5.000000  n, so

state_reg[2]              CFD2QXL         tc240c          5.000000  n, so

wrt_reg                   CFD2QX1         tc240c          7.000000  n, so

--------------------------------------------------------------------------------

Total 388 cells                                           1108.000000

1

report_net

****************************************

Report : net

Design : SCALAR_PROCESSOR

Version: C-2009.06-SP5

Date   : Fri Dec  5 03:58:23 2014

****************************************

Operating Conditions: WCCOM25   Library: tc240c

Wire Load Model Mode: top

Net                 Fanout     Fanin      Load   Resistance    Pins   Attributes

--------------------------------------------------------------------------------

ALU_OUT[0]               3         1     19.74         0.00       4

ALU_OUT[1]               3         1     25.84         0.00       4

ALU_OUT[2]               3         1     21.20         0.00       4

ALU_OUT[3]               3         1     21.23         0.00       4

ALU_OUT[4]               4         1     29.83         0.00       5

ALU_OUT[5]               4         1     28.77         0.00       5

ALU_OUT[6]               4         1     29.27         0.00       5

ALU_OUT[7]               4         1     39.54         0.00       5

ARRAY[0][0]              5         1     42.11         0.00       6

ARRAY[0][1]              5         1     42.11         0.00       6

ARRAY[0][2]              5         1     42.11         0.00       6

ARRAY[0][3]              5         1     42.11         0.00       6

ARRAY[0][4]              5         1     42.11         0.00       6

ARRAY[0][5]              5         1     42.11         0.00       6

ARRAY[0][6]              5         1     42.11         0.00       6

ARRAY[0][7]              5         1     42.11         0.00       6

ARRAY[1][0]              3         1     17.66         0.00       4

ARRAY[1][1]              3         1     16.39         0.00       4

ARRAY[1][2]              3         1     17.66         0.00       4

ARRAY[1][3]              3         1     17.66         0.00       4

ARRAY[1][4]              3         1     16.39         0.00       4

ARRAY[1][5]              3         1     16.39         0.00       4

ARRAY[1][6]              3         1     17.66         0.00       4

ARRAY[1][7]              3         1     16.39         0.00       4

ARRAY[2][0]              3         1     18.14         0.00       4

ARRAY[2][1]              3         1     18.14         0.00       4

ARRAY[2][2]              3         1     18.14         0.00       4

ARRAY[2][3]              3         1     18.14         0.00       4

ARRAY[2][4]              3         1     16.49         0.00       4

ARRAY[2][5]              3         1     16.49         0.00       4

ARRAY[2][6]              3         1     16.49         0.00       4

ARRAY[2][7]              3         1     16.49         0.00       4

ARRAY[3][0]              6         1     69.17         0.00       7

ARRAY[3][1]              5         1     52.27         0.00       6

ARRAY[3][2]              5         1     53.08         0.00       6

ARRAY[3][3]              5         1     52.27         0.00       6

ARRAY[3][4]              5         1     52.27         0.00       6

ARRAY[3][5]              5         1     52.27         0.00       6

ARRAY[3][6]              5         1     52.27         0.00       6

ARRAY[3][7]              5         1     53.46         0.00       6

FR[0]                    2         1     16.22         0.00       3

FR[1]                    2         1     25.70         0.00       3

FR[2]                    2         1     16.48         0.00       3

FR[3]                    2         1     25.70         0.00       3

N64                     13         1    147.32         0.00      14

N65                     13         1    105.26         0.00      14

N66                     11         1    140.09         0.00      12

N67                     11         1     97.91         0.00      12

N92                      1         1      2.61         0.00       2

N93                      1         1      2.61         0.00       2

N94                      3         1      7.76         0.00       4

N95                      1         1      2.61         0.00       2

N99                      2         1     14.51         0.00       3

N100                     2         1     14.51         0.00       3

N101                     2         1     14.51         0.00       3

N102                     2         1     14.51         0.00       3

N103                     2         1     14.90         0.00       3

N104                     2         1     14.90         0.00       3

N105                     2         1     14.90         0.00       3

N106                     2         1     14.90         0.00       3

N107                     2         1     24.12         0.00       3

N108                     2         1     24.12         0.00       3

N109                     2         1     24.12         0.00       3

N110                     2         1     24.12         0.00       3

N111                     2         1     24.12         0.00       3

N112                     2         1     24.12         0.00       3

N113                     2         1     24.12         0.00       3

N114                     2         1     24.12         0.00       3

N245                     1         1      8.79         0.00       2

N246                     1         1      8.79         0.00       2

N247                     1         1      8.79         0.00       2

N248                     1         1      8.79         0.00       2

N249                     1         1      8.79         0.00       2

N250                     1         1      8.79         0.00       2

N251                     1         1      8.79         0.00       2

N253                     1         1      8.79         0.00       2

N470                     1         1      7.50         0.00       2

OP1[0]                   3         1     32.80         0.00       4

OP1[1]                   3         1     32.80         0.00       4

OP1[2]                   3         1     32.80         0.00       4

OP1[3]                   3         1     32.80         0.00       4

OP1[4]                   3         1     32.80         0.00       4

OP1[5]                   3         1     32.80         0.00       4

OP1[6]                   3         1     32.80         0.00       4

OP1[7]                   3         1     32.80         0.00       4

OP2[0]                   2         1     24.73         0.00       3

OP2[1]                   2         1     24.73         0.00       3

OP2[2]                   2         1     24.73         0.00       3

OP2[3]                   2         1     24.73         0.00       3

OP2[4]                   2         1     24.73         0.00       3

OP2[5]                   2         1     24.73         0.00       3

OP2[6]                   2         1     24.73         0.00       3

OP2[7]                   2         1     24.73         0.00       3

OPRN                    12         1    196.89         0.00      13

addr[0]                  2         1     10.06         0.00       3

addr[1]                  2         1     10.06         0.00       3

addr[2]                  2         1     10.06         0.00       3

addr[3]                  2         1     10.06         0.00       3

addr[4]                  2         1     10.06         0.00       3

addr[5]                  2         1     10.06         0.00       3

addr[6]                  2         1     10.06         0.00       3

addr[7]                  2         1     10.06         0.00       3

clk                     82         1    240.96         0.00      83

cout                     2         1     36.91         0.00       3

dat[0]                   4         2     29.37         0.00       5

dat[1]                   4         2     34.13         0.00       5

dat[2]                   4         2     29.37         0.00       5

dat[3]                   4         2     29.37         0.00       5

dat[4]                   5         2     38.07         0.00       6

dat[5]                   5         2     37.93         0.00       6

dat[6]                   5         2     38.07         0.00       6

dat[7]                   4         2     40.00         0.00       5

dat_T[0]                 2         1     22.82         0.00       3

dat_T[1]                 2         1     22.82         0.00       3

dat_T[2]                 2         1     22.82         0.00       3

dat_T[3]                 2         1     22.82         0.00       3

dat_T[4]                 2         1     22.82         0.00       3

dat_T[5]                 2         1     22.82         0.00       3

dat_T[6]                 2         1     22.82         0.00       3

dat_T[7]                 2         1     22.82         0.00       3

n18                      9         1     49.72         0.00      10

n19                      9         1     47.21         0.00      10

n21                      1         1      3.99         0.00       2

n22                      8         1     74.66         0.00       9

n23                      9         1     84.43         0.00      10

n25                      1         1      3.99         0.00       2

n27                      1         1      3.99         0.00       2

n29                      1         1      3.99         0.00       2

n31                      1         1      3.99         0.00       2

n33                      1         1      3.99         0.00       2

n35                      1         1      3.99         0.00       2

n37                      1         1      3.99         0.00       2

n39                      2         1     11.13         0.00       3

n40                     10         1     78.97         0.00      11

n42                      8         1     56.80         0.00       9

n43                      1         1      7.89         0.00       2

n44                      1         1     10.14         0.00       2

n46                     10         1     99.92         0.00      11

n48                      8         1     81.13         0.00       9

n50                      1         1      7.89         0.00       2

n51                      1         1     10.14         0.00       2

n54                      1         1      9.26         0.00       2

n55                      1         1     10.06         0.00       2

n58                      1         1      9.26         0.00       2

n59                      1         1     10.06         0.00       2

n62                      1         1      7.89         0.00       2

n63                      1         1     10.14         0.00       2

n64                      2         1     14.06         0.00       3

n66                      1         1      7.89         0.00       2

n68                      2         1     25.79         0.00       3

n70                      1         1      7.89         0.00       2

n71                      1         1     10.14         0.00       2

n72                      2         1     14.06         0.00       3

n74                      1         1      7.89         0.00       2

n76                      2         1     25.79         0.00       3

n79                      3         1     16.42         0.00       4

n81                      4         1     20.33         0.00       5

n83                      1         1      9.92         0.00       2

n84                      8         1     62.82         0.00       9

n85                      1         1     10.05         0.00       2

n86                      1         1      7.40         0.00       2

n88                      3         1     25.87         0.00       4

n89                      3         1     36.50         0.00       4

n90                      9         1     78.42         0.00      10

n91                      1         1      6.65         0.00       2

n93                      4         1     36.32         0.00       5

n94                     13         1     82.35         0.00      14

n96                      2         1     11.89         0.00       3

n97                      1         1      6.65         0.00       2

n100                     1         1      3.99         0.00       2

n101                     1         1      6.65         0.00       2

n105                     1         1      7.89         0.00       2

n106                     1         1      6.65         0.00       2

n111                     1         1      7.89         0.00       2

n113                     2         1     20.25         0.00       3

n114                     1         1      9.27         0.00       2

n116                     8         1     77.24         0.00       9

n117                     1         1     10.00         0.00       2

n119                     1         1     10.14         0.00       2

n120                     5         1     46.20         0.00       6

n121                     5         1     48.79         0.00       6

n122                     1         1      9.27         0.00       2

n124                     1         1     10.00         0.00       2

n126                     1         1      3.99         0.00       2

n127                     1         1      9.27         0.00       2

n129                     1         1     10.00         0.00       2

n131                     1         1     10.14         0.00       2

n132                     1         1      9.27         0.00       2

n134                     1         1     10.00         0.00       2

n136                     1         1      3.99         0.00       2

n137                     3         1     17.72         0.00       4

n138                     4         1     27.72         0.00       5

n140                     9         1     60.06         0.00      10

n141                     2         1     13.47         0.00       3

n142                     2         1     11.07         0.00       3

n143                     2         1     13.47         0.00       3

n144                     2         1     13.47         0.00       3

n145                     2         1     11.07         0.00       3

n146                     2         1     11.07         0.00       3

n147                     2         1     13.86         0.00       3

n148                     2         1     11.07         0.00       3

n150                     9         1     73.97         0.00      10

n151                    20         1    167.12         0.00      21

n154                     6         1     55.49         0.00       7

n156                     1         1      3.99         0.00       2

n157                     1         1      6.64         0.00       2

n159                     1         1      9.97         0.00       2

n161                     2         1     11.93         0.00       3

n162                    18         1    148.96         0.00      19

n166                     1         1      9.92         0.00       2

n167                     3         1     20.59         0.00       4

n168                     2         1     10.45         0.00       3

n170                     1         1      9.17         0.00       2

n171                     1         1     10.00         0.00       2

n172                     2         1     10.47         0.00       3

n174                     8         1    131.77         0.00       9

n175                     1         1      2.73         0.00       2

n176                     1         1      2.73         0.00       2

n177                     1         1      2.73         0.00       2

n178                     1         1      2.73         0.00       2

n179                     1         1      2.73         0.00       2

n180                     1         1      2.73         0.00       2

n181                     1         1      2.73         0.00       2

n182                     1         1      2.73         0.00       2

n184                     1         1      2.73         0.00       2

n185                     1         1      2.73         0.00       2

n186                     1         1      2.73         0.00       2

n187                     1         1      2.73         0.00       2

n188                     1         1      2.73         0.00       2

n189                     1         1      2.73         0.00       2

n190                     1         1      2.73         0.00       2

n191                     1         1      2.73         0.00       2

n192                     1         1      2.73         0.00       2

n193                     1         1      2.73         0.00       2

n194                     1         1      2.73         0.00       2

n195                     1         1      2.73         0.00       2

n196                     1         1      2.73         0.00       2

n197                     1         1      2.73         0.00       2

n198                     1         1      2.73         0.00       2

n199                     1         1      2.73         0.00       2

n200                     1         1      2.73         0.00       2

n201                     1         1      2.73         0.00       2

n202                     1         1      7.78         0.00       2

n203                     1         1      7.78         0.00       2

n204                     1         1      2.73         0.00       2

n205                     1         1      2.73         0.00       2

n206                     1         1      2.73         0.00       2

n207                     1         1      2.73         0.00       2

n208                     1         1      2.73         0.00       2

n209                     1         1      2.73         0.00       2

n210                     1         1      2.73         0.00       2

n211                     1         1      2.73         0.00       2

n212                     1         1      2.73         0.00       2

n213                     1         1      2.73         0.00       2

n214                     1         1      2.73         0.00       2

n215                     1         1      2.73         0.00       2

n216                     1         1      2.73         0.00       2

n217                     1         1      2.73         0.00       2

n218                     1         1      2.73         0.00       2

n219                     1         1      2.73         0.00       2

n220                     1         1      2.73         0.00       2

n221                     1         1      2.73         0.00       2

n222                     1         1      2.73         0.00       2

n223                     1         1      2.73         0.00       2

n224                     1         1      2.73         0.00       2

n225                     1         1      2.73         0.00       2

n226                     1         1      2.73         0.00       2

n227                     1         1      2.73         0.00       2

n228                     1         1      2.73         0.00       2

n229                     1         1      2.73         0.00       2

n230                     1         1      2.73         0.00       2

n231                     1         1      2.73         0.00       2

n232                     1         1      2.73         0.00       2

n233                     1         1      2.73         0.00       2

n234                     1         1      2.73         0.00       2

n235                     1         1      2.73         0.00       2

n236                     1         1      2.73         0.00       2

n237                     1         1      2.73         0.00       2

n238                     1         1      2.73         0.00       2

n239                     1         1      2.73         0.00       2

n240                     1         1      2.73         0.00       2

n241                     1         1      2.73         0.00       2

n242                     1         1      2.73         0.00       2

n243                     1         1      2.73         0.00       2

n244                     1         1      2.73         0.00       2

n245                     1         1      2.73         0.00       2

n246                     1         1      2.73         0.00       2

n247                     1         1      2.73         0.00       2

n248                     1         1      2.73         0.00       2

n249                     1         1      2.73         0.00       2

n250                     1         1      2.73         0.00       2

n251                     1         1      2.73         0.00       2

n252                     1         1      2.73         0.00       2

n253                     1         1      3.66         0.00       2

n254                     1         1      3.66         0.00       2

n255                     1         1      8.89         0.00       2

n256                     1         1      9.38         0.00       2

n257                     1         1     10.14         0.00       2

n258                     1         1      8.79         0.00       2

n259                     1         1      8.79         0.00       2

n260                     1         1     10.14         0.00       2

n261                     1         1      8.79         0.00       2

n262                     1         1     10.14         0.00       2

n263                     1         1      8.79         0.00       2

n264                     1         1      8.79         0.00       2

n265                     1         1     10.14         0.00       2

n266                     1         1      8.79         0.00       2

n267                     1         1     10.14         0.00       2

n268                     1         1      8.79         0.00       2

n269                     1         1     10.14         0.00       2

n270                     1         1      8.79         0.00       2

n271                     1         1      8.79         0.00       2

n272                     1         1      8.79         0.00       2

n273                     1         1      7.80         0.00       2

n274                     1         1      7.14         0.00       2

n275                     1         1      7.80         0.00       2

n276                     1         1      8.07         0.00       2

n277                     1         1      7.80         0.00       2

n278                     1         1      8.89         0.00       2

n279                     1         1      3.71         0.00       2

n280                     1         1      3.99         0.00       2

n281                     1         1      8.79         0.00       2

n282                     1         1      7.80         0.00       2

n283                     1         1      8.89         0.00       2

n284                     1         1      3.71         0.00       2

n285                     1         1      3.99         0.00       2

n286                     1         1      8.79         0.00       2

n287                     1         1      8.89         0.00       2

n288                     1         1      9.38         0.00       2

n289                     1         1      9.38         0.00       2

n290                     1         1      9.38         0.00       2

n291                     1         1      3.99         0.00       2

n292                     1         1      3.99         0.00       2

n293                     1         1      3.99         0.00       2

n294                     1         1      3.99         0.00       2

n295                     1         1      3.99         0.00       2

n296                     1         1      3.99         0.00       2

n297                     1         1      3.99         0.00       2

n298                     1         1      3.99         0.00       2

n299                    82         1    915.81         0.00      83

n300                     8         1     60.12         0.00       9

n301                     8         1     71.68         0.00       9

n302                     9         1     79.19         0.00      10

n303                     1         1      9.11         0.00       2

n304                     1         1      9.11         0.00       2

n305                     1         1      9.11         0.00       2

n306                     1         1      9.11         0.00       2

n307                    17         1    116.45         0.00      18

n308                    23         1    188.12         0.00      24

n309                     9         1     67.62         0.00      10

n310                     8         1     60.92         0.00       9

n311                     2         1     11.03         0.00       3

n312                     2         1     18.15         0.00       3

n313                     7         1     51.93         0.00       8

n314                     1         1      4.05         0.00       2

n315                     9         1     39.70         0.00      10

n316                     9         1     37.94         0.00      10

n317                     2         1     16.93         0.00       3

n318                     3         1     15.62         0.00       4

n319                     1         1      7.51         0.00       2

n320                     5         1     31.88         0.00       6

n321                     1         1     10.03         0.00       2

n322                     1         1      3.70         0.00       2

n323                     1         1      9.55         0.00       2

n324                     3         1     23.92         0.00       4

n325                     3         1     23.04         0.00       4

n326                     2         1     10.35         0.00       3

n327                     1         1      7.16         0.00       2

n328                     3         1     16.86         0.00       4

n329                     1         1      8.54         0.00       2

n330                     3         1     16.86         0.00       4

n331                     1         1      8.54         0.00       2

n332                     3         1     14.05         0.00       4

n333                     1         1      7.16         0.00       2

n334                     3         1     13.91         0.00       4

n335                     1         1      7.16         0.00       2

n336                     2         1      7.40         0.00       3

n337                     1         1      7.16         0.00       2

n338                     2         1     10.35         0.00       3

n339                     2         1      7.40         0.00       3

n340                     1         1      9.97         0.00       2

n341                     1         1      7.16         0.00       2

n342                     1         1      9.97         0.00       2

n343                     1         1      7.16         0.00       2

n344                     1         1      3.70         0.00       2

n345                     1         1      3.70         0.00       2

n346                     1         1      3.70         0.00       2

n347                     1         1      3.70         0.00       2

n348                     1         1      3.70         0.00       2

n349                     1         1      3.70         0.00       2

n350                     1         1      3.70         0.00       2

n351                     1         1      3.70         0.00       2

n_state[0]               7         1     35.26         0.00       8

n_state[1]               4         1     30.37         0.00       5

n_state[2]               5         1     38.02         0.00       6

opcode[0]                4         1     39.22         0.00       5

opcode[1]                4         1     34.94         0.00       5

opcode[2]                7         1     51.43         0.00       8

opcode[3]                7         1     55.34         0.00       8

ov                       1         1      6.74         0.00       2

rd                       2         1     39.31         0.00       3

rst                      1         1     26.06         0.00       2

state[0]                 3         1     34.99         0.00       4

state[1]                 4         1     43.37         0.00       5

state[2]                 5         1     49.26         0.00       6

wrt                      2         1      7.76         0.00       3

--------------------------------------------------------------------------------

Total 406 nets        1200       414   9326.97         0.00    1606

Maximum                 82         2    915.81         0.00      83

Average               2.96      1.02     22.97         0.00    3.96

1

update_timing

Information: Updating design information... (UID-85)

1

report_timing -max_paths 10

****************************************

Report : timing

-path full

-delay max

-max_paths 10

Design : SCALAR_PROCESSOR

Version: C-2009.06-SP5

Date   : Fri Dec  5 03:58:23 2014

****************************************

Operating Conditions: WCCOM25   Library: tc240c

Wire Load Model Mode: top

Startpoint: wrt_reg (rising edge-triggered flip-flop)

Endpoint: dat[7] (output port)

Path Group: default

Path Type: max

Point                                    Incr       Path

-----------------------------------------------------------

wrt_reg/CP (CFD2QX1)                     0.00       0.00 r

wrt_reg/Q (CFD2QX1)                      0.40       0.40 r

U312/Z (CNR2IX4)                         0.40       0.80 r

dat_tri[7]/Z (CTSX2)                     0.30       1.10 f

dat[7] (inout)                           0.00       1.10 f

data arrival time                                   1.10

max_delay                                1.10       1.10

output external delay                    0.00       1.10

data required time                                  1.10

-----------------------------------------------------------

data required time                                  1.10

data arrival time                                  -1.10

-----------------------------------------------------------

slack (MET)                                         0.00

Startpoint: dat[7] (input port)

Endpoint: ARRAY_reg[2][7]

(rising edge-triggered flip-flop)

Path Group: default

Path Type: max

Point                                    Incr       Path

-----------------------------------------------------------

input external delay                     0.00       0.00 f

dat[7] (inout)                           0.00       0.00 f

U256/Z (CANR2X2)                         0.13       0.13 r

U252/Z (COAN1X1)                         0.26       0.39 r

U422/Z (COND2X1)                         0.11       0.51 f

U295/Z (COR2X1)                          0.22       0.73 f

ARRAY_reg[2][7]/D (CFD2QXL)              0.00       0.73 f

data arrival time                                   0.73

max_delay                                1.10       1.10

library setup time                      -0.37       0.73

data required time                                  0.73

-----------------------------------------------------------

data required time                                  0.73

data arrival time                                  -0.73

-----------------------------------------------------------

slack (MET)                                         0.00

Startpoint: wrt_reg (rising edge-triggered flip-flop)

Endpoint: dat[4] (output port)

Path Group: default

Path Type: max

Point                                    Incr       Path

-----------------------------------------------------------

wrt_reg/CP (CFD2QX1)                     0.00       0.00 r

wrt_reg/Q (CFD2QX1)                      0.40       0.40 r

U312/Z (CNR2IX4)                         0.40       0.80 r

dat_tri[4]/Z (CTSX2)                     0.30       1.10 f

dat[4] (inout)                           0.00       1.10 f

data arrival time                                   1.10

max_delay                                1.10       1.10

output external delay                    0.00       1.10

data required time                                  1.10

-----------------------------------------------------------

data required time                                  1.10

data arrival time                                  -1.10

-----------------------------------------------------------

slack (MET)                                         0.00

Startpoint: wrt_reg (rising edge-triggered flip-flop)

Endpoint: dat[6] (output port)

Path Group: default

Path Type: max

Point                                    Incr       Path

-----------------------------------------------------------

wrt_reg/CP (CFD2QX1)                     0.00       0.00 r

wrt_reg/Q (CFD2QX1)                      0.40       0.40 r

U312/Z (CNR2IX4)                         0.40       0.80 r

dat_tri[6]/Z (CTSX2)                     0.30       1.10 f

dat[6] (inout)                           0.00       1.10 f

data arrival time                                   1.10

max_delay                                1.10       1.10

output external delay                    0.00       1.10

data required time                                  1.10

-----------------------------------------------------------

data required time                                  1.10

data arrival time                                  -1.10

-----------------------------------------------------------

slack (MET)                                         0.00

Startpoint: wrt_reg (rising edge-triggered flip-flop)

Endpoint: dat[5] (output port)

Path Group: default

Path Type: max

Point                                    Incr       Path

-----------------------------------------------------------

wrt_reg/CP (CFD2QX1)                     0.00       0.00 r

wrt_reg/Q (CFD2QX1)                      0.40       0.40 r

U312/Z (CNR2IX4)                         0.40       0.80 r

dat_tri[5]/Z (CTSX2)                     0.30       1.10 f

dat[5] (inout)                           0.00       1.10 f

data arrival time                                   1.10

max_delay                                1.10       1.10

output external delay                    0.00       1.10

data required time                                  1.10

-----------------------------------------------------------

data required time                                  1.10

data arrival time                                  -1.10

-----------------------------------------------------------

slack (MET)                                         0.00

Startpoint: dat[4] (input port)

Endpoint: ARRAY_reg[3][4]

(rising edge-triggered flip-flop)

Path Group: default

Path Type: max

Point                                    Incr       Path

-----------------------------------------------------------

input external delay                     0.00       0.00 f

dat[4] (inout)                           0.00       0.00 f

U259/Z (CANR2X1)                         0.15       0.15 r

U258/Z (CND2X1)                          0.14       0.29 f

U443/Z (CANR2X1)                         0.22       0.51 r

U442/Z (COND3X1)                         0.21       0.72 f

ARRAY_reg[3][4]/D (CFD2QXL)              0.00       0.72 f

data arrival time                                   0.72

max_delay                                1.10       1.10

library setup time                      -0.37       0.73

data required time                                  0.73

-----------------------------------------------------------

data required time                                  0.73

data arrival time                                  -0.72

-----------------------------------------------------------

slack (MET)                                         0.01

Startpoint: dat[5] (input port)

Endpoint: ARRAY_reg[3][5]

(rising edge-triggered flip-flop)

Path Group: default

Path Type: max

Point                                    Incr       Path

-----------------------------------------------------------

input external delay                     0.00       0.00 f

dat[5] (inout)                           0.00       0.00 f

U255/Z (CANR2X1)                         0.15       0.15 r

U261/Z (CND2X1)                          0.14       0.29 f

U445/Z (CANR2X1)                         0.22       0.51 r

U444/Z (COND3X1)                         0.21       0.72 f

ARRAY_reg[3][5]/D (CFD2QXL)              0.00       0.72 f

data arrival time                                   0.72

max_delay                                1.10       1.10

library setup time                      -0.37       0.73

data required time                                  0.73

-----------------------------------------------------------

data required time                                  0.73

data arrival time                                  -0.72

-----------------------------------------------------------

slack (MET)                                         0.01

Startpoint: wrt_reg (rising edge-triggered flip-flop)

Endpoint: dat[1] (output port)

Path Group: default

Path Type: max

Point                                    Incr       Path

-----------------------------------------------------------

wrt_reg/CP (CFD2QX1)                     0.00       0.00 r

wrt_reg/Q (CFD2QX1)                      0.40       0.40 r

U312/Z (CNR2IX4)                         0.40       0.80 r

dat_tri[1]/Z (CTSX2)                     0.29       1.09 f

dat[1] (inout)                           0.00       1.09 f

data arrival time                                   1.09

max_delay                                1.10       1.10

output external delay                    0.00       1.10

data required time                                  1.10

-----------------------------------------------------------

data required time                                  1.10

data arrival time                                  -1.09

-----------------------------------------------------------

slack (MET)                                         0.01

Startpoint: dat[4] (input port)

Endpoint: ARRAY_reg[2][4]

(rising edge-triggered flip-flop)

Path Group: default

Path Type: max

Point                                    Incr       Path

-----------------------------------------------------------

input external delay                     0.00       0.00 f

dat[4] (inout)                           0.00       0.00 f

U262/Z (CANR2X1)                         0.14       0.14 r

U299/Z (COAN1X1)                         0.26       0.40 r

U425/Z (COND2X1)                         0.11       0.51 f

U298/Z (COR2X1)                          0.23       0.75 f

ARRAY_reg[2][4]/D (CFD2QX2)              0.00       0.75 f

data arrival time                                   0.75

max_delay                                1.10       1.10

library setup time                      -0.34       0.76

data required time                                  0.76

-----------------------------------------------------------

data required time                                  0.76

data arrival time                                  -0.75

-----------------------------------------------------------

slack (MET)                                         0.01

Startpoint: dat[5] (input port)

Endpoint: ARRAY_reg[2][5]

(rising edge-triggered flip-flop)

Path Group: default

Path Type: max

Point                                    Incr       Path

-----------------------------------------------------------

input external delay                     0.00       0.00 f

dat[5] (inout)                           0.00       0.00 f

U254/Z (CANR2X1)                         0.14       0.14 r

U306/Z (COAN1X1)                         0.26       0.40 r

U424/Z (COND2X1)                         0.11       0.51 f

U305/Z (COR2X1)                          0.23       0.75 f

ARRAY_reg[2][5]/D (CFD2QX2)              0.00       0.75 f

data arrival time                                   0.75

max_delay                                1.10       1.10

library setup time                      -0.34       0.76

data required time                                  0.76

-----------------------------------------------------------

data required time                                  0.76

data arrival time                                  -0.75

-----------------------------------------------------------

slack (MET)                                         0.01

1

report_area

****************************************

Report : area

Design : SCALAR_PROCESSOR

Version: C-2009.06-SP5

Date   : Fri Dec  5 03:58:23 2014

****************************************

Library(s) Used:

tc240c (File: /apps/toshiba/sjsu/synopsys/tc240c/tc240c.db_WCCOM25)

Number of ports:               20

Number of nets:               406

Number of cells:              388

Number of references:          44

Combinational area:        657.500000

Noncombinational area:     450.500000

Net Interconnect area:      undefined  (No wire load specified)

Total cell area:          1108.000000

Total area:                 undefined

1

report_power

Loading db file '/apps/toshiba/sjsu/synopsys/tc240c/tc240c.db_WCCOM25'

Warning: Main library 'tc240c' does not specify the following unit required for power: 'Leakage Power'. (PWR-424)

Information:PRopagating switching activity (low effort zero delay simulation). (PWR-6)

Warning: There is no defined clock in the design. (PWR-80)

Warning: Design has unannotated primary inputs. (PWR-414)

Warning: Design has unannotated sequential cell outputs. (PWR-415)

****************************************

Report : power

-analysis_effort low

Design : SCALAR_PROCESSOR

Version: C-2009.06-SP5

Date   : Fri Dec  5 03:58:24 2014

****************************************

Library(s) Used:

tc240c (File: /apps/toshiba/sjsu/synopsys/tc240c/tc240c.db_WCCOM25)

Operating Conditions: WCCOM25   Library: tc240c

Wire Load Model Mode: top

Global Operating Voltage = 2.3

Power-specific unit information :

Voltage Units = 1V

Capacitance Units = 1.000000ff

Time Units = 1ns

Dynamic Power Units = 1uW    (derived from V,C,T units)

Leakage Power Units = Unitless

Cell Internal Power  =   2.1754 mW   (88%)

Net Switching Power  = 301.2914 uW   (12%)

---------

Total Dynamic Power    =   2.4767 mW  (100%)

Cell Leakage Power     =   0.0000

1

write -hierarchy -format verilog -output scalar_netlist.v

Writing verilog file '/home/pa/pate8552/271new/scalar_netlist.v'.

1

quit

Thank you...

C.4	Screenshot Circuits from Synthesis (Design Compiler)

- Half Adder :

- Full Adder :

- 8-bit Ripple Carry Adder (RCA)

- ALU

- SCALAR Processor

- Schematic Layout of Scalar Processor